3GPP TSG-RAN WG2 Meeting #130	R2-2503554

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.3

Source: 	Fujitsu

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion and Decision

- Introduction

The following objective of the work item aims at specifying measurement related enhancements for New WID [1] on NR mobility enhancements:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution will provide more considerations on the remaining open issues for the event triggered L1 measurement reporting.

- Discussions

LTM event evaluation

For event LTM 3/4/5, candidate beams will be evaluated. In Rel-18, serving cell can be configured as LTM candidate cell. In this case, RAN2 needs to discuss whether beams of the serving cell need to be evaluated. RAN2 has agreed that candidate cell IDs can be configured in L1 reporting configuration. If a candidate cell ID is not provided, the UE is not required to evaluate the RSs belonging to the candidate ID [2]. We think that this can be applied to the case, i.e. current serving cell configured as LTM candidate cell.

Proposal 1: When current serving cell is configured as one of the candidate cells, the UE will perform event evaluation according to the network configuration.

MAC CE for measurement report (MR MAC CE)

Regarding the content of MR MAC CE triggered by LTM2, one of essential open issues listed in the E-mail discussion summary [3] is whether only the current beam information can be included in the MR MAC CE or the MR can include measurements for LTM candidates as well. Event LTM2 is that beam of serving cell becomes worse than a threshold. How to use this event depends on the network implementation. For example, LTM2 can be configured for the network to trigger early synchronization. In this case, it is beneficial to include measurements for LTM candidates. If s-measure is not configured, or it is configured and larger than the LTM2 threshold, there could be available measurement results for LTM candidates when the MR is triggered by the event LTM2. Besides, the network can control the content of MR MAC CE triggered by LTM2, e.g. by configuring a smaller s-measure or configuring the number of beams to be reported.

Proposal 2: For the MR triggered by LTM2, based on the network configuration, the MR MAC CE can include measurements for LTM candidates in addition to the current beam. The number of beams to be reported N is applied.

Prioritization/multiplexing of MR MAC CEs

There is a possibility that more than one MR MAC CEs are generated. For this case, the UE can include these MAC CEs in one MAC PDU or select one of them to include in the MAC PDU. Considering the resource utilization and signalling overhead, we prefer slightly to support the multiplexing of MR MAC CEs.

Proposal 3: To support more than one (truncated) MR MAC CEs to be multiplexed in one UL grant, if available.

BSR for MR MAC CEs

In exiting mechanism [4], BSR can be triggered by UL data and MAC CE will not trigger the BSR. In addition, buffer size calculation considers data volume in PDCP and RLC. MAC CEs will not be counted for buffer size. If BSR for MR MAC CEs is supported, significant standardization impact is predicted. So, this will not be supported.

Proposal 4: RAN2 will not support BSR for MR MAC CEs.

Other

For L3 measurement reporting, to address transmission failure of the L3 measurement report, a timer T312 can be configured. The timer T312 can be started if the L3 measurement report is triggered when T310 is running. Failure reporting or connection re-establishment procedure is initiated when the timer expires.

For L1 measurement reporting, RAN2 has agreed that the event triggered L1 measurement reporting should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization

- Select the target beam/cell and trigger LTM cell switch procedure

T312-like mechanism can be used for the L1 measurement reporting triggered by the events defined for LTM decision. With this, the UE can take some actions when the transmission of the event triggered L1 measurement report fails.

Proposal 5: RAN2 considers T312-like mechanism, e.g. for the event configured for LTM decision.

- Conclusion

This document discussed the remaining open issues for the L1 event triggered measurement report and made the following proposals:

LTM event evaluation

Proposal 1: When current serving cell is configured as one of the candidate cells, the UE will perform event evaluation according to the network configuration.

MAC CE for measurement report (MR MAC CE)

Proposal 2: For the MR triggered by LTM2, based on the network configuration, the MR MAC CE can include measurements for LTM candidates in addition to the current beam. The number of beams to be reported N is applied.

Prioritization/multiplexing of MR MAC CEs

Proposal 3: To support more than one (truncated) MR MAC CEs to be multiplexed in one UL grant, if available.

BSR for MR MAC CEs

Proposal 4: RAN2 will not support BSR for MR MAC CEs.

Other

Proposal 5: RAN2 considers T312-like mechanism, e.g. for the event configured for LTM decision.

# Reference

- RP-242356, “Revised Work Item: NR mobility enhancements Phase 4”, RAN#105, Sep. 9-12, 2024

- R2-2502981, “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung), RAN2#129bis

- R2-2503766, “Discussion summary and list of MAC open issue for Mob Ph4”, vivo, RAN2#130, May of 2025

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification"