3GPP TSG-RAN WG2 Meeting #130	R2-2503566

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.7.5

Source:	Nokia, Nokia Shanghai Bell

Title:	RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129bis:

Agreements on RLC enahncements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

This contribution investigates the remaining points to complete the work on RLC enhancements for XR in Rel-19.

# 2	Autonomous Retransmissions

## 2.1	Trigger

Another discard timer in PDCP is discardTimerForLowImportance, which aims at discarding PDUs that are of less importance (e.g. in case of congestion). It would obviously not make sense to use radio resources to trigger autonomous retransmission for these low-importance SDUs.

Proposal 1 (RRC-3, RRC-4): expiry of discardTimerForLowImportance does not trigger autonomous RLC retransmission, nor early polls.

## 2.2	Threshold Ownership

One question that arose during the email discussions is whether the new thresholds should be configured per PDCP entity or per RLC entity. It seems logical to have the threshold configured in the entity owning the corresponding timer, and relying on PDCP telling RLC when the threshold is met instead of having RLC having to constantly track a timer value. As a matter of fact, the running RLC and PDCP CR exemplifies this.

Proposal 2 (RRC-x): the remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

## 2.3	Other Issues

The main concerns expressed with autonomous retransmissions had to do with capacity implications. However, the autonomous retransmissions can be kept to a minimum provided that RLC ACKs are provided rapidly. What makes the current RLC Status PDU difficult for rapid acknowledgment is that, among the SDUs that are acknowledged, all the other SDUs are negatively acknowledged. The t-Reassembly timer avoids such premature NACKs but also holds back the ACKs.

Observation 1: among the SDUs that are acknowledged in a RLC Status PDU, all the other SDUs are always negatively acknowledged.

Since RAN2 agreed to focus specifically on timely uplink retransmissions, a complete solution to this problem would be to make it configurable whether the UE considers SDUs for retransmission when NACKed. A UE that would not, would allow the network to send RLC ACKs rapidly without any concerns about premature NACKs.

Proposal 3 (RLC-9): to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Another mechanism that may hold back RLC ACKs is LCP. Currently in LCP, there is no consideration regarding prioritizing control PDUs such as RLC Status PDU. The RLC Status PDU is prioritized over data PDU only within the same bearer/LCH when the LCH is treated in LCP. As a result, during LCP, the LCHs with RLC Status PDU awaiting transmission, may end up not getting enough allocation to put such PDUs in the MAC PDU being generated for transmission, delaying transmission of these PDUs.

Proposal 4 (RLC-9): RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

With autonomous RLC retransmissions in place, comes the question whether they should be reflected in BSR. If they are, the gNB is not able to distinguish the data volume for initial transmissions from the data volume for retransmissions and as a result, may prioritise the wrong UE. To avoid that, either autonomous retransmission data is not included in data volumes, or it is reported separately.

Proposal 5 (RLC-5): autonomous retransmission data is either not included in data volumes, or is reported separately.

# 3	Unnecessary Retransmissions Avoidance

## 3.1	PDCP-transmission management

Related to RLC ACKs being sent also for discarded SDUs, received RLC ACKs are also used locally by PDCP, for instance to determine how high-numbered PDCP SDUs can be transmitted by PDCP:

NOTE 1: Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

Indeed, the PDCP specification also contains the following:

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows: - if RCVD_SN < SN(RX_DELIV) – Window_Size: - RCVD_HFN = HFN(RX_DELIV) + 1. - else if RCVD_SN >= SN(RX_DELIV) + Window_Size: - RCVD_HFN = HFN(RX_DELIV) – 1. - else: - RCVD_HFN = HFN(RX_DELIV); - RCVD_COUNT = [RCVD_HFN, RCVD_SN]. After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU and consider it as not received; - if RCVD_COUNT < RX_DELIV; or - if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before: - discard the PDCP Data PDU;

That is, a received PDCP SN that is more than Window_Size (equal to half of the PDCP SN space) ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN. In the current specification, the PDCP transmitter is expected to take this into account by maintaining a local estimate of RX_DELIV at the peer entity, obtained as the lowest COUNT value not yet positively acknowledged. In absence of a PDCP Status report, these acknowledgements come in the form of RLC ACKs.

Observation 2: a received PDCP SN that is more than half of the PDCP SN space ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN.

Observation 3: to determine how high-numbered PDCP SDUs can be transmitted, the PDCP transmitter currently maintains a local estimate (lower bound) of RX_DELIV at the peer entity, which in absence of a PDCP Status report is obtained as the lowest COUNT value not yet RLC-ACKed.

By previous RAN2 agreements and the current running CR, the new RLC timer t-RxDiscard can be started by a received SDU segment not yet delivered to PDCP. It follows that for an unreceived RLC SDU, t-RxDiscard may start and expire earlier than PDCP’s t-Reordering, and consequently that the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU. This means that RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

Observation 4: accounting for possible RLC SDU segments, for an unreceived RLC SDU, the new RLC timer t-RxDiscard may expire earlier than PDCP’s t-Reordering, and consequently the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU.

Observation 5: RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

As a replacement, regular Status reporting would seem to be needed at PDCP.

Proposal 6: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

## 3.2 	Stopping of transmissions and t-PollRetransmit

The current running CR states the following.

#### 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured); or - consider any RLC SDU which has not been positively acknowledged for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured). - include a poll in an AMD PDU as described in clause 5.3.3.2.

In the e-mail discussion an FFS was raised on the above new, underlined text:

FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

As raised by Nokia and Apple below

If the transmitting window is stalled, to un-stall the window ASAP there seems to be no alternative to re-sending the poll in a retransmitted SDU when t-PollRetransmit expires.

If the transmitting window is not stalled, there seems to be no reason to keep t-PollRetransmit running if the  transmissions of all transmitted but non-ACKed SDUs have been stopped due to discard indication from PDCP.

Proposal 7 (RLC-11):	If the transmitting window is stalled when t-PollRetransmit expires, the poll is re-sent in a retransmitted SDU, even if only SDUs discarded by PDCP are buffered.

Proposal 8 (RLC-11):	When an RLC ACK or a PDCP-discard indication for an SDU is received, if the transmitting window is not stalled, t-PollRetransmit is stopped if, after the reception, all SDUs previously submitted to lower layer have been either ACKed or their transmissions have been stopped due to discard indication from PDCP.

# 4	Reporting PDCP SN gap upon mobility

Based on the current Rel-18 specifications, the following alternative courses of events may have happened with a DRB mapped on RLC AM, just before the UE receives an RRC command for an event involving network-side change of the DRB’s PDCP-termination point (handover, or bearer-type change between MN- and SN-terminated bearer):

A.	The source RAN node successfully received a PDCP SN Gap report from the UE:

-	The current specs, such as XnAP SN Status Transfer, provide no means to transfer this information from the source RAN node to the target RAN node;

-	This case was main the topic of the recent RAN3 discussions.

B.	The UE transmitted a PDCP SN Gap report, but its RLC transmissions did not successfully complete before the layer was flushed by the RRC command:

-	In this case, there is no information to be transferred from the source RAN node to the target RAN node.

In the latter case B, a more detailed sequence can be as follows:

0.	All PDCP SDUs before SDU #X have been successfully delivered from UE to the network;

1.	SDU #X is discarded, and PDCP SN Gap report is triggered and transmitted by RLC, together with some SDUs from #(X+1) onwards;

2.	UE receives and executes an RRC command for handover, including RLC re-establishment for the DRB;

-	The RLC re-establishment takes place before successful delivery of any PDCP PDU transmitted in step 1;

3.	In line with the above, XnAP SN Status transfer from source RAN node to target RAN node indicates #X as the First missing UL SDU (and no bitmap indicating higher-numbered received SDUs);

4.	After handover completion by the UE (and the related delay), as part of PDCP Re-establishment, UE PDCP retransmits the SDUs from #(X+1) onwards. At this point, reception of these SDUs while SDU #X remains missing starts t-Reordering at the target RAN node, which will run until expiry before the received SDUs are delivered to upper layers.

It can be observed that t-Reordering runs unnecessarily in step 4, only because the RAN is missing the PDCP-SN-gap information.

Observation 6: Handover-related retransmissions by UE PDCP, of SDUs following a discarding gap, can unnecessarily start t-Reordering at the handover-target RAN node if the RAN is missing the related PDCP-SN-gap information.

Proposal 8: RAN2 agree that unnecessary starting of t-Reordering at PDCP-mobility target RAN node because of PDCP-SN-gap information missing in the RAN is a problem to be solved in Rel.19.

The only solution direction that also solves case B seems to involve the UE retransmitting PDCP SN Gap information to the target RAN node.

Observation 7: The UE PDCP may have transmitted a PDCP SN Gap report just before a PDCP-mobility event that interrupts the report’s RLC transmission. The only way to recover the report’s information seems to be retransmission by the UE PDCP, to the target RAN node.

A simple fix seems to be that, as part of PDCP Re-establishment (which is always invoked at any PDCP termination-point change), the UE re-transmit any previously transmitted PDCP SN Gap report(s) for which no lower-layer confirmation of successful delivery has been received.

Proposal 9: as part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

# 5	Conclusion

This document has proposed the following:

Proposal 1 (RRC-3, RRC-4): expiry of discardTimerForLowImportance does not trigger autonomous RLC retransmission, nor early polls.

Proposal 2 (RRC-x): the remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

Proposal 3 (RLC-9): to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Proposal 4 (RLC-9): RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

Proposal 5 (RLC-5): autonomous retransmission data is either not included in data volumes, or is reported separately.

Proposal 6: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

Proposal 7 (RLC-11):	If the transmitting window is stalled when t-PollRetransmit expires, the poll is re-sent in a retransmitted SDU, even if only SDUs discarded by PDCP are buffered.

Proposal 8 (RLC-11):	When an RLC ACK or a PDCP-discard indication for an SDU is received, if the transmitting window is not stalled, t-PollRetransmit is stopped if, after the reception, all SDUs previously submitted to lower layer have been either ACKed or their transmissions have been stopped due to discard indication from PDCP.

Proposal 9 (RLC-12): RAN2 agree that unnecessary starting of t-Reordering at PDCP-mobility target RAN node because of PDCP-SN-gap information missing in the RAN is a problem to be solved in Rel.19.

Proposal 10 (RLC-12): as part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.