3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2503578

St. Julians, Malta, May 19th – 23rd , 2025

Source:	CATT

Title:	Views on LS on RTP retransmission (S4-250739)

Agenda Item:	8.20.2

Document for:	Discussion and Decision

Introduction

In the SA4 incoming LS [1], the potential enhancement of RTP retransmission has been introduced, and the following question to RAN2 is indicated:

Question to RAN2: Considering the options identified above (including potential mapping of source and retransmission PDUs into distinct QoS flows in the 5GC), are there any potential additional benefits to the RAN from receiving application-layer retransmission information when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the RAN.

In this contribution, our views on this issue are provided for discussion.

Discussion

According to the LS [1] as below, the motivation is to improve the timely delivery of RTP retransmission PDUs, and the suggested solution is that the 5GC maps source and retransmission PDUs associated to a media flow into distinct QoS flows.

Retransmission PDUs have stricter delay requirements to ensure meeting the decoding deadlines of corresponding ADUs at the receiver. One identified approach to address this requirement is configuring an RTP sender to mark source and retransmission PDUs into different PDU Sets, which the 5GC maps then into distinct QoS flows. This approach can improve the timely delivery of retransmission PDUs by providing better QoS.

Since currently there is “no mechanism to inform the 5G network whether an application uses RTP retransmission” , SA4 asks RAN2 whether it’s beneficial for RAN to receive application-layer retransmission information, and the similar question to SA2 is also made as below.

Currently, there is no mechanism to inform the 5G network whether an application uses RTP retransmission (e.g. a flag in the RTP HE for PDU Set marking) or any related information, such as how long source packets are kept in the RTP sender buffer for potential retransmission. Question to SA2: Considering the options identified above, are there any benefits to providing application-layer retransmission information to the 5GC when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the 5GC. Question to RAN2: Considering the options identified above (including potential mapping of source and retransmission PDUs into distinct QoS flows in the 5GC), are there any potential additional benefits to the RAN from receiving application-layer retransmission information when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the RAN.

Before discussing the QoS flow mapping, according to the LS [1], it’s not clear how many RTP PDUs need to be retransmitted, e.g., the RTP retransmission PDUs refer to all PDUs in a PDU set, or only the key PDUs like I frame PDUs need to be retransmitted, or only the unsuccessfully transmitted PDUs need to be retransmitted. And it’s also not clear how often the RTP PDU retransmission should be performed. In our understanding, if there are only 1% RTP PDUs that need to be retransmitted, it’s not reasonable to establish a separate QoS flow, and they may be mapped directly to the default QoS flow. And if the retransmitted RTP PDUs take up 50%, a separate QoS flow could be beneficial.

Observation 1：Whether a separate QoS flow is necessary for RTP retransmission PDUs depends on how many and how often RTP PDUs need to be retransmitted, but this is not clear according to the SA4 LS.

Even if a separate QoS flow is configured during PDU session establishment, the corresponding QoS parameters can be assigned based on the existing specification. From RAN2 perspective, it’s up to gNB implementation to map different QoS flows to same/different DRBs, and also up to gNB implementation to achieve the corresponding scheduling to fulfill the QoS requirement. Therefore, no additional assistance information from application layer seems to be really necessary.

Further enhancement in PDU session establishment procedure may be identified by SA2, e.g., additional per QoS flow parameter indicating QoS Y is for the retransmission of QoS X, then gNB can map QoS X and QoS Y into different DRBs. But this is out of RAN2 scope, we can rely on SA2 to discuss first.

Observation 2: It’s up to gNB implementation to map different QoS flows to same/different DRBs, and also up to gNB implementation to achieve the corresponding scheduling to fulfill the QoS requirement. No additional assistance information from application layer seems to be really necessary.

Observation 3: RAN2 work is postponed until SA2 determines what RAN enhancement is needed for mapping between QoS flow and DRB.

Based on the analysis above, we make the corresponding proposal:

Proposal 1: RAN2 to reply to SA4 with the following information:

- Whether a separate QoS flow is necessary for RTP retransmission PDUs depends on how many and how often RTP PDUs need to be retransmitted, but this is not clear according to the SA4 LS.

- It’s up to gNB implementation to map different QoS flows to same/different DRBs, and also up to gNB implementation to achieve the corresponding scheduling to fulfill the QoS requirement. No additional assistance information from application layer seems to be really necessary.

- RAN2 work is postponed until SA2 determines what RAN enhancement is needed for mapping between QoS flow and DRB.

Conclusion

According to the analysis in section 2, we have the observations as below:

Observation 1：Whether a separate QoS flow is necessary for RTP retransmission PDUs depends on how many and how often RTP PDUs need to be retransmitted, but this is not clear according to the SA4 LS.

Observation 2: It’s up to gNB implementation to map different QoS flows to same/different DRBs, and also up to gNB implementation to achieve the corresponding scheduling to fulfill the QoS requirement. No additional assistance information from application layer seems to be really necessary.

Observation 3: RAN2 work is postponed until SA2 determines what enhancement is needed for mapping between QoS flow and DRB.

And we propose:

Proposal 1: RAN2 to reply to SA4 with the following information:

- Whether a separate QoS flow is necessary for RTP retransmission PDUs depends on how many and how often RTP PDUs need to be retransmitted, but this is not clear according to the SA4 LS.

- It’s up to gNB implementation to map different QoS flows to same/different DRBs, and also up to gNB implementation to achieve the corresponding scheduling to fulfill the QoS requirement. No additional assistance information from application layer seems to be really necessary.

- RAN2 work is postponed until SA2 determines what RAN enhancement is needed for mapping between QoS flow and DRB.

Reference

- S4-250739, LS on RTP retransmission