3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2503580

St. Julian, Malta, 19th – 23rd May, 2025

Source:	CATT

Title:	Discussion on Rx BSF optimization for NR RRM Ph5

Agenda Item:	8.20.1

Document for:	Discussion and Decision

Introduction

An LS on Rx BSF optimization for NR RRM Ph5 from RAN4 was received [1]. In this contribution, our consideration on Rx BSF optimization is provided.

Discussion

It is observed that FR2-1 SSB based RRM requirements allow excessively long delay for UE in L3 measurement, according to the large Rx beam sweeping factor and/or CSSF factor as specified in corresponding requirement tables of TS38.133. Therefore, Rx beam sweeping optimization is one of the objectives in NR Radio Resource Management (RRM) Phase 5 WI [2] to reduce FR2-1 SSB based L3 measurement delay reduction for connected mode, as shown below.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

According to the WID, the enhancement of Rx BSF (beam sweeping factor) is for the UE supporting Multi-Rx simultaneous reception on single carrier. It means UE can receive multiple SSBs from different directions on the same target frequency layer within a SMTC window. Then it would be possible to perform Rx beam sweeping in parallel on different panels at UE and the corresponding L3 detection/measurement delay requirement can be reduced significantly.

Observation 1: For the UE supporting multi-Rx simultaneous reception on single carrier, FR2-1 L3 measurement delay can be reduced by optimizing Rx BSF (beam sweeping factor) if UE performs Rx beam sweeping in parallel on different panels.

But it is not advisable to have the UE always operate with two/more activated panels considering UE power consumption. Therefore, RAN4 agreed to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. RAN4 also agreed that the criteria/threshold(s) are configured by RRC signaling.

Observation 2: In order to reduce UE power consumption, triggering condition to activate/deactivate multi-Rx for L3 measurement which is configured by RRC signalling was agreed in RAN4.

And according to RAN4 LS, the scenario of Rx BSF optimization is limited the serving cell to a FR2 serving cell.

Agreement in RAN4#114bis: - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. Agreement in RAN4#112: - Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-Rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different.

Observation 3: The scenario of Rx BSF optimization is limited to a FR2 serving cell, without CA/DC.

But according to RAN4’s discussion [3], there are still open issues on Rx BSF optimization which may impact RAN2 signalling design, including:

  - Whether to add the hysteresis and TimeToTrigger parameters to the triggering condition.

  - Quitting conditions and re-evaluation mechanism for Rx BSF optimization.

Considering above open issues, we prefer RAN2 to wait for RAN4 further conclusions for Rx BSF optimization.

Observation 4: There are still open issues on Rx BSF optimization which may impact RAN2 signalling design.

Proposal 1: RAN2 waits for RAN4 further progress for Rx BSF optimization.

Conclusion

According to section 2, we observe and propose:

Observation 1: For the UE supporting multi-Rx simultaneous reception on single carrier, FR2-1 L3 measurement delay can be reduced by optimizing Rx BSF (beam sweeping factor) if UE performs Rx beam sweeping in parallel on different panels.

Observation 2: In order to reduce UE power consumption, triggering condition to activate/deactivate multi-Rx for L3 measurement which is configured by RRC signalling was agreed in RAN4.

Observation 3: The scenario of Rx BSF optimization is limited to a FR2 serving cell, without CA/DC.

Observation 4: There are still open issues on Rx BSF optimization which may impact RAN2 signalling design.

Proposal 1: RAN2 waits for RAN4 further progress for Rx BSF optimization.

Reference

- R4-2504962, LS on Rx BSF optimization for NR RRM Ph5, RAN4 LS

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN#105

- R4-2504916, AdHoc minutes for [114bis][208][209] NR_RRM_Ph5