3GPP TSG-RAN WG2 Meeting #130	R2-2503633

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO for LTM and near failure handling for CHO with candidate SCGs

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses details regarding the optimization of early TA acquisition and open issues relating to near failure handling for CHO with candidate SCGs.

# 2	MRO for LTM

The following agreements were reached on the topic of MRO for LTM before this meeting:

- RAN2#125bis

- RAN2 considers SHR, RA report and RLF for MCG LTM SON.

- RAN2#127

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report.

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

## 2.1	Early TA acquisition optimization

The main rational behind and strength of LTM is the RACH-less cell change, which is only possible with knowing the timing advance (TA) when cell switch is triggered. One critical part of the LTM is therefore the phase called early TA acquisition (ETA), which the UE may be instructed to perform before receiving the cell switch command. If the ETA is successful, the interruption time during the cell switch will be significantly reduced as random access (RA) can be skipped. If the UE received the cell switch command that does not include a valid TA previously acquired for the target cell, the UE can still access the cell via regular RA procedure, but the benefits of LTM compared to the legacy L3 handover types are lost.

In case of the network-triggered early TA acquisition, the two trigger criteria for early TA acquisition and cell switch command need to be properly synced for a successful RACH-less LTM. Firstly, the TA once acquired may become invalid because of changed UE position or changed propagation conditions with respect to the candidate cell and the network will need to trigger early TA acquisition again. Secondly, the early TA acquisition procedure should not be performed too often as ETA blocks UL scheduling requests and thus is associated with some interruption time.

Observation 1: Timely and correct early TA acquisition is crucial for LTM to guarantee its benefits, namely a RACH-less cell switch for minimum handover interruption time.

For example, the network should avoid unnecessary early TA acquisition (e.g. obtain the TA for cells that are then not used for cell switch) and should be able to identify instances where the TA was acquired but became invalid by the time cell switch command was sent.

Proposal 1: RAN2 agrees that optimization of timing of ETA is a crucial part of MRO for LTM

Proposal 2: RAN2 agrees to log additional information regarding TAs that were unnecessarily acquisitioned.

In other cases, the TA may be obtained and included in the cell switch command but upon UE’s attempt to use it, access to the target cell fails. The UE then has to switch RACH-based access via LTM recovery/re-establishment procedure where a new TA value is provided by the network in RAR message. For such cases, reporting of the information about the difference between the TA provided in the Cell switch command for which the RACH-less LTM failed, and TA received consequently in RAR for recovery/re-establishment may help to optimize the ETA procedure.

Proposal 3: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

As an alternative to the network triggered TA, the UE may be instructed to obtain the TA itself based on measurements. The two methods may even be configured in parallel. However, in such cases, if the network includes a TA in the Cell switch command the UE must use it. It may happen that the TA obtained based on UE measurements is very close to TA received consequently in RAR for recovery/re-establishment in case of RACH less LTM failure for TA reported in the Cell switch command. Consequently, the UE, based on some similar observation of other LTM cell switch procedures may prove the trend of better precision of UE measurement-based TA. Then it may be desirable that the UE indicates to the network that the use of the UE measurement-based TA is preferred.

Proposal 4: RAN2 to discuss signalling that allows the UE to indicate that the use of the UE measurement-based TA is preferred.

# 3		Near failure handling for CHO with candidate SCGs

The following agreements were reached on the topic of MRO for CHO with candidate SCG with regard to information to be logged in case of near failure in SHR and SPR:

RAN2#125bis: RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

RAN2#128: Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

RAN2#129bis: 
UE includes the target PCell ID in SPR for successful CHO with candidate SCG. 
In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

At the start of the release both failure and near failure scenarios were agreed to be studied for CHO with candidate SCGs. However, no near failure scenarios were discussed or defined in RAN3 until now and RAN2 also did not clearly decide on when and how near failure related information is to be logged.

In the spirit of aligning the report content with RLF report and legacy SHR, some agreements were made in RAN2 on enhancing its content. In the last meeting the same was agreed for SPR. There are cases where both SHR and SPR are generated for the same CHO with candidate SCGs execution and therefore the root cause analysis should include both near failure reports as they are both pointing to the same CHO with candidate SCGs execution. In this way, the maximum information is gathered and can be used by the network for optimization actions. An individual treatment of SHR and SPR would not embrace the strong inter-dependency of the two links and each report represents only the link-specific view of the issue. Not knowing that there was another near failure on the second link can result in wrong root cause decision. If wrong counters are concluded by separate analysis, a mutual statistical post-processing will not help.

For that reason, the two near failure cases need to be analyzed in a unified manner and need to be correlated. This issue was also identified as an open issue during the 38.331 CR review after RAN2#129 [[Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)]. In the absence of such correlation, the network may have a hard time understanding what happened and may even take wrong corrective actions. Such a correlation may be easily achieved by including for instance a coexistence flag, in each report pointing to the existence of the other report. The UE is aware of both near failures occurring for same CHO with candidate SCGs execution and can add such an indication to both reports.

Proposal 5: RAN2 to discuss and decide on a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution, for example by introducing a coexistence flag in each report.

Furthermore, a T310-percentage triggered SHR or SPR, respectively, is to be treated differently depending on which cell change condition was fulfilled already when near failure is experienced. Therefore, another interesting aspect in case of SHR/SPR triggered by T310-percentage criterion would be knowing at which point T310 was started in relation to the two execution conditions’ fulfillments. For example, if T310 on either link was started after the fulfillment of one execution condition but before fulfilment of the second execution condition, this information could more clearly point to the root cause of the near failure, e.g. the second fulfilled condition needs to be updated.

Proposal 6: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s) and an indication about this in SHR and SPR.

# 4	Conclusion

This document has made the following observation and proposals for TA optimization:

Observation 1: Timely and correct early TA acquisition is crucial for LTM to guarantee its benefits, namely a RACH-less cell switch for minimum handover interruption time.

Proposal 1: RAN2 agrees that optimization of timing of ETA is a crucial part of MRO for LTM

Proposal 2: RAN2 agrees to log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 3: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

Proposal 4: RAN2 to discuss signalling that allows the UE to indicate that the use of the UE measurement-based TA is preferred.

This document has made the following proposals for MRO for CHO with candidate SCGs:

Proposal 5: RAN2 to discuss and decide on a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution, for example by introducing a coexistence flag in each report.

Proposal 6: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s) and an indication about this in SHR and SPR.