3GPP TSG RAN WG2 Meeting #130	R2-2503689

St Julian, Malta, May 19 - 23, 2025

Title:	IoT-NTN TDD mode SI scheduling and UE procedures

Source: 	Iridium, CCL

Document for:	Discussion and Decision

Agenda Item:	8.17

# Introduction

The IoT-NTN TDD mode work item was approved in RAN#105 [3] and later revised in RAN#106 [1] following the conclusion of the RAN1 study phase. The revised WID outlines the following objectives for RAN2:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. Other necessary impacts on higher layers [RAN2]

In RAN2#129bis following agreements were made [6]:

Agreements: 1. Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. 2. In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell. 3. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes) 4. When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe 5. When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe 6. For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

For NB-IoT frame structure type-1 to be utilized on the same frequency in both uplink and downlink, separate spectrum time is allocated for each, separated by a time gap. The period during which uplink operations are allowed is referred to here as the active uplink with duration of U subframes, and the period during which downlink operations are allowed is referred to as the active downlink with duration of D subframes in Figure 1. In RAN#106 it was agreed that the periodicity of the active uplink and downlink times are fixed for NGSO satellite system operating in L-band (1616 – 1626.5 MHz) at 90ms with N=9. There is no RF transmission or reception during inactive times, resulting in discontinuous downlink transmissions. It is also agreed that duration of uplink and downlink active time i.e. D=U=8ms and the gap from the end of downlink active time to the start of next uplink active are fixed for MSS spectrum spanning 1616.0 to 1626.5 MHz.

Figure 1 IoT-NTN TDD mode with HD-FDD frame structure

In RAN1#120, it is agreed that any subframes within inactive downlink and uplink times are not considered valid NB-IoT downlink or uplink subframes, respectively.

The “non-D” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the IoT-NTN TDD structure. Similarly, the “non-U” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the IoT-NTN TDD frame structure.

In this contribution, we present our views on necessary adaptations required for SI scheduling and RACH procedure to work in IoT-NTN TDD mode. We have also added clarification on aspects like MAC timer and RA-RNTI handling.

# Discussion

## Scheduling

In this section, we present our views on various scheduling aspects covering MIB-NB, SIB1-NB, SI, NPDCCH, NPDSCH, NPUSCH and NPRACH with IoT-NTN TDD frame pattern.

#### SIB1-NB

In NB-IoT FDD mode, SIB1 uses a fixed schedule with a periodicity of 2560ms. It is transmitted on subframe 4 of every other frame and 8 subframes make up one transport block. The starting frame for the first transmission of the SIB1 is derived from the cell PCID.

The transport block size and the repetition rate of SIB1 is configurable, ranging from 208 to 680 bits, and 4, 8 or 16 block repetitions within a 2560ms period.

The IoT-NTN TDD frame pattern limits the number of SIB1 subframes a UE can see within a SIB1 periodicity window of 2560ms. Table 1 shows the maximum number of SIB1 subframes that are visible and can be soft combined within a SIB1 periodicity window of 2560ms.

| SIB1 Repetitions | Maximum number of SIB1 subframes |
| --- | --- |
| 4 | 3 |
| 8 | 7 |
| 16 | 15 |

Table 1 Maximum number of SIB1 subframes visible within SIB1 period of 2560ms in IoT-NTN TDD mode with N=9 and D=8

The link-level simulation results for SIB1-NB decoding performance were provided in our contribution paper [4], which was discussed at RAN1#120. The observations and proposals are captured here.

Observation 1: When operating in IoT-NTN TDD mode, the SIB1 repetition of 16 radio frames works even with the maximum allowed transport block size of 680 bits with adequate link budget margin of 3.91 dB and 4.51 dB for LEO-600 and LEO-1200 respectively.

Observation 2: When operating in IoT-NTN TDD mode, SIB1 repetition pattern should be configured as 16 radio frames to allow every alternate active downlink opportunity to carry SIB1 subframes for optimal SIB1 decoding performance and acquisition time.

Observation 3: When operating in IoT-NTN TDD mode, with SIB1 repetition pattern of every 16 RFs, maximum delay in receiving SIB1 is 2520ms.

Proposal 1: When operating in IoT-NTN TDD mode, there is no need to update the SIB1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

#### System Information (SI) Scheduling

In RAN1#120 it is concluded that, SI scheduling aspect is decided by RAN2.

As per RAN1#120: Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any. Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications.

The SI messages are transmitted within periodically occurring time domain windows called SI-windows using scheduling information provided in SIB1. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap.

Within the SI-window, the corresponding SI message can be transmitted several times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS. Any TBS higher than 120 bits uses 8 consecutive NB-IoT downlink subframes.

Every SI window starts at subframe#0 which gets postponed to subframe#1 as subframe#0 is reserved for NPBCH.

With IoT-NTN TDD mode frame pattern, it is possible that the SI window start time falls in the inactive downlink time. This can be resolved by postponing the SI window to the next active downlink opportunity.

Observation 4: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Proposal 2: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Observation 5: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks within a single downlink active time opportunity when SI TBS is greater than 120 bits.

Proposal 3: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

##### Overlap Within SI Window

SI transmission postponement to the next active downlink opportunity could result in overlap with the next repetitions within the SI window. This can be resolved by truncating SI transport block transmission if it overlaps with the next SI repetition. As illustrated in Figure 2, transmission of first repetition gets truncated at SFN 8 where start of second repetition is scheduled. Thus, next downlink active time carries subframes from the second repetition instead of carrying the postponed last three subframes of the first repetition.

Figure 2 Truncation of SI repetition with repetition pattern every 8th radio frame

In case of overlap, truncating the overlapped part of the first transmission is preferred over allowing the first transmission and disabling the initial overlapped part of the second transmission. This approach simplifies UE implementation, as it can schedule receiving the next SI message from the current time without needing to check past SI transmissions that may have been postponed.

Observation 6: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 7: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time results in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Proposal 4: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

##### SI Window Overflow

The SI transmission postponement to the next downlink active time could also result in SI transmission in the subframes outside of the configured SI window. This could be resolved by proposing changes in (36.331, 18.1.0) to truncate any SI transmissions outside of the SI transmission window as shown in Figure 3.

Figure 3 SI window overflow shall not be allowed

Observation 8: With IoT-NTN TDD mode frame pattern, the postponement of transmission of SI transport block to the next downlink active time can result in SI transmission in the subframes outside of the configured SI window.

Proposal 5: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 6: In IoT-NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows:

1> if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB, and SystemInformationBlockType1-NB, and in the IoT-NTN TDD mode excluding subframes outside of active downlink time. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

In the IoT-NTN TDD mode, if postponement of SI subframes due to downlink inactive time results in overlap with the subframes of the next SI repetition, or falls outside SI window, SI transmission should be truncated to the last non-overlapping subframe or last subframe within the SI window.

#### NPDCCH Scheduling

RAN1 has not yet decided on whether a new NPDCCH configuration parameter is required for IoT-NTN TDD mode but in RAN1#120 it is agreed that NPDCCH and NPDSCH are scheduled only in active downlink subframes by means of postponement to next valid NB-IoT downlink subframe.

The following relevant agreements are made in RAN1#120bis:

Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc.

In IoT-NTN TDD mode, with the existing set of configuration parameters, it is possible that NPDCCH search spaces can fall within the downlink inactive times. However, with the above RAN1#120 agreement, these search spaces shall be postponed to the next downlink active time. Based on our understanding, the existing set of NPDCCH configuration parameters can work for IoT-NTN TDD mode even if the NPDCCH period is scaled up by some scaling factor. This scaling factor need not be signaled to the UE but can specified as a fixed value for a EUTRA band supporting IoT-NTN TDD mode.

Proposal 7: In IoT-NTN TDD mode, there is no need to introduce additional NPDCCH ASN.1 configuration parameter.

Observation 9: In IoT-NTN TDD mode, if NPDCCH period scaling is agreed in RAN1, it can be specified as a fixed value in RAN1 specification.

## UE Procedures

#### Random Access Procedure

In RAN1#120bis following agreements are made related to NPRACH configuration parameters:

Agreement NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU. - NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported

In FDD mode, NPRACH opportunities can be configured with periodicities of 40ms, 80ms, 160ms, 240ms, 320ms, 640ms, 1280ms, and 2560ms, none of which are multiples of 90ms. The current NPRACH configuration parameters do not allow for full alignment with IoT-NTN TDD mode uplink active time occurrences. In RAN1#120bis, for IoT-NTN TDD mode, it is agreed that NPRACH transmissions are postponed to the next uplink active time subframes, where the unit of postponement is one Preamble Resource Unit (PRU). It is also agreed that NPRACH periodicities of 40ms and 80ms are not supported; instead, new periodicities of 90ms and 180ms are introduced.

Proposal 8: For IoT-NTN TDD mode, update the RRC ASN.1 IE NPRACH-ConfigSIB-NB to include new NPRACH periodicities of 90ms and 180ms.

Proposal 9: For IoT-NTN TDD mode, add a note to RRC ASN.1 parameters description that NPRACH periodicity of 40ms and 80ms are not supported if the new ASN.1 only provides additional periodicities instead of full supported set.

##### RA-RNTI Calculation

During the RACH procedure, after transmission of the Random-Access Preamble, the UE starts monitoring NPDCCH common search space to check for the RAR scrambled with the RA-RNTI. For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random-Access Preamble is transmitted, is computed as:

RA-RNTI=1+floor SFNid4+256*carrier_id

where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier.

In IoT-NTN TDD mode, subject to agreement in RAN1, if NPRACH opportunities that fall within inactive uplink times (non-U subframes) are postponed to the next active uplink times (U subframes), the calculation of RA-RNTI becomes ambiguous. The eNodeB calculates RA-RNTI based on the SFN of the first radio frame in which the Random-Access Preamble is received. Therefore, the UE should calculate RA-RNTI based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted after the postponement, not the actual occurrence of the NPRACH opportunity.

Proposal 10: In IoT-NTN TDD mode, The RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted after the postponement, not the actual occurrence of the NPRACH opportunity.

#### MAC Timers

In RAN2#129bis the following relevant agreements are made:

Agreements: Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes)

There are various protocol timers in the MAC which are specified in multiples of NPDCCH period (pp). These timers are copied here for ease of reference:

logicalChannelSR-ProhibitTimer-r13 = ENUMERATED {pp2, pp8, pp32, pp128, pp512, pp1024, pp2048, spare}

PeriodicBSR-Timer-NB-r13 = ENUMERATED {pp2, pp4, pp8, pp16, pp64, pp128, infinity, spare}

RetxBSR-Timer-NB-r13 =ENUMERATED {pp4, pp16, pp64, pp128, pp256, pp512, infinity, spare}

pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

mac-ContentionResolutionTimer-r13 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

C-DRX related timers:

onDurationTimer-r13 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, spare}

drx-InactivityTimer-r13 ENUMERATED {pp0, pp1, pp2, pp3, pp4, pp8, pp16, pp32}

drx-RetransmissionTimer-r13 ENUMERATED {pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24, pp33,}

drx-ULRetransmissionTimer-r13 ENUMERATED {pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24, pp33, pp40, pp64, pp80, pp96, pp112, pp128, pp160, pp320}

These timers are not counted in terms of postponed NPDCCH search spaces around NB-IoT downlink subframes. If the NPDCCH period is lower than 64, not all the allowed values for these timers are practical and can have a negative impact on the protocol performance.  For instance, with the minimum NPDCCH period value of 4ms, timer values lower than pp16 are not functional in IoT-NTN TDD mode.

Network operators are expected to configure practical values for the NPDCCH period, using a value of 64ms or higher. For the upcoming RAN1#121 meeting, we have proposed either to provide network configuration guidance or to make it explicit in the specification that, for the IoT-NTN TDD mode, the minimum allowed value of the NPDCCH period is 64ms instead of the current minimum allowed value of 4ms.

In the current NB-IoT specification, while calculating the absolute time for MAC timers specified in NPDCCH periods, invalid subframes are included in the timer duration. For example, if the MAC timer is specified as 2pp and the NPDCCH period is 64ms, the MAC timer duration is 128ms, which can include both downlink invalid and valid subframes. A similar approach can also work for IoT-NTN TDD mode if the configured NPDCCH period is greater than 64 ms.

Proposal 11: In IoT-NTN TDD mode, for MAC timers specified in NPDDCH periods, invalid subframes are considered part of the timer duration, just like the legacy NB-IoT FDD. In IoT-NTN TDD mode, it is assumed that minimum allowed NPDCCH period value is 64ms.

## EUTRA Overall Description

With the introduction of IoT-NTN TDD mode, 36.300 needs to be updated to include IoT-NTN TDD mode definitions and the associated changes as per the proposals below.

Proposal 12: A description of IoT-NTN TDD mode shall be added in 36.300 [§5] as follows:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP) as per Figure 1. This pattern is repeated every N radio frames. The values of N, D, U, [TBD for GP and start subframe for D] are fixed per IoT-NTN TDD band specified in [TBD]

Proposal 13: Anchor carrier definitions in 36.300 §3 needs updating:

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT-NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Proposal 14: In 36.300 §23.21 should be updated to reflect that NTN is applicable to both FDD and IoT-NTN TDD system:

In this release of the specification, NTN is only applicable to FDD and IoT-NTN TDD system.

Note: As per RAN2 VC instructions after the RAN2 #129 [5], the rapporteur of IOT NTN TDD WI has submitted the above three proposals for consideration into RAN1 120-bis and again into RAN1 121,  and the above three proposals brought up here are simply for completeness.

# Conclusion

In this contribution, we discussed the impact of IoT-NTN TDD mode on associated RAN2 procedures. The NPDCCH scheduling aspect is still under discussion in RAN1, but based on RAN1#120bis agreements we can assess the impact on RAN2, including any required ASN.1 changes. Our observations and proposals are listed below:

Observation 1: When operating in IoT-NTN TDD mode, the SIB1 repetition of 16 radio frames works even with the maximum allowed transport block size of 680 bits with adequate link budget margin of 3.91 dB and 4.51 dB for LEO-600 and LEO-1200 respectively.

Observation 2: When operating in IoT-NTN TDD mode, SIB1 repetition pattern should be configured as 16 radio frames to allow every alternate active downlink opportunity to carry SIB1 subframes for optimal SIB1 decoding performance and acquisition time.

Observation 3: When operating in IoT-NTN TDD mode, with SIB1 repetition pattern of every 16 RFs, maximum delay in receiving SIB1 is 2520ms.

Observation 4: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Observation 5: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks within a single downlink active time opportunity when SI TBS is greater than 120 bits.

Observation 6: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 7: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time results in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Observation 8: With IoT-NTN TDD mode frame pattern, the postponement of transmission of SI transport block to the next downlink active time can result in SI transmission in the subframes outside of the configured SI window.

Observation 9: In IoT-NTN TDD mode, if NPDCCH period scaling is agreed in RAN1, it can be specified as a fixed value in RAN1 specification.

Proposal 1: When operating in IoT-NTN TDD mode, there is no need to update the SIB1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

Proposal 2: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Proposal 3: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

Proposal 4: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

Proposal 5: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 6: In IoT-NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows:

1> if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB, and SystemInformationBlockType1-NB, and in the IoT-NTN TDD mode excluding subframes outside of active downlink time. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

In the IoT-NTN TDD mode, if postponement of SI subframes due to downlink inactive time results in overlap with the subframes of the next SI repetition, or falls outside SI window, SI transmission should be truncated to the last non-overlapping subframe or last subframe within the SI window.

Proposal 7: In IoT-NTN TDD mode, there is no need to introduce additional NPDCCH ASN.1 configuration parameter.

Proposal 8: For IoT-NTN TDD mode, update the RRC ASN.1 IE NPRACH-ConfigSIB-NB to include new NPRACH periodicities of 90ms and 180ms.

Proposal 9: For IoT-NTN TDD mode, add a note to RRC ASN.1 parameters description that NPRACH periodicity of 40ms and 80ms are not supported if the new ASN.1 only provides additional periodicities instead of full supported set.

Proposal 10: In IoT-NTN TDD mode, The RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted after the postponement, not the actual occurrence of the NPRACH opportunity.

Proposal 11: In IoT-NTN TDD mode, for MAC timers specified in NPDDCH periods, invalid subframes are considered part of the timer duration, just like the legacy NB-IoT FDD. In IoT-NTN TDD mode, it is assumed that minimum allowed NPDCCH period value is 64ms.

Proposal 12: A description of IoT-NTN TDD mode shall be added in 36.300 [§5] as follows:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP) as per Figure 1. This pattern is repeated every N radio frames. The values of N, D, U, [TBD for GP and start subframe for D] are fixed per IoT-NTN TDD band specified in [TBD]

Proposal 13: Anchor carrier definitions in 36.300 §3 needs updating:

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT-NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Proposal 14: In 36.300 §23.21 should be updated to reflect that NTN is applicable to both FDD and IoT-NTN TDD system:

In this release of the specification, NTN is only applicable to FDD and IoT-NTN TDD system.

Note: As per RAN2 VC instructions after the RAN2 #129 [5], the rapporteur of IOT NTN TDD WI has submitted the above three proposals for consideration into RAN1 120-bis and again into RAN1 121,  and the above three proposals brought up here are simply for completeness.

# References

RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

RP-242652, Status Report - WID on introduction of IoT-NTN TDD mode, Iridium, RAN#106, 2024-12, Madrid, ES.

RP-242415, “New WID on introduction of IoT-NTN TDD mode”, RAN #105, Melbourne, Australia, 2024-09

RP-2501614, “Feature lead summary #3 on IoT-NTN TDD mode”, 3GPP TSG RAN WG1 Meeting #120, Athens, Greece, February 17-21, 2025.

R2-2501333 “Report from Break-out session on NR-NTN and IoT-NTN”, 3GPP TSG RAN WG2 Meeting #129, Athens, Greece, February 17-21, 2025.

R2-2502983 “Report from Break-out session on NR-NTN and IoT-NTN”, 3GPP TSG RAN WG2 Meeting, Wuhan, China, April 7 - 11, 2025