3GPP TSG-RAN WG2 Meeting #130                                       R2-2503782

St Julian’s, Malta, May 19th - 23rd, 2025

Agenda Item:	8.3.2

Source:	Mediatek Inc.

Title:	Discussion on Functionality Management for AI Mobility

Document for:	Discussion, Decision

Introduction

In the last RAN2 #129 bis meeting, RAN2 agreed to apply the general LCM framework as the baseline for AI mobility.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

In this contribution, we focus on the applicability reporting and inference configuration. We first examine the general framework and procedure agreed upon in RAN2 AI-PHY. We then discuss whether it is applicable and the necessary modifications for the AI mobility use case.

Discussion

Applicability Report Procedure

For AI BM, RAN2 has agreed the applicable functionality report procedure as follows [1],

Figure 1: Illustration of general applicability report procedure 3.1-1[1]

The description of each step is given as follows.

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

The additional descriptions of Step 3 are agreed in [2] as given below.

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

The applicability procedures allow the UE to report whether the inference configuration provided by NW is applicable or not. In the first two steps, NW sends the UECapabilityEnqiry to UE, and UE sends the UECapablityInformation back to NW. The UE capability should be discussed in the normative work and is out of scope for this contribution. Based on the UECapabiblityInformation, NW can provide related inference configuration to UE via RRCReconfiguration in Step 3. Two different options are considered.

- Option A: NW provides full inference configurations.

- Option B: NW provides inference parameters.

In Option A, full inference configuration is proactively sent to UE, meaning all the necessary information to perform AI inference is directly transmitted to UE directly in Step 3. In Option B, NW initially provides only the inference parameters, which contain the necessary information for UE to report its applicability. After receiving the applicability report, the full inference configuration is then reactively transmitted to UE based on the applicability report. Following the agreements in AI-PHY, both options should be supported for AI mobility use cases. The detailed content of inference configuration and inference parameters can be FFS.

With the configuration in Step 3, UE can determine the applicability based on

- Full inference configurations/one or multiple sets of inference parameters

- NW-sided additional conditions, e.g., associated ID (if provided)

- UE-sided additional conditions, e.g., available power, memory, etc.

- Available AI model

In addition to the inference configuration/parameters, NW may provide additional NW-sided information such as associated ID. It can help the UE identify the relation between the inference condition and training condition (where the training condition affects the available AI models on the UE side). Discussion of the associated ID will be provided in the later section of this contribution. Besides the information provided by NW, UE will also consider its own status, including the power, memory status, and the available AI model to determine the applicability report.

If Option A is applied in Step 3, the applicability report identifies the UE applicability for specific inference configurations. If Option B is applied, it identifies the UE applicability for all possible inference configurations related to the given inference parameters. In both cases, multiple applicability corresponding to different full inference configurations/inference parameters is allowed in an applicability report. The detailed content of the applicability report is FFS.

After receiving the applicability report, NW sends the RRCReconfiguration to UE in Step 5. If full inference configuration is transmitted in Step 3 (i.e., Option A), it is up to NW implementation to update the configuration or not. Otherwise (i.e., Option B) detailed inference configurations are transmitted to UE in Step 5. Since NW has received the UE applicability report, the inference configurations beyond UE applicability are not expected to be configured.

Proposal 1: The general applicability report procedures agreed upon in RAN2 AI-PHY can be applied as the baseline for AI mobility use cases. RAN2 needs to discuss the necessary information required in the full inference configuration and the inference parameters corresponding to Option A and Option B for AI mobility.

Inference Configuration

In the last meeting, RAN2 agreed to consider the RRM measurement framework as the baseline for AI mobility inference configuration as given below.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

In this section, we discuss the necessary modification of the RRM measurement framework for AI mobility inference configuration. As mentioned, there are two options for the configuration in Step 3. We will separate the discussion in the following two sub-sections.

Full Inference Configuration

The legacy RRM measurement configuration contains the necessary information for measurement reports. Here we list part of the related components for the AI mobility inference configuration as follows.

- Measurement object (MO)

- Report configuration

- Measurement identities

The information of measurement targets is carried by MO, which includes the timing and frequency locations, cells list, etc. The report configuration contains the report triggering condition, e.g., event-triggered, periodical, and the report quality, e.g., RSRP, SINR, RSSI (via ReportQuantityCell). A measurement/report task is identified by a measurement ID which contain one MO ID and one report configuration ID. This framework can also be used for AI mobility inference configuration.

However, it is clear that in AI mobility inference configuration, in addition to the MO, we need to identify the information of the inference target, i.e., the predicted object. It should contain the timing and frequency location of the predicted target. It should also include additional information for different sub-use cases, for example, the prediction window (PW) length for Temporal domain case A, and the skipping pattern and MRRT for Temporal domain case B.

Observation 1: For UE-sided model AI mobility, NW needs to configure information about the predicted target to UE. This may include the timing and frequency location of the predicted target and the related information to perform inference such as PW length, skipping pattern, MRRT, etc.

Similar to the legacy measurement, the report configuration provides the measurement report triggering condition and the report quantity, however, for AI mobility use cases, the measurement report can be generated based on not only real measurements but also predicted results. Also, similar to the measurement ID, for AI mobility inference, we can use “inference ID” to figure out the mapping of an MO, a predicted object, a report configuration, and an inference report configuration. The overall configuration structure can be shown in the following figure where the inference configuration contains the following components:

- Measurement object: Describes the information of measurement/observed target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report. The measurement report can be generated based on both real measurement and inference results.

- Predicted object: Describes the information of inference/predicted target.

- Inference identity: Provides the mapping of a measurement object, a predicted object, a report configuration, and an inference report configuration.

Figure 2: Overall structure of inference configuration.

Proposal 2: RAN2 consider the inference configuration which contains

- Measurement object: Describes the information of measurement/observed target.

- Predicted object: Describes the information of inference/predicted target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report.

- Inference identity: Provides the mapping of a measurement object, a predicted object, and a report configuration.

Proposal 3: The predicted object contains

- Timing and frequency information, e.g., smtc, ssbfrequency, subcarrier, etc.

- PW length (for temporal case A)

- Skipping pattern, MRRT (for temporal case B)

- Measured and predicted beam/cell pattern, MRRS (for spatial domain prediction)

The details of the content can be FFS.

With the inference configuration, UE can figure out the AI mobility use case, e.g., Temporal domain case A, Temporal domain case B, Spatial domain, …etc., from the information carried by measurement object, predicted object, and report configuration directly, without additional explicit description.

Observation 2: Based on the measurement object, predicted object, and report configuration, UE can realize the task of AI inference (the sub-use case of AI mobility) without explicit description.

Inference Parameters

As mentioned in the previous section, NW may configure full inference configurations (Option A) or only inference parameters (Option B) in Step 3. The inference parameters only provide the necessary information for the applicability report, it may contain

- PW length (for temporal case A)

- Skipping pattern, MRRT (For temporal case B)

- Measured and predicted beam/cell pattern, MRRS (For spatial domain prediction)

- Measured and predicted frequency (For inter-freq domain prediction)

The inference parameters do not indicate the specific measurement and predicted target, i.e., MO and predicted object. Instead, they describe the necessary capability for AI inference. The PW length, skipping pattern, MRRT, measured and predicted beam/cell pattern, and MRRS indicate the AI model to be used. The associated ID identifies the expected inference condition, and UE can use it to check whether the available AI model is suitable (or matched for the training condition) for current usage.

Proposal 4: RAN2 consider “inference parameters” in measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain

- PW length (for temporal case A)

- Skipping pattern, MRRT (for temporal case B)

- Measured and predicted beam/cell pattern, MRRS (for spatial domain prediction)

- Measured and predicted frequency (For inter-freq domain prediction)

The detailed configuration is FFS.

Associated ID

The NW additional information, e.g., associated ID, has been discussed in the AI-PHY to help UE understand the correlation between the current inference condition and the training condition. This correlation may determine whether the AI model in UE is suitable for the current inference task or not. There may be some NW information that NW does not want to reveal explicitly. For AI-PHY, they discuss that NW provides the associated ID instead. When the associated IDs are the same (or matched), it implies the inference and training condition are the same or similar (and the AI model to can be used to perform inference). This information helps UE check whether their AI model is suitable or out of date.

Even though the intention is clear, what kind of information should be covered by using the associated ID is still vague in the AI mobility use case. We believe that before studying the impact of associated ID, we need to determine the necessity, i.e., what kind of NW information will affect the performance of the AI model that NW does not want to reveal. In our understanding, one possible factor would be the antenna downtilt angle, which may impact the coverage of cells and thus affect the mobility behavior. We suggest RAN2 examine the impact of downtilt angle as the starting point.

Observation 3: NW antenna downtilt angle may impact the coverage of cells and thus affect the mobility behavior. It may be one of the possible factors that will affect the AI mobility performance and be covered by the associated ID implicitly.

Proposal 5: RAN2 identifies the NW-sided assistance information required for each use case, for example, the NW antenna downtilt angle. Others can be FFS. FFS those NW-sided assistance information should be provided explicitly or covered implicitly by an associated ID.

Conclusion

Applicability Report Procedure:

Proposal 1: The general applicability report procedures agreed upon in RAN2 AI-PHY can be applied as the baseline for AI mobility use cases. RAN2 needs to discuss the necessary information required in the full inference configuration and the inference parameters corresponding to Option A and Option B for AI mobility.

Inference Configuration:

Full inference configuration:

Observation 1: For UE-sided model AI mobility, NW needs to configure information about the predicted target to UE. This may include the timing and frequency location of the predicted target and the related information to perform inference such as PW length, skipping pattern, MRRT, etc.

Observation 2: Based on the measurement object, predicted object, and report configuration, UE can realize the task of AI inference (the sub-use case of AI mobility) without explicit description.

Proposal 2: RAN2 consider the inference configuration which contains

- Measurement object: Describes the information of measurement/observed target.

- Predicted object: Describes the information of inference/predicted target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report.

- Inference identity: Provides the mapping of a measurement object, a predicted object, and a report configuration.

Proposal 3: The predicted object contains

- Timing and frequency information, e.g., smtc, ssbfrequency, subcarrier, etc.

- PW length (for temporal case A)

- Skipping pattern, MRRT (for temporal case B)

- Measured and predicted beam/cell pattern, MRRS (for spatial domain prediction)

The details of the content can be FFS.

Inference parameters:

Proposal 4: RAN2 consider “inference parameters” in measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain

- PW length (for temporal case A)

- Skipping pattern, MRRT (for temporal case B)

- Measured and predicted beam/cell pattern, MRRS (for spatial domain prediction)

- Measured and predicted frequency (For inter-freq domain prediction)

The detailed configuration is FFS.

Associated ID:

Observation 3: NW antenna downtilt angle may impact the coverage of cells and thus affect the mobility behavior. It may be one of the possible factors that will affect the AI mobility performance and be covered by the associated ID implicitly.

Proposal 5: RAN2 identifies the NW-sided assistance information required for each use case, for example, the NW antenna downtilt angle. Others can be FFS. FFS those NW-sided assistance information should be provided explicitly or covered implicitly by an associated ID.

Reference

[1] 	R2-2407848 LS on applicable functionality reporting.docx, RAN2

[2] 	3GPP RAN2#129 meeting chair notes