3GPP TSG-RAN WG2 Meeting #130	R2-2503793

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.7.4.1

Source:	China Telecom

Title:	Discussion on Remaining Issues on LCP Enhancements

Document for:	Discussion and Decision

- Introduction

In the last RAN2#129bis meeting, the agreements on LCP enhancement were made as follow.

Working assumption (to be revisited next meeting): No Bj enhancement is introduced. It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

After the meeting, the running MAC CR for XR were discussed, and leaving the following open issues related to the LCP enhancements.

- [MAC-01] Impact of priority adjustment on SR priority determination

- [MAC-02] Impact of congestion (i.e. PSI-based SDU discard) on priority adjustment

- [MAC-13] Priority fallback with consideration of PDU Set integrated handling

The paper aims to discuss the above issues related to the LCP enhancements and provide some considerations.

2 Discussion

In the previous meetings, it has agreed that the additional LCP priority will be used for UL grant priority determination in intra-UE prioritization. But it is undetermined whether the additional LCP priority should be used for SR priority determination in intra-UE prioritization. Per TS38.321, the mechanism of intra-UE prioritization procedure is shown as follow.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

It can be observed that the priority of SR is determined by the priority of logical channel that triggers SR. If the LCH priority-adjusted data is in the corresponding logical channel, it is more reasonable to apply the additional priority for the SR. When the SR overlaps with other UL grant, additional priority will be used for SR priority determination in intra-UE prioritization. Otherwise, the delay-critical data may not be transmitted timely.

In the legacy, there is no LCP procedure for the LCH when SR is triggered. To apply the additional priority for the SR, the additional priority LCH should be specified during the SR procedure, which has limited impact on the specification.

Proposal 1: Additional priority is used for SR priority determination with the limited specification impact.

To support importance-based packet discard for XR, a PDU Set Importance (PSI) indication is introduced. Specifically, the PSI indicates the relative importance of a PDU set compared to other PDU sets within the same QoS flow, which can be used to enable PDU set discard in congestion scenarios.

In case of uplink congestion, the network instructs the UE to perform the PSI based discard via PSI-based SDU discard activation/deactivation MAC CE. If the PSI based discard is configured for a DRB, the discard timer​​ for the low-importance PDCP SDUs (i.e. discardTimerForLowImportance) is started. Upon the PSI-based discarding is activated, the UE only supports DSR reporting for high important data.

On the other hand, once the PSI based discard is activated, it means that the LCH is in congestion. In this case, if there are two LCHs and the priority of LCH1 and LCH2 are p1 and p2 (p1 > p2), respectively. If the priority of LCH2 is upgraded to p1 due to the delay-critical data, the LCH2 will complete the resource with LCH1, which may worsen the congestion, especially when the LCH1 has more importance data than LCH2. Therefore, to guarantee the high importance data transmission, the priority adjustment for delay-critical data should also consider the data importance into account. In other words, in case of UL congestion, LCP adjustments introduced for delay-critical data are only applicable for important data.

Proposal 2: In case of UL congestion, LCP adjustments should take into account both the remaining time and the data importance.

In Rel-18, the PSIHI (PDU Set Integrated Handling Information) is introduced to indicate whether all PDUs of the PDU set are need for application layer information reconstruction. Taken the remaining time into consideration, whether the PDCP SDUs in the same PDU set should use one timer or different timers need further discussed. Obviously, using different timers for different PDCP SDUs may introduce extra specification complexity. For instance, in the same PDU set, the remaining time of some PDCP SDUs may smaller than threshold, while the others may larger than threshold. In addition, in case the additional priority is applied to the PDU set and all delay-critical PDCP SDUs are transmitted, whether the priority should fallback to the legacy one should also be specified. Therefore, applying multiple timers for PDCP SDUs in the same PDU set may be very complicated. To reduce the specification complexity, if the PDU set integrated handling is applied, only one timer will be used for the PDU set to reduce the complexity.

Proposal 3: Only one timer is used for the PDU set to reduce the complexity.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: Additional priority is used for SR priority determination with the limited specification impact.

Proposal 2: In case of UL congestion, LCP adjustments should take into account both the remaining time and the data importance.

Proposal 3: Only one timer is used for the PDU set to reduce the complexity.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China