3GPP TSG-RAN WG2 Meeting #130			         R2-2503837

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.4.2

Source: 	LG Electronics Inc.

Title:         	Remaining issues on LP-WUS in RRC IDLE or INACTIVE

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss remaining open issues on the LP-WUS operation in RRC_IDLE/INACTIVE.

Discussion

[Open issue 38304-5] UEs expecting MBS group notification

Since the group paging for MBS, i.e. MBS group notification, and the dedicated paging are transmitted in the same PO, UEs expecting MBS group notification are not allowed to monitor PEI and required to monitor the paging occasion, as shown in below:

7.2.1 Paging Early Indication reception The UE may use Paging Early Indication (PEI) in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI (except for the UEs expecting MBS group notification) can monitor PEI using PEI parameters in system information according to the procedure described below.

For the same reason, UEs expecting MBS group notification should monitor the paging occasion regardless of the LP-WUS in order to ensure no MBS group notification is missed.

Proposal 1	UEs expecting MBS group notification does not monitor LP-WUS.

[Open issue RRC-8/38304-4] FFS on whether/how to reduce the threshold number for LP-WUS/WUR

In the last meeting, we started discussion on reducing the total number of thresholds that the network needs to configure for LP-WUS operation by introducing a common entry/exit condition across different LP-WUS related procedures. While no conclusion was reached, we observed that companies were considering the following two options:

- Option 1. merge LPWUS monitoring and serving cell offloading

- Option 2. merge LPWUS monitoring and Rel-19 RRM relaxation

Since the serving cell offloading typically uses a higher threshold than R19 RRM measurement relaxaiton, if all three procedures - LP-WUS monitoring, serving cell offloading, and R19 RRM relaxation- are configured under option 1, the UE would be unable to monitor LP-WUS while the R19 RRM relaxation.

However, even if the serving cell quality is not sufficient to perform the RRM offloading to LR, there is no reason to prevent the UE from monitoring LP-WUS if the LP-WUS reception quality remains reliably high during RRM relaxation. Therefore, if we really need to reduce the number of thresholds for LP-WUS, we prefer to go with option 2.

That said, the exit condition for Rel-19 RRC relaxation is still FFS [Open issue RRC-7/ 38304-3]. If it is eventually defined as ‘not fulfilling the entry condition’, this alone could help reduce the threshold complexity. Therefore, we propose to first introduce a unified entry condition for LP-WUS monitoring and R19 RRM relaxation.

|  | LP-WUS monitoring | Serving cell offloading | R19 RRM relaxation |
| --- | --- | --- | --- |
| Entry condition | MR (M), LR(Op) | MR (M), LR(Op) | MR(M), LR(Op) |
| Exit condition | LR (M) | LR (M) | FFS |

Proposal 2	Support a unified entry condition for LP-WUS monitoring and Rel-19 RRM relaxation.

The purpose of this discussion is not to reduce the signalling overhead required for LP-WUS configuration, but rather to reduce the total number of thresholds that the network needs to configure when operating LP-WUS. Therefore, there is no need to consider a separate solution for cases where Rel-19 RRM relaxation is not configured. In other words, if the network configures LP-WUS monitoring and RRM offloading without enabling Rel-19 RRM relaxation, it is appropriate to define separate entry/exit conditions for those two features.

Proposal 3	Do not unify the entry/exit condition for LP-WUS monitoring and RRM offloading. (I.e. when network configures LP-WUS monitoring and RRM offloading without Rel-19 RRM relaxation, separate entry/exit conditions are configured for those two features.)

[Open issue RRC-2] Whether entry/exit condition is mandatory or optional

There has been extensive discussion on whether the entry/exit condition of using LP-WUS is mandatory or optional present over the past several meetings, but RAN2 have yet to reach a conclusion. The proponents of the optional entry/exit condition for using LP-WUS are divided into three camps, each with a different intended UE behaviour when the exit/entry condition is absent in the LP-WUS related configuration, as follows:

- Camp 1. Leave it up to UE implementation, if the entry/exit conditions are absent

- Camp 2. Use default entry/exit condition, if the entry/exit conditions are absent

- Camp 3. Assume that the entry condition is always met, if the entry/exit conditions are absent

In fact, this is not simply an issue of whether to support the optional configuration. Rather, we need to discuss separately whether to support UE behaviour that each camp considers when the entry condition is absent.

Issue #1: Whether the decision on the entry/exit condition of using LP-WUS can be up to UE

The first camp argues that smart UEs can make sensible UE implementation when to start/stop LP-WUS monitoring. However, if UE conservatively determines the entry/exit condition to ensure reliable LP-WUS reception, the MR may lose the opportunity to enter the ultra-deep-sleep and the power saving gain from using LP-WUS will somewhat decreases. Conversely, if the threshold for the entry condition is set too low, or if the threshold for the exit condition is set too high, the UE will monitor LP-WUS with low reliability and may miss paging. Since only a few bits are expected to configure the entry/exit condition, it seems unnecessary to tolerate such potential issues.

Issue #2: Whether to support default entry/exit condition of using LP-WUS

Since the LP-WUS transmission operates on a cell basis, the appropriate entry/exit conditions may vary for each cell. We are not sure a single default condition can be used in any cell. Considering that only 8bits are needed to configure an RSRP/RSRQ threshold and the entry/exit condition would consist of up to twelve RSRP/RSRQ thresholds, signalling gain that can be achieved by not configuring the entry/exit conditions would be trivial.

Issue #3: Need for entry/exit condition of LP-WUS monitoring when LP-WUS and the cell have the same coverage

If the LR and MR coverage are identical, continuous LP-WUS monitoring by UE while camping on the cell would not cause any issue. However, specifying a separate UE behaviour solely to reduce a few bit in such an exceptional case is not considered desirable. Therefore, we propose to configure the entry/exit condition for LP-WUS monitoring as mandatory field in the LP-WUS configuration.

Proposal 4	The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration.

[Open issue RRC-12] Whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

It is also still FFS whether to support per UE de-activation of LP-WUS monitoring and we discussed two solutions for this issue:

- Solution 1. LP-WUS monitoring de-activation per UE via RRC release message

- Solution 2. LP-WUS monitoring de-activation per UE via NAS signalling

Another open issue is whether to allow UEs with emergency PDU session to use LP-WUS. Due to the long time offset between LP-WUS and the paging occasion, the paging latency may not be acceptable when UE has an emergency PDU session. In contrast to gNB, the CN is aware of whether the UE has an emergency PDU session. Therefore, adopting the solution 2 would allow both issues to be addressed.

Proposal 5	Support LP-WUS monitoring de-activation per UE via NAS signalling.

Conclusion

Proposal 1	UEs expecting MBS group notification does not monitor LP-WUS.

Proposal 2	Support a unified entry condition for LP-WUS monitoring and Rel-19 RRM relaxation.

Proposal 3	Do not unify the entry/exit condition for LP-WUS monitoring and RRM offloading. (I.e. when network configures LP-WUS monitoring and RRM offloading without Rel-19 RRM relaxation, separate entry/exit conditions are configured for those two features.)

Proposal 4	The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration.

Proposal 5	Support LP-WUS monitoring de-activation per UE via NAS signalling.