3GPP TSG-RAN WG2 Meeting #130			         R2-2503839

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.3

Source: 	LG Electronics Inc.

Title:         	Remaining issues on OD-SIB1 operation

Document for: 	Discussion and Decision

Introduction

In this contribution we discuss the open issues listed below:

In the summary of [POST129b][113][NES] 38.304

- FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

- FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304.

In the summary of [POST129b][111][NES] 38.331

- FFS Co-existence of SDT and OD-SIB1

- FFS reference for where are the details on how UE is obtaining SIB1, possibly RAN1 specification

- FFS Co-existence of SDT and OD-SIB1

- FFS depending SIBxx/UL WUS validity discussion details

- FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1.E.g. “The UE supporting OD-SIB1 in RRC_CONNECTED regards the stored SIB1 is the latest SIB1.”

Discussion

FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

The existing list of exclude-listed intra-/inter-frequency neighbouring cells, i.e. intraFreqExcludedCellList and interFreqExcludedCellList, are optional in SIB3 and SIB4 respectively. That is, the specification already supports the deployment scenario where there are no excluded cells for legacy UEs. In such a deployment, if OD-SIB1 cells are additionally operated, the legacy excluded cell list would consist of only the OD-SIB1 cells.

If Rel-19 excluded cell list can be configured as an empty list, the network would set a Rel-19 excluded cell list with no entries for OD-SIB1 capable UEs in this situation so that they can ignore the legacy excluded cell list based on it. However, if Rel-19 excluded cell list does not support empty list, similar to the legacy excluded cell list, then OD-SIB1 capable UEs should ignore the legacy excluded cell list irrespective of whether Rel-19 excluded cell lists being provided. Therefore, we propose to choose one of above solutions.

Proposal 1	For legacy excluded cell list issue, choose one of the following options:

- Option 1. UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided.

- Option 2. Rel-19 excluded cell list can be configured as an empty list (No new UE behaviour).

FFS Co-existence of SDT and OD-SIB1

If the cell re-selection criteria are fulfilled while SDT procedure is ongoing, the UE performs the cell re-selection and stop SDT. That is, performing cell re-selection is prioritized over the on-going SDT. To avoid impact on UE mobility, this principle should also apply to cell re-selection to OD-SIB1 cell.

Proposal 2	UE can initiate SIB1 request while SDT procedure is ongoing.

FFS reference for where are the details on how UE is obtaining SIB1, possibly RAN1 specification

RAN1 has defined the SIB1 monitoring window for OD-SIB1, during which the UE monitors for SIB1 after requesting it. If the UE behaviour for receiving the OD-SIB1 within the SIB1 monitoring window is clearly specified in the RAN1 specification, it should be sufficient for RAN2 specification to describe the UE behaviour simply by referencing the RAN1 specification as follows:

2> if acknowledgement for on demand SIB1 request is received from lower layers: 3> acquire the requested SIB1 message as defined in TS 38.213 [13];

Alternatively, RAN1 may expect the detailed UE behaviour related to OD-SIB1 acquisition to be captured in RRC. Therefore, we should first ask RAN1 whether they intend to capture the detailed UE behaviour in RAN1 specification or not. If RAN1 prefers this UE behaviour to be captured in RRC, we should describe it in RRC by reusing the existing UE behaviour for SI request as much as possible.

Proposal 3	Ask RAN1 first whether they will capture the details on how UE acquire OD-SIB1 in RAN1 specification.

FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304.

RAN2 has defined three cases in which the UE considers the OD-SIB1 cell as barred.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

The first and the second cases have already been captured in running CR for 38.331. There is no reason not to capture the last case as well. Since cell barring is a RAN2 behaviour, we cannot expect it to be captured in RAN1 specification.

If the detailed UE behaviour for OD-SIB1 acquisition is described in RAN1 spec only, it would not be appropriate to capture the third case in RRC as it is. Therefore, we suggest to first agree to capture the UE behaviour in third case in RRC, and then discuss how to capture it after concluding the previous issue.

Proposal 4	Capture explicitly the failure case of OD-SIB1 window expiry in 38.331, as other failure cases.

FFS depending SIBxx/UL WUS validity discussion details

The followings were agreed to allow the UEs in RRC_CONNECTED to request and acquire OD-SIB1 when T311 is running:

Agreements @RAN2#129 When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. The UE follows the legacy validity principle of stored SIB. Agreements @RAN2#129bis SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

These agreements are captured in running CR TS 38.331 as follows:

3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x;

However, the legacy SIB validity check works only when the UE has a serving cell, because it is based on comparing the first PLMN identity, SI area ID, and the value tag of the SIB received from the current serving cell and those values associated with the stored SIB as shown in below.

TS 38.331 V18.5.1 3> else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell;

Since UE has no serving cell while T311 is running, UE cannot check the validity of the stored SIB X using the existing method.

Observation 1	Since UE has no serving cell when T311 is running, the UE cannot check the validity of the stored SIB X using the existing method.

Since the cell selection will be performed immediately after an RLF occurs in the PCell, the SIB X that was valid in the most recent serving cell is highly likely to still be valid when T311 is running. Therefore, it seems reasonable when T311 running, UE checks the validity of the stored SIB X by comparing the PLMN-Identity, systemInformationAreaID and valueTag of the last serving cell and those of stored SIB X.

Proposal 5	When T311 is running, if the first PLMN-Identity, systemInformationAreaID and valueTag received from the last serving cell are identical to those of the stored SIB X, UE considers the stored SIB X as valid.

FFS how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1.E.g. “The UE supporting OD-SIB1 in RRC_CONNECTED regards the stored SIB1 is the latest SIB1.”

Since the change of si-BroadcastStatus does not result in SI change notification, if this change is not captured, the UE in RRC_CONN (if CSS for other SI is configured in the active BWP) will try to acquire SIB1 before initiating SI request procedure and eventually consider the cell as barred due to no SIB1.

However, the UE supporting OD-SIB1 should consider the stored SIB1 is the latest SIB1 only just before initiating the SI request procedure to OD-SIB1 cell. The UE supporting OD-SIB1 should re-acquire SIB1 if the serving cell is not the OD-SIB1 cell, or upon reception of SI change notification even from OD-SIB1 cell. Therefore, it seems better to capture this change in 5.2.2.3.5 as a NOTE as follows:

NOTE: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest si-BroadcastStatus, if the UE has a stored valid version of od-SIB1-Config for PCell.

Proposal 6	Add a below NOTE in 5.2.2.3.5:

NOTE: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one, if the UE has a stored valid version of od-SIB1-Config for PCell.

Additional Issue: Triggering condition for OD-SIB1 request

RAN2 defined separate triggering condition for OD-SIB1 request for UEs in RRC_IDLE/INACTIVE or RRC_CONNECTED.

Agreements @RAN2#128 Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. Agreements @RAN2#129 Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

However, these two agreements are not captured at all in either running CR for 38.304 or 38.331. According to running CRs, the triggering condition for OD-SIB1 request is up to UE implementation. These agreements should be captured in TS 38.331 as follows:

2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE and if the cell satisfies the cell re-selection criteria as defined in TS 38.304 [20]; or 4> if the UE is in RRC_CONNECTED while T311 is running, and if the cell satisfies the cell selection criteria as defined in TS 38.304 [20]: 5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x;

Proposal 7	Capture the triggering conditions for OD-SIB1 request in 5.2.2.3.1.

Conclusion

Proposal 1	For legacy excluded cell list issue, choose one of the following options:

- Option 1. UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided.

- Option 2. Rel-19 excluded cell list can be configured as an empty list (No new UE behaviour).

Proposal 2	UE can initiate SIB1 request while SDT procedure is ongoing.

Proposal 3	Ask RAN1 first whether they will capture the details on how UE acquire OD-SIB1 in RAN1 specification.

Proposal 4	Capture explicitly the failure case of OD-SIB1 window expiry in 38.331, as other failure cases.

Proposal 5	When T311 is running, if the first PLMN-Identity, systemInformationAreaID and valueTag received from the last serving cell are identical to those of the stored SIB X, UE considers the stored SIB X as valid.

Proposal 6	Add a below NOTE in 5.2.2.3.5:

NOTE: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one, if the UE has a stored valid version of od-SIB1-Config for PCell.

Proposal 7	Capture the triggering conditions for OD-SIB1 request in 5.2.2.3.1.