3GPP TSG-RAN WG2 Meeting#130        			                                                               R2-2503868

St.Julians, Malta, May 19th – 23th, 2025

Agenda item:	8.2.3

Source:	ETRI

Title:	Discussion on Msg1 transmission for random access

Document for:	Discussion and Decision

# Introduction

In last RAN2#129bis meeting, followings are agreed for Msg1 in random access procedure[1].

Agreements on msg1 1. In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes 2. RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario.

In this paper, we discuss technical issues for Msg1 transmission for random access and propose our views.

# Discussions

## Configuration for Msg1 transmission

The device needs to select D2R resources for Msg1 transmission. To support this, it was agreed in the RAN2#126 meeting that the relevant information will be provided through the paging message as follows[2].

AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/ preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

In addition, in the RAN2#129 meeting, it was agreed that the paging message includes resource information for Msg1 transmission[3].

1. the A-IoT paging message can include a number of msg1 resources 2. From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

However, details on the specific parameters for Msg1 transmission were not discussed. Therefore, further discussion is needed for Msg1 transmission.

At the RAN1#120bis meeting, the following agreement was reached regarding the D2R resources for Msg1 transmission[4]. The D2R resources can be configured with multiple time/frequency domain resources. Therefore, one or more Msg1 transmissions can be performed during a random access procedure using the configured resources.

For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource (s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

Observation 1. One or more Msg1 transmissions can be supported on D2R transmission resource(s) for a single random access procedure.

There are two methods to indicate the resources for Msg1 transmission: by using a resource index or by representing the resource location as coordinates, respectively. In the resource index method, a unique number is assigned to each resource (e.g. access occasion, AO), and these indices are used to identify each resource. In the coordinate method, each resource is assigned a coordinate that represents its time domain position and frequency domain position, which are then used to indicate the resource. In the Figure 1, the CBRA resource AO#2 can be represented as ‘2’ when using a resource index, or as X = 1, Y = 2 when using resource coordinates.

Proposal 1. Indicating D2R resources for Msg1 transmission can be achieved by either assigning a unique resource index to each resource or representing the resource location as coordinates.

Random access can be classified into contention-free RA (CFRA) and contention-based RA (CBRA).

In the case of CFRA, dedicated resources are configured per device ID, so the device ID and the D2R resource have one to one mapping.

There are two methods for this configuration: explicit and implicit.

- The explicit method explicitly assigns resources to each device ID. In this approach, the device ID is directly linked to a specific resource—using either a resource index or coordinate—to indicate which resource is allocated.

- The implicit method sequentially maps devices to resources, starting from a predefined resource, where resources are assigned to CFRA devices based on their order and a starting point.

In the Figure 1:

- Using the explicit method, device 6 and device 9 are respectively assigned to resource b (AO#b) and resource a (AO#a).

- Using the implicit method, if the CFRA resource start point is set to AO#a, then device 9 and device 6 are sequentially assigned to resources AO#a and AO#b.

In contrast, in CBRA, multiple devices select one resource from a shared resource pool and transmit Msg1 using the selected resource. Therefore, the reader provides the device ID(s) targeted by CBRA, the Q value (which defines the window for random resource selection), and information about the resources to indicate the shared resource pool. The Q value may be configred considering the maximum number of D2R resources available for CBRA in a single paging round. The resources in the shared resource pool can be represented as either a range of the resource index or a range of resource coordinates, as mentioned earlier. In the Figure 1, devices with a group identifier (e.g., device#x) use a Q value and the shared resource pool(AO#0 ~  AO#7) to select a R2D resource for Msg1 transmission.

Proposal 2. For CFRA, the device ID maps to the dedicated D2R resource for Msg1 transmission, as indicated in the paging message.

Proposal 3. For CBRA, the device ID(s) maps to the shared D2R resource pool for Msg1 transmission in the paging message, and a 'Q' is also provided to the devices for random resource selection within shared D2R resource pool.

In CBRA, RN16 is transmitted in Msg1, whereas for CFRA, a device identifier and upper-layer data can be transmitted in Msg1. Consequently, the Msg1 content of CBRA and CFRA may be different, and accordingly, the size of Msg1 can also vary. This means different D2R resource sizes are allocated to CFRA and CBRA. Therefore, the paging should provide resource size for each case depending on the random access type.

Proposal 4. Different D2R resource sizes may be configured for Msg1 transmission for CFRA and CBRA.

Figure 1. An example of D2R resource configuration

## Device behavior

The device recognizes a paging round as the interval between receiving one paging message and the next subsequent paging message. Within a paging round, the device may receive one or more R2D trigger messages, each of which may initiate a random access procedure as shown in Figure 2. The new R2D message that was agreed upon in the last RAN2#129bis meeting is referred to as the R2D trigger message in this paper.

The R2D trigger message can be used to indicate the start of a new random access procedure. This message may be defined as either L1 signaling or an L2 message. However, in the RAN1#120bis meeting, it was agreed that scheduling information related to resource allocation for D2R transmission is handled via higher-layer signaling. Therefore, the R2D trigger message should be designed as MAC control signaling.

Proposal 5. The R2D trigger should be designed as MAC message.

There may be multiple times of random access opportunities within a paging round. In this case, we need to discuss whether each D2R resource structure has same or different across R2D trigger rounds. Considering different D2R resource structure for each R2D tirgger round, the R2D trigger message would need to provide the different configuration for D2R resource, which could lead to signaling overhead. Therefore, given that a paging round is not particularly long and there is no strong motivation to dynamically change D2R  resource structure for each R2D trigger round, it is reasonable to maintain a consistent D2R resource configuration throughout the paging round.

Proposal 6. The D2R resource configuration for Msg1 transmissions within a single paging round have identical.

Figure 2. paging round vs R2D trigger round

Upon receiving the configuration information for Msg1 transmission from the reader, a device proceeds to perform the random access procedure.

In the case of CFRA, a dedicated resource for Msg1 transmission is allocated to the paged device. Therefore, the device uses the configured resource to transmit its device identifier and upper-layer information. The device can identify the D2R resource using either a resource index or coordinate indicating resource location included in the paging message.

Proposal 7. For CFRA, the device transmits Msg1 on the dedicated resource indicated by the paging message.

In the case of CBRA, the device randomly selects a resource for contention-based Msg1 transmission. To do this, the device uses the Q valeu  as upper limit of selection window, and transmits Msg1 using the resource corresponding to selected value. There are two methods for selecting a random value:

- choosing a value from 0 to Q−1, or

- choosing a value from 0 to 2Q−1.

Considering the amount of signaling, Method 2 is more efficient as it allows the information to be represented using fewer bits. For example, if a paging round allocates up to 32 CBRA resources, Q can be set to 5.

Proposal 8. For resource selection for Msg1 transmission in CBRA, the device selects a random value from the range 0 to 2Q − 1 using the Q value provided in the paging message.

A device performing CBRA transmits Msg1 at the resource where the randomly selected value (q) becomes zero. To achieve this, the device decrements the selected random value ‘q’ by the number of CBRA resources configured in the paging message each time it receives an R2D trigger message. If the random value ‘q’ is less than or equal to the number of CBRA resources, the device selects a resource within the R2D message round and transmits RN16.

To determine the location of the Msg1 transmission resource, the device calculates the time domain resource location by dividing the random value ‘q’ by the number of frequency domain resources, and determines the frequency resource resource location by subtracting the product of the time index and the number of frequency resources from the random value ‘q’.

Proposal 9. For CBRA, the device decrements	 the selected random value ‘q’ by 1 each time a D2R resource for Msg1 transmission becomes available. When the random value reaches 0, the device transmits Msg1 (RN16).

Regarding the message type in Msg1, it is not necessary to include it since the reader already identifies the resources for Msg1 transmission in both CBRA and CFRA. Furthermore, considering resource usage aspect, the message type is not essential information in Msg1. Therefore, we think there is no strong reason to include the message type in Msg1.

Proposal 10. The message type is not included in Msg1.

# Conclusions

Based on the discussion in the contribution, we have the following observation and proposals:

Observation 1. One or more Msg1 transmissions can be supported on D2R transmission resource(s) for a single random access procedure.

Proposal 1. Indicating D2R resources for Msg1 transmission can be achieved by either assigning a unique resource index to each resource or representing the resource location as coordinates.

Proposal 2. For CFRA, the device ID maps to the dedicated D2R resource for Msg1 transmission, as indicated in the paging message.

Proposal 3. For CBRA, the device ID(s) maps to the shared D2R resource pool for Msg1 transmission in the paging message, and a 'Q' is also provided to the devices for random resource selection within shared D2R resource pool

Proposal 4. Different D2R resource sizes may be configured for Msg1 transmission for CFRA and CBRA.

Proposal 5. The R2D trigger should be designed as MAC message.

Proposal 6. The D2R resource configuration for Msg1 transmissions within a single paging round have identical.

Proposal 7. For CFRA, the device transmits Msg1 on the dedicated resource indicated by the paging message.

Proposal 8. For resource selection for Msg1 transmission in CBRA, the device selects a random value from the range 0 to 2Q − 1 using the Q value provided in the paging message.

Proposal 9. For CBRA, the device decrements the selected random value ‘q’ by 1 each time a D2R resource for Msg1 transmission becomes available. When the random value reaches 0, the device transmits Msg1 (RN16).

Proposal 10. The message type is not included in Msg1.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #129bis”, April. 2025

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #126”, May. 2024

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #129”, Feb. 2025

- Chair’s Note, “3GPP TSG-RAN WG1 Meeting #120bis”, Feb. 2025