3GPP TSG RAN WG2 #130          						R2- 2503871

Malta, Malta, May 19th – 23rd, 2025

Source:		Tejas Networks Ltd.

Title:		Discussion on A-IoT paging message format

Agenda item:		8.2.2

Document for:        Discussion and Decision

# Introduction

As discussed in TR38.769, the A-IoT devices receive Msg0 paging signal from reader or base station (BS), upon receiving Msg0, based on the Msg0 signal, one or multiple A-IoT devices transmit Msg1 which carries a unique random ID or actual device ID based on 2-step or 4-step contention-based random access (CBRA) or contention-free random access (CFRA).  Figs. 1(a) and 1(b) shows the 2-step and 4-step A-IoT communication process respectively.

The CFRA can be a 2-step process as shown in Fig. 1(a) as follows:

- Step A: Msg0 (R2D): Device receives an inventory command from the reader

- Step B: Msg1 (D2R): Device transmits UL data (such as device identifier, device type, etc.) to the reader

The CBRA can be a 4-step process as shown in Fig. 1(b) as follows:

- Step 0: Msg0 (R2D): Device receives an inventory command from the reader

- Intermediate step (R2D): A R2D trigger message to send Msg1

- Step 1: Msg1 (D2R): Device transmits a device random ID to the reader

- Step 2: Msg2 (R2D): Device receives an acknowledgment of the device random ID from the reader and scheduling information required to transmit Msg3

- Step 3: Msg3 (D2R): Device transmits UL data (such as device identifier, device type, etc.) to the reader

- Step 4 (optional): Msg4 (R2D): Device receives an ACK/NAK signal for Msg3 from the reader

Fig. 1(a): 2-step CFRA/CBRA.                                                  Fig. 1(b): 4-step CFRA/CBRA.

Fig. 2: System-level deployment scenario of D1T1-topology.

The system-level deployment scenario is shown in Fig. 2, where BS communicates with the core network (CN) and devices. The timing diagram of A-IoT communication between the reader and a device is shown in Fig. 3. Each transaction cycle may consist of one or multiple communication slots. At every communication slot or occasion, the reader initiates the communication by sending a paging signal and the slot ends when the device received acknowledgement of msg3 or the command response. Since the readers are not in sync, few devices may receive paging messages or service requests from multiple BSs, as shown in Fig. 2.

Fig. 3: Timing diagram of A-IoT communication involved multiple transactions.

# A-IoT Device Behaviour for Parallel Service Request

The following agreements were made for A-IoT device behaviour for parallel service request in RAN2#129-bis [1]:

Agreements

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

It needs to be further discussed for the scenario that different readers may send A-IoT paging messages, which are associated with the same service request from the CN, to the same device for response. If this scenario is in the scope, it needs to be further discussed, by considering the progress from all the WGs.

As presented above, in case the A-IoT paging message does not contain any identifier, it indicates that all the devices receiving the A-IoT paging message need to respond. In such scenario, if devices receive paging message signal from multiple readers, they have to decide on to whether to response to all the readers or to prioritize one reader. In related to that condition, the following has been studied in TR 38.769, section 6.3.3 for A-IoT paging identifier details [1]:

Consider the scenario depicted in Fig. 2, where multiple BSs are equally spaced, and the devices are randomly distributed in the region. If the A-IoT devices are deployed at the cell edge or corner of a cell, such devices can receive and decode DL signals from more than one BSs. For the device to identify from which reader the message was sent, the DL messages should also contain a base station ID or reader ID. The following are agreed in RAN2#129 [2]:

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Observation 1: Based on the deployment position, the A-IoT devices may get one or multiple service requests from one or multiple base stations.

Proposal 1: If the A-IoT device receives multiple service request in parallel, it will respond to the msg0 it receives first and start communicating with that reader. The parallelly received msg0 signals are discarded by the device until the ongoing process is completed successfully. Thus, Parallel service requests should have separate transaction ID to identify at the device side whether this is a different service request or same request from a different reader or same request from a same reader.

Example scenario:

Fig. 4: Device receives paging messages from multiple BSs/readers.

Consider reader 1 starts an inventory process for a specific transaction requested from the CN. Another reader starts another inventory process with some offset from the reader 1, as shown in Fig. 3. The inventory process may involve multiple communication slots. If a device successfully sends the information at one slot it will not respond to the next slots for the same transaction or service request from the same reader to free up the resources for the other devices. Responding to the same service request from the same reader may block the resources for the other devices. However, if the device receives the same service request from another reader (say, reader 2) it may respond to that reader.

The A-IoT communication between a device and multiple readers can be divided into two scenarios. Consider a scenario where the device is already involved in a service requested from reader 1, as shown in Fig. 6. If the device already responds to the initial service request received from one reader it will not respond to the new paging messages or service request from the same or different reader with the same or different transaction ID. Thus, the device will discard the subsequent service requests.

Consider another scenario where the device has completed the initial service request and goes to idle or listening mode (shown in Fig. 6) and the device has sufficient energy to complete another transaction. If the device receives another service request, it has to decide whether to respond to that request or not.

The device may receive query from the same or different readers for the same or different services, which can be divided into following cases:

Case-1: Device receives the same service request from the same reader

If the device has completed the previous transaction successfully and goes to idle or listening mode, it may discard the same service request from the same reader in the subsequent communication slots to reduce the probability of contention for the remaining devices. In that case, the device may compare the transaction ID stored in its memory with the transaction ID received in the paging message. If the transaction IDs are same, the device may choose to discard the service request.

Case-2: Device receives the same service request from different reader

If the device has completed the previous transaction successfully and goes to idle or listening mode, it may receive the same service request from a different reader in the subsequent communication slots. In that case, the device may compare the transaction ID stored in its memory with the transaction ID received in the paging message. If the transaction IDs are not same, the device may choose to respond the service request.

Case-3: Device receives different service request from the same reader

If the device has completed the previous transaction successfully and goes to idle or listening mode, it may receive another service request from the same reader in the transaction or inventory cycle. In that case, the device may compare the transaction ID stored in its memory with the transaction ID received in the paging message. If the transaction IDs are not same, the device may choose to respond the service request.

Case-4: Device receives different service request from different reader

If the device has completed the previous transaction successfully and goes to idle or listening mode, it may receive different service request from a different reader in the subsequent communication slots. In that case, the device may compare the transaction ID stored in its memory with the transaction ID received in the paging message. If the transaction IDs are not same, the device may choose to respond the service request.

The following agreements were made on NACK-based feedback in RAN2#129-bis [1]:

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

Observation 2: The inventory process may involve multiple paging occasions. Once a device has completed the initial service request and goes to idle or listening mode, it can decide whether to respond to the next paging request based on transaction ID and/or Reader ID.

Proposal 2: The reader should send an ACK signal if the Msg3 is successfully received at the reader. If the device doesn’t receive ACK and it receives next R2D trigger, it considers the transaction is failed.

Proposal 3: If the ongoing transaction is failed, it retransmits Msg1 after it receives the subsequent paging Msg0 for the same transaction. Until the ongoing transaction is successful, or reader specifically send a release message, the device will discard other Msg0 signals with different transaction ID.

Proposal 4: If a device successfully sends the information at one slot it will not respond to the next slots for the same transaction or service request from the same reader to free up the resources for the other devices.

Proposal 5: If a device successfully sends the information at one slot and goes to idle or listening mode, it may respond to the next slots for the same transaction or service request from a different reader.

Proposal 6: The R2D trigger message should contain the transaction ID so that the device can identify the trigger messages for that transaction.

The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

The following agreements were made on AIoT paging message in RAN2#129 [2]:

As the service ID or the transaction ID plays an important role in order to decide the device behaviour when the device is in idle or listening mode, the device needs to differentiate among the different cases of multi-reader or multi-paging scenarios.

If one correlation ID is dedicated to one service, two readers may have the same correlation ID for the same service. In that case, without out sending the reader ID, the device cannot differentiate that the same service is from the same reader or different readers. The following options can be adopted to solve this problem:

    - Option 1: Send both reader ID and correlation ID received from the CN as transaction ID. The reader ID can also act as an associate ID for that particular transaction cycle.

    - Option 2: Generate a new transaction ID from both the correlation ID received from the CN and the reader ID. The transaction ID can also act as an associate ID for that particular transaction cycle.

Observation 3: As the service ID or the transaction ID plays an important role in order to decide the device behaviour when the device is in idle or listening mode, the device needs to differentiate among the different cases of multi-reader or multi-paging scenarios.

Proposal 7: For the device to differentiate among the different cases of multi-reader or multi-paging scenarios, the transaction ID should be different for different reader for same service as well as it should be different for same reader for different services. The following options can be adopted to solve this problem:

    - Option 1: Concatenate reader ID and correlation ID received from the CN as transaction ID.

    - Option 2: Generate a new transaction ID from both the correlation ID received from the CN and the reader ID. FFS on operation to generate the Transaction ID from correlation ID and Reader ID.

Proposal 8: If reader sends multiple msg2 signals, the msg2 signal can carry an indication to convey the device if it is the last msg2 signal in this communication slot.

# A-IoT Paging Message Content and Paging Identifier Details

As to the A-IoT paging message, the identifier may be required to identify the device/group of devices in this trigger message (e.g., for the case of reaching a single or a group of devices). Following cases are studied:

-	The A-IoT paging message containing an identifier of a single A-IoT device

-	The A-IoT paging message containing a group ID that maps to multiple A-IoT devices

-	The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond

-	The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in [151]. From RAN2 perspective, it is feasible to support paging multiple identifiers of A-IoT devices, pending on TB size and multiplexing design of A-IoT paging message.

The following has been studied in TR 38.769, section 6.3.3 for A-IoT paging message content [3]:

The following agreements were made on AIoT paging message in RAN2#129 [2]:

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

The following agreements were made on CFRA/CBRA in RAN2#129-bis [1]:

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Unified Msg0 structure for all the paging types

The R2D transmission in way-1 as a part of paging message can be a unique or unified message structure for all query types mentioned above in TR38.769 for both CBRA and CFRA.

Fig. 3: Unified msg0 structure for all the paging/query types for 2/4-step CRFA/CBRA.

The PRDCH data part contains the R2D information bits from layer2 is shown in Fig. 3. The unified msg0 structure is proposed for the data part, where the reader conveys the query type to the device and accordingly allocate the time and frequency domain resources. The field parameters of msg0 are described below:

- Message type

- Transaction ID

- Reader ID (if required)

- Paging ID presence indicator

- Paging ID length

- Paging ID

- CBRA/CFRA

- Number of R2D triggers

- Time domain resources (X=1 or X=2)

- Frequency domain resources

Paging ID: This field contains device ID(s), in case the reader sends query to one or few specific devices. This field contains group ID if the reader sends query to a group of devices. This field is either empty or contains ‘0’ when device sends query to all the devices within the coverage.

Time domain resources: The msg0 may carry 1-bit to convey the number of time domain resources to the device. Bit ‘1’ indicates X=1 within two R2D triggers. Bit ‘0’ indicates X= 2 within two R2D triggers.

Observation 4: The R2D transmission as a part of paging message can be a unique or unified message structure for all query/paging types for both CBRA and CFRA.

Proposal 9: The field parameters of msg0 includes:

- Message type

- Transaction ID

- Reader ID (if required)

- Paging ID presence indicator

- Paging ID length

- Paging ID

- CBRA/CFRA

- Number of R2D triggers

- Time domain resources (X=1 or X=2)

- Frequency domain resources

Proposal 10: Msg0 signal may carry 1-bit to convey the number of time domain resources for an R2D trigger to the device. Bit ‘1’ indicates X=1 within two R2D triggers. Bit ‘0’ indicates X= 2 within two R2D triggers.

# Conclusion

A-IoT device behaviour in multi-reader scenario and A-IoT paging message content are discussed in this work in the context of one or multiple service request(s) received by the device from one or multiple base stations. We have made the following observations and proposals related to the above-mentioned topics:

Observation 1: Based on the deployment position, the A-IoT devices may get one or multiple service requests from one or multiple base stations.

Proposal 1: If the A-IoT device receives multiple service request in parallel, it will respond to the msg0 it receives first and start communicating with that reader. The parallelly received msg0 signals are discarded by the device until the ongoing process is completed successfully. Thus, Parallel service requests should have separate transaction ID to identify at the device side whether this is a different service request or same request from a different reader or same request from a same reader.

Observation 2: The inventory process may involve multiple paging occasions. Once a device has completed the initial service request and goes to idle or listening mode, it can decide whether to respond to the next paging request based on transaction ID and/or Reader ID.

Proposal 2: The reader should send an ACK signal if the Msg3 is successfully received at the reader. If the device doesn’t receive ACK and it receives next R2D trigger, it considers the transaction is failed.

Proposal 3: If the ongoing transaction is failed, it retransmits Msg1 after it receives the subsequent paging Msg0 for the same transaction. Until the ongoing transaction is successful, or reader specifically send a release message, the device will discard other Msg0 signals with different transaction ID.

Proposal 4: If a device successfully sends the information at one slot it will not respond to the next slots for the same transaction or service request from the same reader to free up the resources for the other devices.

Proposal 5: If a device successfully sends the information at one slot and goes to idle or listening mode, it may respond to the next slots for the same transaction or service request from a different reader.

Proposal 6: The R2D trigger message should contain the transaction ID so that the device can identify the trigger messages for that transaction.

Observation 3: As the service ID or the transaction ID plays an important role in order to decide the device behaviour when the device is in idle or listening mode, the device needs to differentiate among the different cases of multi-reader or multi-paging scenarios.

Proposal 7: For the device to differentiate among the different cases of multi-reader or multi-paging scenarios, the transaction ID should be different for different reader for same service as well as it should be different for same reader for different services. The following options can be adopted to solve this problem:

    - Option 1: Concatenate reader ID and correlation ID received from the CN as transaction ID.

    - Option 2: Generate a new transaction ID from both the correlation ID received from the CN and the reader ID. FFS on operation to generate the Transaction ID from correlation ID and Reader ID.

Proposal 8: If reader sends multiple msg2 signals, the msg2 signal can carry an indication to convey the device if it is the last msg2 signal in this communication slot.

Observation 4: The R2D transmission as a part of paging message can be a unique or unified message structure for all query/paging types for both CBRA and CFRA.

Proposal 9: The field parameters of msg0 includes:

- Message type

- Transaction ID

- Reader ID (if required)

- Paging ID presence indicator

- Paging ID length

- Paging ID

- CBRA/CFRA

- Number of R2D triggers

- Time domain resources (X=1 or X=2)

- Frequency domain resources

Proposal 10: Msg0 signal may carry 1-bit to convey the number of time domain resources for an R2D trigger to the device. Bit ‘1’ indicates X=1 within two R2D triggers. Bit ‘0’ indicates X= 2 within two R2D triggers.

# References

[1] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #129-bis.

[2] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #129.

[3] TR 38.769, Study on solutions for ambient IoT (Internet of Things) in RAN, V2.0.0.