3GPP TSG-RAN WG2 Meeting #130                                      R2-2503875

St Julian's, Malta, 19 - 23 May 2025

Source:        ZTE Corporation

Title:           Discussion on L3 measurements in SBFD

Agenda item:   8.11.3

Document for:  Discussion and Decision

- Introduction

This contribution provides our view on some issues in L3 measurement when SBFD is introduced.

- Discussion

L3 measurement filtering on SBFD symbol

UE measures SSB and CSI-RS for L1 and L3 measurement. When SBFD is enabled for a carrier, the SSB and CSI-RS can still be received and measured on SBFD symbols, as RAN1’s agreement indicates:

RAN1#118-bis Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

RAN1#117 Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Observation 1: The SSB and CSI-RS can be allocated and measured by the UE on the SBFD symbols, for L1 measurement and L3 measurement.

According to 38.300 [1], the UE measures multiple beams (at least one) of a cell and the measurements results (power values) are averaged to derive the cell quality. In doing so, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams.

Usually the interference of DL reception on SBFD symbols is larger than the DL reception on non-SBFD symbols. So even if gNB transmits same RS power on the two symbol types, the received RSRP/RSRQ/SINR on SBFD symbols will be different than the received RSRP/RSRQ/SINR on non-SBFD symbols.

Observation 2: Due to the interference, even if the RS transmission power is the same, the received RS-RSRP/RSRQ/SINR on SBFD symbols will be different than the received RS-RSRP/RSRQ/SINR on non-SBFD symbols.

For CSI-RS measurement, RAN1 has made the agreement that UE only uses one symbol type (SBFD symbol or non-SBFD symbol) to generate L1 CSI report:

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

For CSI-RS, separate measurement and report on different symbol type is ok, because CSI-RS is configured very flexible and may be truncated. However SSB configuration/transmission is more fixed. The SSB indexes/beams on the SBFD symbol can be fixed considering the SSB pattern is fixed and SBFD pattern is also fixed for the cell. This means some specific SSB beam directions may always be on the SBFD symbol, so the SSB beams on SBFD symbol will have different level of RSRP/RSRQ/SINR compared to other SSB beams. When generating L3 measurements, such SSB beams on the SBFD symbols should also participate in L3 filtering with equal possibility compared to the SSB beams on the non-SBFD symbols, in order to evaluate the cell quality fairly/accurately. Otherwise, those SSB beams on the SBFD symbols will always be absent in deriving the cell measurement.

Observation 3: Since the SBFD configuration and SSB beam/index/periodicity are cell-specific and pretty fixed, it is very likely that some specific SSB beam directions are always on the SBFD symbols.

Observation 4: If UE only takes the several specific SSB beam directions on SBFD symbol and non-SBFD symbol for L3 measurement, the UE cannot well evaluate the cell quality equally/accurately since a cell may have multiple beams with different directions.

Observation 5: When generating L3 measurements, the SSB beams on the SBFD symbols should also be taken into L3 filtering with equal possibility compared to the SSB beams on the non-SBFD symbols, in order to evaluate the cell quality fairly/accurately.

Proposal 1: Support UE to generate L3 measurement using the RS (CSI-RS/SSB) beam measurements made on SBFD symbols.

Since legacy only has one symbol type, legacy beam measurement selection is based on a single configured power threshold. Only the beam measurement that the received power is beyond the power threshold will be selected to derive the cell measurement. However if the beam measurement on SBFD symbols also adopt the same power threshold, it is likely that beam measurement on SBFD symbols will easily fail to derive the cell measurement since the receive power on SBFD symbols will be lower.

Based on the above observations, RAN2 should consider how to apply proper filtering for UE to generate a cell measurement using the beam measurements on SBFD symbols. The procedure from L1 measurement to L3 measurement is as following:

TS38.300: Figure 9.2.4-1: Measurement Model

There can be at least the following methods:

- Method 1: From A to A1, UE itself adds a power offset to those RS beam measurement (RSRP/RSRQ/SINR) on SBFD symbols, to compensate the interference occurred on SBFD symbols. This method may require RAN4’s further feasibility analysis.

- Method 2: From A1 to B, gNB can configure different power threshold on SBFD symbols and non-SBFD symbols, i.e., gNB configures additional power threshold to UE, which is dedicated for the beam measurement made on SBFD symbols. Only when the RS beam measurement on the SBFD symbols is beyond the additional power threshold, the RS beam measurement is allowed to consolidate a cell measurement by the UE. The method 2 is more flexible since gNB knows the RS transmit power on SBFD symbols, and gNB can determine the value of the additional power threshold based on current interference level, which makes the cell measurement more accurate and credible.

Based on above, we propose that RAN2 supports method 2.

Proposal 2: gNB configures additional power threshold to UE, the additional power threshold is used for the consolidation of cell measurement from the beam measurements made on SBFD symbols.

Gap requirement for RRM measurement on SBFD symbol

UE measures SSB and CSI-RS for L1 and L3 measurement. When SBFD is enabled for a carrier, the SSB and CSI-RS can still be received and measured on SBFD symbols, as RAN1’s agreement indicates:

RAN1#118-bis Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

RAN1#117 Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

And the configured/active BWP may likely be divided into UL subband and DL subband, as RAN1’s agreement indicates:

RAN1#116 Agreement For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL usable PRBs are allowed - FFS SSB symbols - DL receptions within DL usable PRBs are allowed - UL transmissions outside UL usable PRBs are not allowed - DL receptions outside DL usable PRBs are not allowed - This restriction is not applicable for CLI measurement RAN1#117 Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). RAN1#118-bis Agreement If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs. - Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols.

Observation 6: The configured or active BWP of a cell may be divided into UL subband and DL subband when SBFD configuration is enabled in the cell.

After introducing SBFD configuration per cell, it is likely that the frequency location of the RS of configured MOs (measurement objects) for L3 measurement (inter-frequency measurement or intra-frequency measurement) and the frequency location of the DL subband have different overlapping situations, see Figure 1:

Figure 1. Frequency location of the MO and the DL subband in a cell

Observation 7: The frequency resource of the configured MOs (measurement objects) may be allocated in the UE’s UL subband on SBFD symbols.

For legacy intra-frequency measurement, if the intra-frequency MO is within active BWP, UE does not need measurement gap to measure such MO; for inter-frequency measurement, if the inter-frequency MO is within active BWP, the UE does not need measurement gap to measure such MO (if UE supports the corresponding capability).

After SBFD is enabled, the RS of a MO on the SBFD symbol may be within the configured (or active) DL BWP and within the DL subband, or within the configured (or active) DL BWP but outside the DL subband. In our understanding, since the SBFD-aware UE can only work on DL subband on SBFD symbols, if the MO is within the active BWP but not within DL subband on SBFD symbols, the legacy gap requirement rule will change, i.e., the UE will need a measurement gap to measure such MO even if such MO is within the active DL BWP.

Although the gap requirement for an intra/inter-frequency measurement is specified in RAN4 specification, it does have specification impact in RAN2, e.g. need for gap design specified in 38.331, and the gap provision rule specified in 38.300. Considering the limited time for the WI, we suggest to ask RAN4 to confirm the understanding so that RAN2 can proceed.

Observation 8: Whether UE needs a gap to measure a MO that is not within the DL subband on SBFD symbol has RAN2 specification impact, e.g. need for gap design in 38.331, and the gap provision rule in 38.300.

Proposal 3: If a MO is on SBFD symbol but it is not within the DL subband (even if the MO is within current active BWP), UE needs measurement gap to measure such MO. Send LS to RAN4 to confirm this understanding.

Serving cell measurement on SBFD symbol

Additionally for serving cell measurement, in Rel-18, if an active BWP has a servingCellMO containing a CD SSB or NCD SSB, UE will take the servingCellMO of the active BWP as the serving cell measurement, so that UE does not need gap to make such serving cell measurement since the RS is completely within UE’s current active BWP. Furthermore, UE use the serving cell measurement to determine whether the configured neighbor cell’s measurement is intra-frequency or inter-frequency measurement.

Each cell can have a CD SSB and one or more NCD SSBs, wherein CD SSB is associated with the initial DL BWP, and one or more NCD SSBs are associated with each configured DL BWPs. If the cell is divided into DL subband and UL subband, the CD SSB is likely to be within DL subband. As for NCD SSB, the purpose of introduce NCD SSB is to reduce overlapping between RedCap UEs and normal UEs, and further in Rel-18 the NCD SSB can also be used by normal UE to make measurements, so the CD SSB and NCD SSB are distributed dispersedly in the frequency domain. Therefore, if all the NCD SSBs of a cell are restricted within DL subband, it is hard for gNB to allocate a proper DL/UL subband frequency division, considering there is only one UL subband per cell, and the location of the UL subband is semi-static.

Observation 9: If all the NCD SSBs of a cell are restricted to be within DL subband, it is hard for gNB to allocate a proper DL/UL subband frequency division.

RAN1 has made the following agreements in RAN1#118-bis:

RAN1#118-bis Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

RAN1 does not clearly clarify whether CD SSB and/or NCD SSB should be within DL subband. Since this issue has obvious spec impact to RAN2/RAN4 on UE’s measurement behavior, we propose to send a LS to RAN1 for clarification.

Proposal 4: RAN2 sends LS to RAN1 (cc RAN4) to ask whether the CD SSB and/or NCD SSB(s) of a cell should be allocated within DL subband.

If the current active BWP is divided into UL subband and DL subband due to SBFD configuration, and NCD SSB can be allocated inside and outside DL subband, for serving cell measurement, the following cases may happen:

- Case 1: DL RS (including NCD SSB) in servingCellMO is within active BWP and within DL subband (i.e., in the intersection);

  - In this case, it will have no problem when UE does as legacy, i.e., UE can take the servingCellMO of current active BWP as serving cell measurement, since the RS of servingCellMO of current active BWP is within the intersection of active BWP and DL subband, no measurement gap is needed.

- Case 2: DL RS (including NCD SSB) in servingCellMO is within active BWP but not in DL subband.

  - In this case, based on the above analysis, UE may need measurement gap to receive such DL RS. If the DL RS is contained in the servingCellMO of the active BWP, a measurement gap for serving cell measurement will be needed. This will increase the interruption of the system considering the serving cell measurement is frequent and mandatory, and is not aligned with the motivation/intention to enhance servingCellMO for each BWP in Rel-18.

Figure 2. Case 1 and case 2

Therefore, gNB should ensure that the frequency resource of RS in the servingCellMO which is associated with each configured BWP should be configured within the DL subband of the cell (i.e., only case 1 is allowed), in order to reduce the UE’s measurement gap requirement for serving cell measurements.

Proposal 5: gNB should ensure that the servingCellMO associated with each configured BWP should be configured within the DL subband of the cell, in order to reduce the measurement gap requirement for serving cell measurements.

- Conclusion

In this contribution, we propose the following observation and proposals:

L3 measurement filtering on SBFD symbol:

Observation 1: The SSB and CSI-RS can be allocated and measured by the UE on the SBFD symbols, for L1 measurement and L3 measurement.

Observation 2: Due to the interference, even if the RS transmission power is the same, the received RS-RSRP/RSRQ/SINR on SBFD symbols will be different than the received RS-RSRP/RSRQ/SINR on non-SBFD symbols.

Observation 3: Since the SBFD configuration and SSB beam/index/periodicity are cell-specific and pretty fixed, it is very likely that some specific SSB beam directions are always on the SBFD symbols.

Observation 4: If UE only takes the several specific SSB beam directions on SBFD symbol and non-SBFD symbol for L3 measurement, the UE cannot well evaluate the cell quality equally/accurately since a cell may have multiple beams with different directions.

Observation 5: When generating L3 measurements, the SSB beams on the SBFD symbols should also be taken into L3 filtering with equal possibility compared to the SSB beams on the non-SBFD symbols, in order to evaluate the cell quality fairly/accurately.

Proposal 1: Support UE to generate L3 measurement using the RS (CSI-RS/SSB) beam measurements made on SBFD symbols.

Proposal 2: gNB configures additional power threshold to UE, the additional power threshold is used for the consolidation of cell measurement from the beam measurements made on SBFD symbols.

Gap requirement for RRM measurement on SBFD symbol:

Observation 6: The configured or active BWP of a cell may be divided into UL subband and DL subband when SBFD configuration is enabled in the cell.

Observation 7: The frequency resource of the configured MOs (measurement objects) may be allocated in the UE’s UL subband on SBFD symbols.

Observation 8: Whether UE needs a gap to measure a MO that is not within the DL subband on SBFD symbol has RAN2 specification impact, e.g. need for gap design in 38.331, and the gap provision rule in 38.300.

Proposal 3: If a MO is on SBFD symbol but it is not within the DL subband (even if the MO is within current active BWP), UE needs measurement gap to measure such MO. Send LS to RAN4 to confirm this understanding.

Serving cell measurement on SBFD symbol:

Observation 9: If all the NCD SSBs of a cell are restricted to be within DL subband, it is hard for gNB to allocate a proper DL/UL subband frequency division.

Proposal 4: RAN2 sends LS to RAN1 (cc RAN4) to ask whether the CD SSB and/or NCD SSB(s) of a cell should be allocated within DL subband.

Proposal 5: gNB should ensure that the servingCellMO associated with each configured BWP should be configured within the DL subband of the cell, in order to reduce the measurement gap requirement for serving cell measurements.

- Reference

- TS38.300-i50, NR; NR and NG-RAN Overall Description; Stage-2; Release-18, 2025-03.