3GPP TSG-RAN WG2 Meeting #129	R2-2503898

St.Julians, Malta,  May 19th – 23rd , 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on inter-CU LTM

Document for:        	Discussion and decision

Agenda Item:         	8.6.2	Inter-CU LTM

- Introduction

In RAN2-129bis meeting [1] and RAN2-127bis [2], RAN2 discussed the detail of inter-CU LTM procedure, and the following was agreed.

Agreements on RRC issues:

⇒At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Agreements on inter-CU LTM

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

Agreements on inter-CU LTM in RAN2-127bis

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

In this contribution, we provide our understanding for inter-CU LTM procedure, such as the procedure related open issue RRC-6 and RRC-8.

- Discussion

For fast failure recovery, RAN2 agreed the UE can perform the fast failure recovery upon the following cases:

- If the selected candidate cell has the same Rel-19 ID as the source cell (no security key change) after an RLF.

- If the selected candidate cell has the same Rel-19 ID as the source cell after an intra-CU LTM failure.

- If the selected candidate cell has the same Rel-19 ID as the target cell after an inter-CU LTM failure.

If fast failure recovery is configured, the UE reverts back to the UE configuration used in the source cell after a reconfiguration with sync failure.

5.3.5.8.3 T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure) The UE shall: … 2> else: 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6: 4> revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG; …

In the last meeting, RAN2 discussed whether the UE can revert back to the security key associated with the source cell and RAN2 continues to discuss this issue in this meeting. In our understanding, if the attemptLTM-Switch is configured and the selected cell is not the LTM candidate cell in the LTM-Candidate IE within ltm-Config associated with the MCG, the UE sends the RRCReestablishmentRequst message to the network. In this case, the UE sets the shortMAC-I with the KRRCint key and integrity protection algorithm that was used in the source cell in the RRCReestablishmentRequst message. Therefore, the UE needs to revert back to the security key associated with the source cell.

Additionally, in the current CR, the UE checks the Rel-19 ID for the target cell (ltm-NoSecurityChangeID) and the Rel-19 ID for the source cell (ltm-ServingCellNoSecurityChangeID) to determine whether the security key update procedure is necessary. Upon the inter-CU LTM execution, the UE replaces the ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID to the ltm-NoSecurityChangeID associated with target cell. Therefore, for fast failure recovery, the UE also needs to revert back to the VarLTM-ServingCellNoSecurityChangeID associated with the source cell.

Finally, in the security key update procedure, the UE derive the security key by using the received NCC from CSC MAC CE and the NCC associated with the current security key (for the source cell). Therefore, the UE also needs to revert back to the NCC value associated with the source cell if the UE reverts back to the security key associated with the source cell. From the above perspectives, the UE needs to revert back to the security configuration (i.e. security key, the value of NCC and the VarLTM-ServingCellNoSecurityChangeID) used in the source cell after reconfiguration with sync failure.

Proposal 1: The UE reverts back to the security configuration (i.e. security key, the value of NCC and the VarLTM-ServingCellNoSecurityChangeID) used in the source cell after reconfiguration with sync failure.

In the last meeting, RAN2 agreed that if the selected candidate cell has the same Rel-19 ID as target cell, the UE performs fast failure recovery and the case of the fast failure recovery with different Rel-19 IDs is left as FFS. If the selected cell is the intra-CU LTM candidate cell, the UE can perform cell switch to the selected cell without the security key update since the UE reverts back to the security key used in the source cell.

If the selected cell is the inter-CU LTM candidate cell, the UE needs to security key update. In the security key update for the legacy CHO, the new source cell receives the only one pair of {NCC, NH} from the AMF after the handover procedure, and computes the new security key for all candidate cells with the received NCC value. Therefore, in our understanding, the source gNB can also derive the security key for all candidate cells with the only one NCC value in the inter-CU LTM. Based on this principle, the UE can also derive the security key for all candidate cells with the NCC value received on CSC MAC CE. Therefore, if the new source cell forwards the new security key to all candidate cells, the UE can select all candidate cells during fast failure recovery.

Proposal 2: The UE can select all candidate cells during fast failure recovery after inter-CU LTM failure.

- Summary and proposal

This paper discussed the procedure for mixture of inter-CU LTM and intra-CU LTM. In summary, the followings were proposed:

Proposal 1: The UE reverts back to the security configuration (i.e. security key, the value of NCC and the VarLTM-ServingCellNoSecurityChangeID) used in the source cell after reconfiguration with sync failure.

Proposal 2: The UE can select all candidate cells during fast failure recovery after inter-CU LTM failure.

- References

[1] R2-2503302, “RAN2#129bis Meeting Report,” ETSI MCC.

[2] R2-2409502, “RAN2#127bis Meeting Report,” ETSI MCC.