3GPP TSG-RAN WG2 Meeting #130	R2-2503913

St. Julian, Malta, 19th – 23rd May 2025

Agenda item:		8.7.5

Source:		Lenovo

Title:		AM RLC enhancement

Document for:		Discussion and Decision

- Introduction

In this contribution we discuss RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

- Discussion

2.1 Avoiding Unnecessary Retransmissions

At the RAN2#129bis meeting, the following agreements regarding unnecessary retransmission avoidance were achieved:

Unnecessary retransmissions avoidance - Agreements on RLC enahncements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

During the CR Review discussion, there was an open issue listed regarding the case that there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, i.e., there is no SDU to retransmit the poll with. According to our view, if stopReTxDiscardedSDU is configured, and only SDUs whose (re)transmissions have been stopped remain in the buffer, there is no need for the RLC transmitter to include a poll (e.g., due to the expiry of t-PollRetransmit, or trigger of enhanced polling, or due to window stalling). This is because regardless of a status report being received in response to the poll, no SDU can be (re)transmitted.

Proposal 1. (RLC-11) A poll is not included in a RLC SDU due to the expiry t-PollRetransmit, trigger of an enhanced poll or window stalling when stopReTxDiscardedSDU is configured and only SDUs whose (re)transmissions have been stopped, remain in the buffer.

During the previous RAN2 meetings, it was also agreed that the value of the new timer will not be lower than that of the t-Reassembly timer. This means that there could exist a corner case where the new timer is set to the same value of the t-Reassembly timer. It has also been agreed that the abandoned SDUs as determined by the new timer will be positively acknowledged in a status report, whereas the expiry of t-Reassembly will also trigger a status report containing negative acknowledgments for incomplete and/ or missing SDUs. Since both the new timer as well as the t-Reassembly have the same start condition, this could lead to discrepancy in the information provided to the RLC Tx entity depending on which status report is received at the transmitter side. Hence, we think RAN2 should discuss a way forward to prevent such a discrepancy, and we think that a simple solution would be to capture a note in the spec indicating that such a discrepancy should be prevented, e.g., by giving precedence to the status report constructed based on the expiry of the new timer over the expiry of t-Reassembly for the same RLC SDUs.

Proposal 2. RAN2 to discuss capturing an additional note in the spec to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value.

In TS 28.552, Layer 2 performance measurement for DL Packet Loss rate is based on the number of DL packets, for which at least a part has been transmitted over the air but not positively acknowledged. However, RAN2 assumes that the legacy RLC status report can be reused to notify the transmission RLC entity of transmission window update due to abandoned RLC SDUs by setting fake ACKs. The transmission RLC entity cannot differentiate between abandoned SDUs and the data which have actually been successfully received. And as a result, this impacts the accuracy of the layer 2 measurement for DL transmission delay and DL Packet Loss rate from point view of QoS monitoring. Therefore, RAN2 needs to discuss the impact of Layer 2 performance due to the avoidance of unnecessary RLC retransmission.

Observation 1. When the receiver RLC entity abandons the outdated RLC SDU based on new timer, it sends the fake ACK for the outdated RLC SDU to the transmitter RLC entity, which impacts at least DL packet error rate measurement.

Proposal 3. RAN2 to discuss the issue on DL packet error rate measurement due to the fake ACK of the abandoned RLC SDUs in the RLC status report.

2.2 Timely Retransmissions

At the RAN2#129bis meeting, the following agreements were made regarding timely retransmissions:

Autonomous retransmissions and polling enhancements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

RAN2 has agreed that retransmission and/or polling can be triggered when the remaining time of an RLC SDU falls below a specified threshold based on the discardTimer in PDCP. RAN2 needs to further discuss if remaining time can be determined based on discardTimerforLowImportance, if configured. In our view, if PSI-based discard is activated, the link is likely to be congested. In order to avoid further aggravating the congestion, autonomous retransmission and/ or enhanced polling can be limited to high-importance packets, i.e., discardTimerforLowImportance is not considered for autonomous retransmission and/or enhanced polling.

Proposal 4. (RLC-9) If PSI based-discard is activated, the discardTimerforLowImportance is not considered for autonomous retransmission and/or enhanced polling.

In legacy RLC status report procedure, the receving RLC entity shall not trigger RLC status report if t-StatusProhibit is running. RAN2 needs to discuss how to avoid the delay of transmission of status report in response to an enhanced poll due to the running of t-StatusProhibit timer, e.g., whether to introduce a new RLC control PDU for inlcuding the polling request. For example, when the peer RLC entity recieves the new RLC control PDU, it can ignore the t-StatusProhibit, if running and trigger RLC status report directly. Alternatively, the transmitting RLC entity may include a new indication for including the enhanced polling request in AMD PDU.

Proposal 5. RAN2 to discuss whether to introduce a new RLC control PDU or enhance AMD PDU for including polling based on remaining time.

Another open issue that needs to be discussed is how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple. In our view, this is not so critical since the RLC receiver already maintains t-StatusProhibit so excessive polling would not lead to excessive status reporting. But in order to maintain consistency across the enhancements for timely retransmissions, we think only one enhanced poll is sufficient per RLC SDU similar to only a single autonomous retransmission. Additionally, we think this would make the enhancement simpler to configure, i.e., only one remaining time threshold for enhanced polling needs to be configured.

Proposal 6. (RLC-8) RAN2 to agree that only one enhanced poll is triggered per RLC SDU.

Furthermore, it makes sense to configure the enhanced poll to be triggered before the autonomous transmission if both features are configured. In order to limit the resource utilization for autonomous retransmission, as well as to provide the RLC transmitter an opportunity to receive a timely status report, we think that if both autonomous retransmission as well enhanced polling are configured simultaneously, the enhanced poll should be triggered first, i.e., enhancedPollingThreshold is configured with a greater value than autonomousReTxThreshold.

Proposal 7. RAN2 to agree that enhancedPollingThreshold is configured with a greater value than autonomousReTxThreshold, if both autonomous retransmission and enhanced polling are configured simultaneously.

Additionally, RAN2 should also consider mechanisms to cancel either enhanced polling or autonomous retransmission or both for a packet to avoid overloading of the link. For example, if a RLC entity is configured with both features and a legacy status report is received for a packet with a negative acknowledgement before the enhanced poll or autonomous retransmission were triggered, the enhanced poll and autonomous retransmission of that packet can be cancelled since a legacy retransmission would already be triggered in response to the NACK. Or, the cancellation of an enhanced poll or autonomous retransmission can be considered when such was already triggered and an ACK (e.g., HARQ ACK or a RLC status-report containing an ACK for the corresponding PDU) is received before the enhanced poll or autonomous retransmission is submitted to lower layers.

Proposal 8. (RLC-9) RAN2 to agree to cancel the triggered enhanced polling and autonomous retransmission based on the reception of status report.

RAN2 needs to discuss whether autonomous retransmission could be counted to the total number of the retransmission. There is no motivation to relax the RLF detection criteria for the autonomous retransmission since RAN2 has already agreed that only one autnomous retransmission is done per RLC SDU. In our view, autonomous retransmission should be counted to total number of the retransmission as legacy retransmision. In addition, RAN2 needs to discuss whether autonomous retransmission can be triggered if the maximum number of RLC retransmission based on status report has reached. We think that autonomous retransmission should not be triggered in such case if RLF has not yet been determined (i.e., a NACK was not received after the last retransmission). We consider this since the autonomous retransmission thershold value should be smaller than the packet delay budget and the maximum number of RLC retransmissions is set by taking packet delay budget into account. Theoretically, autonomous retransmission is most probably triggered before the maximum number of RLC retransmission is reached.

Proposal 9. (RLC-7) RAN2 to agree that autonomous retransmission is counted into RETX_COUNT.

Proposal 10. (RLC-7) RAN2 to agree that autonomous retransmission should not be triggered if the maximum number of RLC retransmission has been reached and RLF is yet to be indicated.

- Conclusion

In this contribution, we discuss the RLC AM enhancement for XR traffic with small delay budget. We have the following observation and proposals:

Observation 1. When the receiver RLC entity abandons the outdated RLC SDU based on new timer, it sends the fake ACK for the outdated RLC SDU to the transmitter RLC entity, which impacts at least DL packet error rate measurement.

Proposal 1. (RLC-11) A poll is not included in a RLC SDU due to the expiry t-PollRetransmit, trigger of an enhanced poll or window stalling when stopReTxDiscardedSDU is configured and only SDUs whose (re)transmissions have been stopped, remain in the buffer.

Proposal 2. RAN2 to discuss capturing an additional note in the spec to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value.

Proposal 3. RAN2 to discuss the issue on DL packet error rate measurement due to the fake ACK of the abandoned RLC SDUs in the RLC status report.

Proposal 4. (RLC-9) If PSI based-discard is activated, the discardTimerforLowImportance is not considered for autonomous retransmission and/or enhanced polling.

Proposal 5. RAN2 to discuss whether to introduce a new RLC control PDU or enhance AMD PDU for including polling based on remaining time.

Proposal 6. (RLC-8) RAN2 to agree that only one enhanced poll is triggered per RLC SDU.

Proposal 7. RAN2 to agree that enhancedPollingThreshold is configured with a greater value than autonomousReTxThreshold, if both autonomous retransmission and enhanced polling are configured simultaneously.

Proposal 8. (RLC-9) RAN2 to agree to cancel the triggered enhanced polling and autonomous retransmission based on the reception of status report.

Proposal 9. (RLC-7) RAN2 to agree that autonomous retransmission is counted into RETX_COUNT.

Proposal 10. (RLC-7) RAN2 to agree that autonomous retransmission should not be triggered if the maximum number of RLC retransmission has been reached and RLF is yet to be indicated.

- References

TS 38.322 v 18.0.0

R2-2410287, AM RLC enhancement, Lenovo