3GPP TSG-RAN WG2 Meeting #130						        R2-2503970

St.Julians, Malta, May 19 – 23, 2025

Source: 	ZTE Corporation, Sanechips

Title: 	Open issues for data transmission and MAC PDU design

Agenda item:	8.2.4

Document for: 	Discussion and Decision

# Introduction

In this contribution we discuss the details of Open issues for data transmission and MAC PDU design for A-IoT data transmission.

# General design for A-IoT MAC layer

# Introduction

During the study phase, RAN2 has made assumption that, on A-IoT interface between the device and the reader, there is A-IoT MAC layer only and no other L2 AS layer. Furthermore, there is no CP/UP protocol stack differentiation. In the following sections, we will discuss the main procedures supported by A-IoT MAC layer, and the associated MAC Protocol Data Units, formats and parameters.

# MAC procedures

# Device state

It was agreed during the study item that from RAN2 perspective, the device is always monitoring PRDCH (as long as there is enough energy). However, the device should be receiving different types of R2D messages depending on various conditions. The following aspects need to be considered:

- Before paging: Before receiving a paging indication, the device should monitor PRDCH for paging messages. This is the only type of message that the device should be monitoring and responding to in this state. Once the device receives paging and determines that the paging is for the device, it should store the transaction ID included in the paging message.

- After paging: Once paging is received and a transaction ID is stored on the device side, the device shall monitor the PRDCH for both broadcast messages (e.g. MSG1 trigger messages, MSG2 etc) and unicast messages (addressed to the devices ASID) that carry the stored transaction ID. Thus, it is necessary that all R2D messages carry the transaction ID.

- Completion of the AS procedure: In Rel-19 only inventory and command procedures are supported. So, once the device is paged, the AS procedure for inventory or command is initiated and the transaction ID and any ASID associated with the transaction ID are stored in the device until the AS procedure is successfully completed or terminated due to failure. Upon both successful completion and failure, the device shall delete the stored transaction ID and the ASID and start monitoring paging again (i.e. go back to step 1 above).

So, based the above, the following proposal is made:

Proposal 1: When the device has sufficient energy, the device shall:

- If there is no stored transaction ID:

  - Monitor PRDCH for paging messages and store the received transaction ID once a paging message addressed to the device is received

- If there is a stored transaction ID:

  - Monitor PRDCH for broadcast (e.g. MSG2 and MSG1 trigger messages) and unicast messages (addressed to the device’s ASID) that include the stored transaction ID until the AS procedure (Inventory/Command) is completed (either successfully or unsuccessfully)

In order to ensure that the device responds to only the reader with the correct transaction ID, every message in R2D direction should include the transaction ID. Paging message anyway will include the transaction ID explicitly as agreed by RAN2. For the other messages, in order to minimise the overhead, it is best if the transaction ID is included implicitly by masking the CRC with the transaction ID. So, in R2D direction the reader shall XoR the CRC with the transaction ID. The device should unmask the CRC using the transaction ID before processing the received messages. Since RAN1 agreed that the CRC length is at least 6 bits and is included for all messages, carrying a 6 bit transaction ID hence comes for free (i.e. without any additional overhead).

Proposal 2: The transaction ID length should be 6 bits

Proposal 3: All messages include a transaction ID either explicitly (e.g. paging message) or implicitly (by masking the CRC with the transaction ID)

# MAC PDU formats and parameters

At RAN2#129 the following message types were agreed:

    - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF))

    - D2R MAC PDU (MSG1)

    - R2D MAC PDU (MSG2)

    - D2R MAC PDU (MSG3 and data)

    - R2D MAC PDU (R2D data)

    - Other message types are FFS.  The message types may evolve based on functionality agreements.

In addition to the above, the following messages are also needed:

    - R2D Scheduling Information (message to schedule the D2R transmission resources)

    - R2D NACK

The general message structure should be as shown below:

Figure 1: General Message structure in R2D and D2R

One of the issues to discuss is whether the message type field is needed for every message. Especially, for MSG1, this seems redundant. However, in theory, the same argument can be made to all D2R messages since these are scheduled one after the other. However, for simplicity and to ensure extendibility (e.g. multi-reader scenarios and for future use cases where different format of MSG1 may be considered etc), it is better to have a consistent format for D2R messages and hence it is proposed that even for CBRA MSG1, message type field is included.

Proposal 4: Message type field is included for all messages (including the Random ID message)

# Example MAC PDU formats for some messages

Figure 2 below shows the detailed structure of a few example messages in R2D and D2R:

Figure 2: Example MAC PDU structure for messages

Proposal 5: The above message structures proposed in the figure should be taken as baseline

# Segmentation and retransmissions

For the retransmission of the first segment/unsegmented D2R message, it was agreed at the last meeting that the reader repeats the R2D command message by including the upper layer command again. When the upper layer command is included, the device will pass the command message to the upper layers and the D2R packet containing the command response is generated. The question then is how to support subsequent retransmissions (i.e. whether to include the command again or not). In order to ensure proper separation between the layers, it should be assumed that the upper layers generate a new command response every time a command is received. Thus, including the command in subsequent retransmissions doesn’t seem appropriate. Thus, it is proposed that for subsequent retransmissions, the reader simply includes an offset. This means that the command response is fetched from upper layers only once and then retransmission of the segments within this response happens based on the same command response.

Proposal 6: For the retransmission of the first segment/unsegmented D2R message, only the R2D command message is included without the offset

Proposal 7: For the retransmission of subsequent segments (i.e. after the first segment), only the offset is included (i.e. R2D command is not included).

Then, finally we also need to agree the granularity of the offset indications. Since the offset indicates the total number of bits received so far, the granularity of the offset should be multiples of D2R message sizes. So, the granularity of the offset should be selected such that 1 or more D2R messages with max D2R message size can be signalled using the offset. Note that extremely fine granularity is not necessary for the offset (as this increases the R2D message size).

Proposal 8: The granularity of the offset should be designed such that it covers 1 or more D2R TB sizes (including the max D2R TB size)

# Conclusion and proposals

The following proposals are made in this contribution:

Proposal 1 (Issue 1-1): When the device has sufficient energy, the device shall:

- If there is no stored transaction ID:

  - Monitor PRDCH for paging messages and store the received transaction ID once a paging message addressed to the device is received

- If there is a stored transaction ID:

  - Monitor PRDCH for broadcast (e.g. MSG2 and MSG1 trigger messages) and unicast messages (addressed to the device’s ASID) that include the stored transaction ID until the AS procedure (Inventory/Command) is completed (either successfully or unsuccessfully)

Proposal 2 (Issue 1-2): The transaction ID length should be 6 bits

Proposal 3 (Issue 1-1): All R2D messages include a transaction ID either explicitly (e.g. paging message) or implicitly (by masking the CRC with the transaction ID)

Proposal 4 (Issue 3-5): Message type field is included for all messages (including the Random ID message)

Proposal 5 (Issue 2-6, 2-7): The message structures proposed in the figure below should be taken as baseline

Proposal 6 (Issue 3-1, 3-2): For the retransmission of the first segment/unsegmented D2R message, only the R2D command message is included without the offset

Proposal 7(Issue 3-1, 3-2): For the retransmission of subsequent segments (i.e. after the first segment), only the offset is included (i.e. R2D command is not included).

Proposal 8(Issue 3-1, 3-2): The granularity of the offset should be designed such that it covers 1 or more D2R TB sizes (including the max D2R TB size)

# References

[1] 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19), December, 2024

[2] 3GPP RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN Plenary#106, December, 2024