3GPP TSG-RAN WG2 #130	R2-2503988

St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		8.5.2	On-demand SSB SCell operation

Title:		Remaining issues on On-demand SSB for SCell

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#129bis, there were some important agreements below for On-Demand SSB for SCell [1] and the post-129bis email discussion for running CRs [2][3].

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In this contribution, we discuss some details of new MAC CE for OD-SSB activation/deactivation and provide our views.

# 2. Discussion

2.1	New MAC CE format

For the OD-SSB deactivation, there were some agreements related to the new MAC CE in the last RAN1#120bis [4]:

Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}. - N2= [8]. - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE]. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

In the last RAN2#129bis, it was agreed that “RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations” with leaving details as FFS. On this issue, there are some fundamental questions below to decide the exact format:

- Q1: whether the network configures common OD-SSB deactivation method for all the relevant SCells (i.e. only either explicit or implicit OD-SSB deactivation at a time) or may configure both in combination (i.e. explicit for some SCells, while implicit for other SCells)?

- Q2: whether the explicit OD-SSB deactivation can override the implicit OD-SSB deactivation, if the MAC CE deactivating OD-SSB is received, if both explicit and implicit can be configured in combination?

- Q3: whether the value “N” (the number of OD-SSB bursts to be transmitted after OD-SSB indication) should be the same for all the OD-SSB SCells or can be different among the OD-SSB SCells?

These may need to wait for RAN1 progress, while it would be good to discuss from RAN2 perspective as well.　In the following, we basically assume the Case #2 (i.e., Always-on SSB on the cell).

Q1: common OD-SSB deactivation method or can be different among SCells?

From network perspective, there is no strong reason to apply different OD-SSB deactivation method among SCells. We expect that having the common method makes the MAC CE design easier. We are open for further discussions but unless there is no critical issue in this limitation, it would be better to agree with the common deactivation method among SCells.

Observation 1. If RAN2 can agree with the restriction that the common OD-SSB deactivation method (either explicit or implicit) among SCells, the MAC CE design will be simpler.

Q2: explicit OD-SSB deactivation can override the implicit OD-SSB deactivation, if both methods co-exist?

This point depends on the outcome of the Q1 and is valid if different OD-SSB deactivation methods among SCells need to be supported. An expected scenario is that the implicit deactivation is configured at first (i.e. at the time of OD-SSB configuration). The OD-SSB is deactivated after configuration with initially activated or after activation via MAC CE. Later, the OD-SSB may be re-activated. For this re-activated OD-SSB, the network may want to change the deactivation method to the explicit one. In this case, it would be useful and flexible to make it possible to change the method by sending MAC CE with explicit deactivation indication directly, i.e. override the deactivation method. To avoid unnecessary complications, this may be applied only if the explicit deactivation is instructed via MAC CE. Otherwise, the UE should assume the original one, i.e. implicit deactivation.

Observation 2. If RAN2 supports different OD-SSB deactivation methods among SCells, it would be useful to support the change of OD-SSB deactivation method dynamically via MAC CE which explicitly deactivates the OD-SSB, when the originally implicit OD-SSB deactivation is configured.

Q3: the value “N” should be the same for all the SCells or can be different among SCells?

According to the RAN1 agreements, RRC can configure multiple candidate values of the “N” and then MAC can indicate which value is applied for explicit OD-SSB deactivation. In general, it is useful and flexible for the network to configure different values among SCells. However, it takes more bits in the MAC CE, especially in the MAC CE supporting up to 31 SCells. Considering the trade-off between the benefit and the signalling overhead, a single value “N” should be assumed from RAN2 perspective. As this is also related to RAN1 discussion, at this moment RAN2 assume the single value and then wait for RAN1 progress or send an LS to RAN1. Given sending an LS, it might be useful to inform RAN4 as well.

Observation 3. From RAN2 perspective, it is better to assume the single value “N” (from multiple candidate values configured by RRC) for all SCells and then confirm with RAN1.

Based on the discussions and observations above, we firstly propose to agree with the common OD-SSB deactivation method for all the SCells. Depending on the conclusion for that, we may have different proposals below.

Proposal 1: Network ensures only one of explicit and implicit OD-SSB deactivation for all the SCells on which OD-SSB is used.

Proposal 2: RAN2 to confirm that RRC configures multiple candidate values of “N” (as per RAN1 agreement).

Proposal 3: RAN2 to assume the MAC CE includes the single value “N” from the configured candidate values, which is applied for all the SCells for which OD-SSB is activated and send an LS to RAN1/4 for confirmation.

If the proposal 1 is not agreeable and thus both OD-SSB deactivation methods can be configured among SCells (i.e. either one per SCell), we propose to discuss the possibility of dynamically changing the OD-SSB deactivation method from implicit to explicit.

Proposal 4: RAN2 to discuss whether the network can change the OD-SSB deactivation method from implicit to explicit, if the P1 is not agreed.

2.2	Need of MAC CE triggered MR

There is one more remaining issue from the last meeting. The point is whether to support the measurement report triggered by new MAC CE for OD-SSB activation. This may need scenario dependent discussions.

For the scenario 2A, the measurement report upon SCell activation is already supported and thus there is no additional trigger is not needed.

#### 5.5.2.3 Measurement identity addition/modification .. The UE shall: 1> for each measId included in the received measIdToAddModList: 2> … 2> if the reportType is set to reportOnScellActivation in the reportConfig associated with this measId: 3> indicate to lower layer to enable the measurement reporting for fast unknown SCell activation.

For the scenario 2, there is the time gap between the OD-SSB activation and the SCell activation. A question is whether a certain measurement report is necessary during this time gap. We could not see any important usage for the corresponding measurement report. Unless there is specific need of that, we assume there is no need of measurement report triggered by the new MAC CE for OD-SSB activation.

Proposal 5: Do not consider additional measurement report triggered by new MAC CE for OD-SSB activation.

# 3. Conclusion

In this contribution we discussed remaining issues for new MAC CE for OD-SSB activation/deactivation and made the following proposals.

Proposal 1: Network ensures only one of explicit and implicit OD-SSB deactivation for all the SCells on which OD-SSB is used.

Proposal 2: RAN2 to confirm that RRC configures multiple candidate values of “N” (as per RAN1 agreement).

Proposal 3: RAN2 to assume the MAC CE includes the single value “N” from the configured candidate values, which is applied for all the SCells for which OD-SSB is activated and send an LS to RAN1/4 for confirmation.

Proposal 4: RAN2 to discuss whether the network can change the OD-SSB deactivation method from implicit to explicit, if the P1 is not agreed.

Proposal 5: Do not consider additional measurement report triggered by new MAC CE for OD-SSB activation.

# References

[1] R2-2502981, Report from session on R18 SL, R18/19 MOB and R19 NES, Vice Chairman (Samsung)

[2] [POST129bis][112][NES] MAC Running CR (IDC)

[3] [POST129bis][111][NES] (Ericsson)

[4] Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0

# Annex for reference

RAN2 agreements:

Agreements on OD-SSB @ #127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell @ #127bis

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Agreements on OD-SSB @#129

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.