3GPP TSG-RAN WG2 #130	R2-2504031

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	8.12.2

Source:	Ericsson

Title:	Asymmetric DL/UL mTRP user plane impact from MIMO ph. 5

Release:	19

Document for:	Discussion

# 1	Introduction

Release 19 MIMO with its justification and objective has been endorsed as one of the features for NR MIMO Phase 5 [1].

The subtopic objectives relevant to this WI are:

- UE initiated/event-triggered beam reporting,

- increased number of ports (up to 128) for CSI measurement and reporting,

- inter-TRP calibration for CJT,

- 3Tx for UL,

- asymmetric DL sTRP/UL mTRP.

This contribution discusses user plane impacts of Asymmetric DL sTRP/UL mTRP that have clear impacts to RAN2 specification, considering some RAN1 related agreements.

# 2	Discussion

2.1 Background Asymmetric DL sTRP UL mTRP

Figure 1 illustrates an asymmetric DL sTRP UL mTRP deployment. It consists of an Anchor TRP (providing both DL and UL service), and a TRP which will be called an UL-only TRP (for ease of reference) to provide UL service, where the downlink reference signals, the downlink control channels, and the downlink data transmissions for a UE are only provided from the Anchor TRP.

Figure 1: UL-only TRP in an asymmetric DL sTRP UL mTRP deployment

The main motivation for asymmetric DL sTRP UL mTRP deployment is to improve UL throughput by UE transmitting UL data to the UL-only TRP. For example, the Anchor TRP in Figure 1 provides coverage with both DL and UL transmissions, and the UL-only TRP is deployed to improve UL performance. Other motivations would be to reduce network energy consumption, by avoid sending downlink data from the UL-only TRP. It is envisioned that a UE in the cell may first perform initial access via the Anchor TRP, and then, if the UE is closer to the UL-only TRP than to the Anchor TRP (i.e. better UL link budget), or if the NW decides to move the UE to an UL-only TRP, e.g., to reduce the UL interference for the Anchor TRP, or to improve resource usage efficiency, the NW may direct the UE to transmit towards the UL-only TRP.

Enhancement on power control is needed to support the asymmetric deployment.

s

UE performs UL transmission to only the UL-only TRP

UE performs UL transmission to two different UL-only TRPs

UE performs UL transmission to one UL-only TRP and to the Anchor TRP

## 2.1	User Plane Impacts

In the Unified TCI state framework, a PL-RS can be associated with an UL TCI state through RRC configuration, and the UE should then use the PL-RS associated with an UL TCI state when transmitting UL signals/channels associated with that UL TCI state. Since the UE uses the estimated pathloss to the Anchor TRP to determine output power for an UL TCI state associated with the UL-only TRP, it is important that the pathloss estimation towards the Anchor TRP reflects the actual pathloss. When the UE moves around in a cell in FR2, the downlink beams and the associated DL TCI state from Anchor TRP must be updated frequently. In a similar way, the pathloss reference signal that the UE uses from Anchor TRP also needs to be update frequently, since pathloss reference signals are expected to be beamformed in FR2 (for example the pathloss reference signal can be one of the beamformed SSBs or TRSs).

Considering mobility also in asymmetric Rel-19 deployments, the DL-TCI state towards the current serving TRP may thus need to be updated frequently. Similarly, the pathloss reference signal and PL Offset that the UE uses may also need to be updated frequently. Since the pathloss reference signal for the UL TCI state targeting the UL-only TRP is currently assumed to be RRC configured, this may require RRC signalling to frequently update the pathloss reference signal towards the serving TRP.

For FR2, when a UE moves around in the cell and needs to update the serving Anchor TRP beam, the pathloss reference signal configured in an UL-TCI state used for UL transmission towards the UL-only TRP may need to be updated with RRC signaling, which generates a significant amount of overhead signaling and latency.

This somewhat defeats the usefulness of additional MAC CE signaling for updated PL Offset as frequent RRC signaling anyway could cater for this in addition. To solve this issue, it would be preferred if in addition to the PL Offset, also the PL-RS associated with an UL-TCI state targeting UL-only TRP could be updated with MAC-CE.

Support that PL-RS of an UL TCI state associated with a UL-only TRP can be updated with the same MAC-CE that updates the path loss offset.

The format then may depend on the PLOffset and PL-RS association to TCI-State(s). A R-bit can be used to indicate if a PL-RS is associated with this TCI-Sate ID.

Fig 3
MAC CE to update pathloss offsets, and DL PL RS per PL-RS associated with UL-TCI state for UL-only TRP linked with a DL-RS associated to an indicated DL TCI state

## 2.2	Control Plane Impacts

In RAN2#129-bis the WA assumption below was confirmed:

From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

Therefore, it should be discussed on how to handle handover and resume case. The NW may provide an RRC configuration to the UE with PL offset and later update this value in MAC CE. This implies that once the NW performs handover, the target NW will not have the latest status of the PL offset from the source RRC configuration message. Hence, if the target wants to apply delta signaling by omitting the PL offset configuration, the target NW would assume the UE would continue to use the PL offset provided in the source RRC configuration, and there would be a mismatch between the PL offset between the UE and the NW.

Similarly, if a source NW sends the UE to RRC_INACTIVE and this UE e.g. attempts to resume on another NW, this other NW will not have the latest info on PL offset value (from the source NW configuration) in case this was updated via MAC CE before sending the UE to RRC_INACTIVE.

To address this, we think it is sufficient to define PL offset as need R. In this manner, whenever the NW would send any RRC configuration message, it should also send a PL offset value if it would want to configure the UE with PL offset, which will avoid any mismatch between the UE and the NW. It should be noted that the current running CR already implements PL offset configuration as need R, hence it just needs to be confirmed that need R will be used and no issue will occur for handover or resume.

Confirm that PL offset configuration is provided as need R. No PL offset mismatch is expected for handover and resume.

Another aspect to discuss is how to capture the following agreements from RAN2#129-bis in RRC:

For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes:

- •	remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- •	a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

For tag2, the current field description in 38.331 is as follows:

tag2

This field is used to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex.

The current field description limits the use of tag2 for Rel-18 2TA, where more than one value for coresetPoolIndex must be present. For Rel-19, this restriction is not present. However, simply remove the restriction would imply that the restriction is also removed for Rel-18 2TA – hence both conditions should be taken into account in RRC.

In our understanding, TS 38.213 also captures such restrictions. Therefore, to avoid mismatches and simplify the description, it could be sufficient to refer to TS 38.213. For instance:

tag2

This field is used to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex if the conditions in TS 38.213 [13] are satisfied.

It can also be considered to include the specific clauses applicable.

The field description of tag2 refers to TS 38.213 for configuration restrictions instead of capturing them explicitly in TS 38.331.

For n-TimingAdvanceOffset2, updates to the field description may still be needed, but RAN1 is still discussing further aspects on n-TimingAdvanceOffset and thus this can be postponed until RAN1 conclusion.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	For FR2, when a UE moves around in the cell and need to update the serving Anchor TRP beam, the pathloss reference signal configured in an UL-TCI state used for UL transmission towards the UL-only TRP may need to be updated with RRC signaling, which generates a significant amount of overhead signaling and latency.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support that PL-RS of an UL TCI state associated with a UL-only TRP can be update with the same MAC-CE that updates the path loss offset .

Proposal 2	Confirm that PL offset configuration is provided as need R. No PL offset mismatch is expected for handover and resume.

Proposal 3	The field description of tag2 refers to TS 38.213 for configuration restrictions instead of capturing them explicitly in TS 38.331.

# 4	References

- RP-242394 NR MIMO Phase 5, Samsung (Moderator), RAN meeting #102 Scotland, December 11-15, 2023