3GPP TSG-RAN WG2#130												R2-2504034

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	8.9.2 Support of Store & Forward

Source: 	NEC

Title:	        Remaining issues on S&F

Document for:	 Discussion

# Introduction

In this contribution, we share our view on some remaining issues:

- Details to provide the transition time from the normal mode to S&F mode

- Whether “S&F operation” indication must be updated when feeder link availability changes

- How to use MME provided satellites IDs

- Consideration on PUR

# Discussion

## Transition time from the normal mode to S&F mode

We agree to introduce the transition time from normal mode to S&F mode, but FFS on details.

Some companies expressed the view to reuse the T-service or to indicate if the transition time is equal to T-service. We prefer not to bind two different functions to the same IE, to keep the specification clean.

=> We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS, and we inform CT1 about this in the LS

Use a separate IE to indicate the transition time from the normal mode to S&F mode (i.e. not reuse or link to T-service)

We will indicate the transition time from S&F mode to normal mode, and if the time duration of the normal mode is short, it is more efficient to indicate the time duration when feeder link is available, and cell is in normal mode:

Broadcast the time duration of the normal mode, instead of the transition time from the normal mode to S&F mode

## “S&F operation” indication

A dynamic “S&F operation” indication was agreed and sent in SIB1:

- When not present:

  - the NTN cell is operating in normal mode (i.e. default/” not-S&F” mode)

- When present, the “S&F operation” indication has two possible settings:

  - ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

  - ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

This means NW will modify system information upon feeder link become available or unavailable as follows:

- Upon feeder link becomes unavailable and NTN cell is operating in “S&F” mode: add the indication

- upon feeder link becomes available and NTN cell is operating in “normal” mode: delete the indication

At meantime, we agreed to have time information on when S&F operation mode switches in both directions. It then becomes duplicated to update “S&F operation” indication at the indicated time. To avoid this duplication and SI modification, we propose to change the dynamic “S&F operation” indication into static “S&F cell” indication

Revisit the agreement on the dynamic “S&F operation” indication and change it into a static “S&F cell” indication (i.e., no need to update the value when feeder link is resumed for short time)

## MME-configured satellite ID list

It is still unclear how to use the MME-configured satellite/satellite set. Below a copy from CR3867 to 23.401[in S2-2502513] is relevant to MME-configured satellites:

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE’s EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with a satellite of the same PLMN that is operating in S&F Mode. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

According to above text, after UE scans and camps successfully on a S&F satellite belong to the same PLMN, if this satellite is one of the MME configured satellites, the satellite/cell potentially holds the UE’s context and or buffers its DL data/signalling. In this case, the UE could re-attempt the connection to continue the NAS procedure or data exchange session. Otherwise, UE may not resume its S&F service. This requires UE to check if the serving satellite ID is one of MME-configured satellite IDs. UE NAS could do the checking. UE AS only need to tell NAS the serving satellite ID

UE NAS may check if the serving satellite is one of the MME-configured satellite ID(s), and to decide whether to resume S&F service

UE AS sends serving satellite ID to NAS

MMC-configured satellite set may be also used for power saving according SA2. Some companies proposed to prioritize the MME-configured satellites during cell reselection. Firstly, it is clearly mentioned that UE may select/camp on “normal” satellite other than the MME-configured ones especially during wait time, particularly when these satellites belong to another PLMN. Hence UE must follow legacy neighbouring satellite information to do cell reselection at least for searching a cell in normal operation mode.

On the other hand, if a neighbouring satellite is a S&F satellites but not one of its MME-configured satellites, UE may refrain from cell reselection to it. This requests network to indicate the operation mode of neighbouring satellites/cells.

RAN2 discuss if network provides operation mode of the neighbouring satellites, and assist UE implementation to prioritize/deprioritize cell reselection to a S&F satellite within/not within MME-configured satellites

Moreover, S&F is tightly linked to discontinuous coverage feature, as stated in WID justification:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

Considering the cases when there is no neighbour cell, for UE in RRC_IDLE, the network may provide information for operating during discontinuous coverage (e.g., ephemeris parameters, the start-time and/or carrier frequency of upcoming NTN payload coverage). And UE shall use this discontinuous coverage information to predict it’s in and out of coverage time window. If UE is out of coverage, it could suspend all AS idle procedure for power saving.

For UE in RRC_CONNECTED, we expect NW will release the UE connection before it is out of coverage. Otherwise, upon declaring RLF, UE may determine not to reestablishment the RRC connection by implementation (e.g., based on ephemeris parameters).

UE(s) who support S&F feature, also support discontinuous coverage-like feature, i.e. upon service link becoming unavailable and there is no neighbor cell:

- For UE in RRC_IDLE, RAN2 assume the UE suspends all AS idle mode operations related to NTN.

- For UE in RRC_CONNECTED mode, either NW will release the UE, or the UE will declare RLF and enter idle mode directly.

## PUR

Transmission using PUR applicable to NB-IoT UEs currently allows pre-configured resources with periodicity ranging from 8 H-SFN to 8192 H-SFN

PUR-PeriodicityAndOffset-r16 ::= CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..255),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

}

And the TA validity, which controls by pur-TimeAlignmentTimer, can have a largest length of 8*PUR periodicity.

pur-TimeAlignmentTimer-r16		INTEGER (1..8)				OPTIONAL,	-- Need OR

Based on the above information, the overall validity of PUR resources could be configured for a relatively long duration. However, PUR will be released upon cell reselection.  In NTN scenario where serving satellite changes frequently, UE needs to repeatedly release and request PUR resources, which reduces the benefits of using PUR.

In one aspect, the condition of running pur-TimeAlignmentTimer can be lifted due to self TA compensation. In another aspect, if UE is provided with MME-configured satellite/satellite sets, potentially all these cells/satellites hold UE’s context (including PUR configuration) even when the UE goes to idle mode and moves to another cell/satellite. This means that UE could store/suspend the configured PUR resources, then resume/reuse it after reconnecting to the MME-configured satellite set.

If a UE is provided with an MME-configured satellite set and PUR resources was configured by one of the MME-configured satellite cells, the UE could resume the PUR configuration once camping on any of the MME-configured satellites.

# Conclusion

In summary, we would like to share our view on remaining details of S&F as follows:

- UE NAS may check if the serving satellite is one of the MME-configured satellite ID(s), and to decide whether to resume S&F service

Proposal 1:	Use a separate IE to indicate the transition time from the normal mode to S&F mode (i.e. not reuse or link to T-service)

Proposal 2:	Broadcast the time duration of the normal mode, instead of the transition time from the normal mode to S&F mode

Proposal 3:	Revisit the agreement on the dynamic “S&F operation” indication and change it into a static “S&F cell” indication (i.e., no need to update the value when feeder link is resumed for short time)

Proposal 4:	UE AS sends serving satellite ID to NAS

Proposal 5:	RAN2 discuss if network provides operation mode of the neighbouring satellites, and assist UE implementation to prioritize/deprioritize cell reselection to a S&F satellite within/not within MME-configured satellites

Proposal 6:	UE(s) who support S&F feature, also support discontinuous coverage-like feature, i.e. upon service link becoming unavailable and there is no neighbor cell:

-	For UE in RRC_IDLE, RAN2 assume the UE suspends all AS idle mode operations related to NTN.

-	For UE in RRC_CONNECTED mode, either NW will release the UE, or the UE will declare RLF and enter idle mode directly.

Proposal 7:	If a UE is provided with an MME-configured satellite set and PUR resources was configured by one of the MME-configured satellite cells, the UE could resume the PUR configuration once camping on any of the MME-configured satellites.