3GPP TSG-RAN WG2 Meeting #130	R2-2504036

St.Julian, Malta,  May 19th – 23rd, 2025

Agenda Item:	8.5.2

Source: 	vivo

Title:         	Remaining issues of on-demand SSB SCell operation

Document for: 	Discussion and Decision

- Introduction

This contribution will discuss the remaining essential open issues of on-demand SSB (OD-SSB) SCell, including details of OD-SSB MAC CE based on the RAN1 progress, L3 RRM measurements for case 1 and case 2, and other stage-3 open issues. RAN1/2/4 agreements of this objective are listed in the Appendix.

- Discussion

OD-SSB MAC CE design

For OD-SSB MAC CE design, there are several issues need to be discussed:

- How to configure multiple values of od-ssb-Periodicity, od-ssb-nrofBurst and ssb-PositionsInBurst for OD-SSB transmission?

- How to support both the explicit and the implicit OD-SSB deactivation with a single MAC CE format?

- If the implicit OD-SSB deactivation has been applied to an OD-SSB, can the network send OD-SSB MAC CE to explicitly deactivate the OD-SSB before the corresponding od-ssb-nrofBurst is reached?

- What is the expected UE behaviour if the newly received OD-SSB MAC CE indicates the activation of an OD-SSB for an SCell while the OD-SSB of that SCell is already in the activated state?

How to configure multiple values of od-ssb-Periodicity, od-ssb-nrofBurst and ssb-PositionsInBurst for OD-SSB transmission?

According to the following RAN1 agreements made in RAN1#118 and RAN1#120bis meetings, RAN1 has agreed that for the following parameters, multiple candidate values can be configured by RRC and the applicable value can be indicated by OD-SSB MAC CE.

- od-ssb-Periodicity

- od-ssb-nrofBurst

- ssb-PositionsInBurst

RAN1#118 Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB FFS: Any other relevant parameters RAN1#120bis Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

And whether these multiple candidate values are reflected as multiple OD-SSB configurations, or a list under a single OD-SSB configuration, is up to RAN2 [1].

Whether more than one on-demand SSB configurations can be configured for the cell to UE or the parameters in row3(periodicity), [row4(offset),row5(half Frame Index)], row6(nr of Burst), and row8 (Positions In Burst) are a list within a on-demand SSB configuration is up to RAN2

In the last RAN2 meeting, RAN2 agreed OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

RAN2#129bis Agreements on OD-SSB - OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

From the above RAN2 agreement, we think it is a common understanding that the multiple candidate values of od-ssb-Periodicity, od-ssb-nrofBurst and ssb-PositionsInBurst are achieved via multiple OD-SSB configurations where each OD-SSB configuration can optionally contain a single value of od-ssb-Periodicity, od-ssb-nrofBurst and ssb-PositionsInBurst. As there is no related formal RAN2 agreement, we suggest RAN2 to first confirm this understanding.

- From signalling perspective, each OD-SSB SCell can be configured with a list of OD-SSB configurations, and each configuration can include the following parameters:

- od-ssb-Periodicity;

- od-ssb-nrofBurst;

- ssb-PositionsInBurst.

How to support both the explicit and the implicit OD-SSB deactivation with a single MAC CE format?

In RAN2#129bis meeting, RAN2 has made the following agreements for OD-SSB MAC CE design, where how to support both the explicit and the implicit OD-SSB deactivation with a single MAC CE format is FFS.

RAN2#129bis Agreements on OD-SSB - For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). - A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. - For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. - OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. - RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

RAN1 has made some agreements related to this issue in RAN1#120bis meeting.

RAN1#120bis Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

With the above RAN1 agreements and our understanding in P1, it is straightforward that when receiving the OD-SSB activation via OD-SSB MAC CE, if the OD-SSB configuration to be activated includes od-ssb-nrofBurst, it means this OD-SSB will be deactivated based on the configured od-ssb-nrofBurst; Otherwise, it means this OD-SSB will be deactivated via OD-SSB MAC CE. Thus, there is no need to define any additional field in the OD-SSB MAC CE to indicate whether the explicit deactivation or the implicit OD-SSB deactivation is applied.

If the implicit OD-SSB deactivation has been applied to an OD-SSB, can the network send OD-SSB MAC CE to explicitly deactivate the OD-SSB before the corresponding od-ssb-nrofBurst is reached?

Actually, we didn’t see any issue to allow the network to do so. Supporting this can provide more flexibility to the network, for example, in scenario #2, the network activates OD-SSB of multiple deactivated SCells (e.g., SCell 1, 2, 3), and may receive the L3 report of some deactivated SCells (e.g., SCell 1) and decide to activate these SCells (e.g., SCell 1) and deactivate the OD-SSB for the other deactivated SCells (e.g., SCell 2, 3).

Given above, we have the following proposal:

- No need for the OD-SSB MAC CE to include a dedicated bit for each OD-SSB SCell to indicate whether explicit or implicit OD-SSB deactivation is applied:

- If the activated OD-SSB configuration for an OD-SSB SCell includes od-ssb-nrofBurst, the activated OD-SSB can be either implicitly deactivated when od-ssb-nrofBurst SSBs are transmitted, or explicitly deactivated by the A/D bit in the OD-SSB MAC CE;

- Otherwise, the activated OD-SSB can only be explicitly deactivated by the A/D bit in the OD-SSB MAC CE.

What is the expected UE behaviour if the newly received OD-SSB MAC CE indicates the activation of an OD-SSB for an SCell while the OD-SSB of that SCell is already in the activated state?

Based on the OD-SSB MAC CE format in Figure 1, the potential UE behaviour is listed below:

- UE behaviour 1: the UE ignores the activation indication of the OD-SSB of this SCell and the corresponding OD-SSB configuration ID filed in the new OD-SSB MAC CE.

- UE behaviour 2: the UE applies the OD-SSB configuration indicated in OD-SSB configuration ID filed in the new OD-SSB MAC CE.

- UE behaviour 3: If the OD-SSB configuration indicated in the new OD-SSB MAC CE is different from the ones the UE is currently using, the UE applies the OD-SSB configuration indicated in the new OD-SSB MAC CE; otherwise, the UE ignores the corresponding OD-SSB configuration ID filed in the new OD-SSB MAC CE.

Figure 1: OD-SSB MAC CE format.

The UE behaviour 1 is against with the RAN1 agreement below, at least for scenario #3B. RAN1 has no conclusion on whether updating the transmission parameter of a transmitted OD-SSB is allowed in other combinations of scenarios and cases, but even if RAN1 concludes that updating is not supported in other combinations, it is not preferred for us to define the different UE behaviour for different combinations in the specification.

RAN1#120bis Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

The main difference between UE behaviour 2 and UE behaviour 3 is that UE behaviour 3 can avoid recounting the od-ssb-nrofBurst of the implicit OD-SSB deactivation. For example, assuming the network activates the OD-SSB of SCell 1 at time T1, and at that time the activated OD-SSB configuration of SCell 1 includes od-ssb-nrofBurst, the UE will consider this OD-SSB will be deactivated based on the configured od-ssb-nrofBurst. At time T2 before the corresponding od-ssb-nrofBurst is reached, the network wants to additionally activate the OD-SSB of SCell 2 but does not want the UE to recount the od-ssb-nrofBurst of the OD-SSB of SCell 1.

From specification change perspective, UE behaviour 2 is simpler, thus is preferred to us. Considering RAN1 may also discuss this issue, e.g., whether the transmission parameter of a transmitted OD-SSB can be updated in other scenarios and cases, and what transmission parameters are allowed to be updated, we suggest to wait for RAN1 further progress on this issue.

- Wait for RAN1 further input about the cases where the OD-SSB transmission parameters can be updated or not.

L3 RRM measurements

Regarding whether always-on SSB (AO-SSB) and/or OD-SSB are measured when both are transmitted in OD-SSB case 2,  RAN2 has agreed to leave this issue to RAN4 to conclude.

RAN2#129 Agreements on OD-SSB

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

Moreover, regarding the frequency location relation between the AO-SSB and OD-SSB in OD-SSB case 2, RAN1 has agreed to support the below scenario.

RAN1#120 Agreement on OD-SSB Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

Thus, L3 measurement on OD-SSB SCell can be left to RAN4, and RAN2 can focus on the discussion for the below open issues:

- Whether there is any specification impact for RRM measurement on neighbour cells?

- If we allow multiple OD-SSBs with the different frequencies for a given SCell?

- How to report the L3 measurement result of OD-SSB?

- Whether dedicated SMTC is configured for OD-SSB configuration?

RRM measurement on neighbour cells:

In the RAN4#112bis meeting, the following agreements have been made.

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. - This agreement can be revisited if it is not aligned with further agreement in other WGs. Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Based on the RAN4 agreements, when the OD-SSB activation indication is received in case 1, the UE performs the OD-SSB based L3 measurement for deactivated SCell, and still uses SMTC for intra-frequency neighbour cell measurements. So, the question is whether to have an additional SMTC for intra-frequency neighbour cell measurements when the OD-SSB activation is indicated for the SCell, where the additional SMTC is aligned with the OD-SSB pattern of the SCell. We understand that from the network perspective, the SSB transmission (e.g., periodicity and time location) of the neighbour cell will not be influenced by the OD-SSB operation of the SCell. In other words, the OD-SSB transmission on an SCell should adapt to the SSB transmission pattern of the neighbor cells, not the other way around.

- No specification impact is needed for the RRM measurement on intra-frequency neighbour cells when the OD-SSB of SCell is activated.

There may be OD-SSB transmission on the neighbour cell, but since this WI only considers OD-SSB for SCell, so if there is any OD-SSB transmission on the neighbour cell,  the L3 measurement of OD-SSB of neighbour cell should be transparent to the UE, i.e., no spec impact is needed for OD-SSB based neighbour cell measurements.

- RAN2 assumes the L3 measurement of neighbour cell OD-SSB is based on legacy SMTC.

If we allow multiple OD-SSBs with the different frequencies for a given SCell?

In RAN2#129bis meeting, the following FFS is identified.

Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

So far, RAN1 has never discussed the multiple OD-SSBs with the different frequencies for this objective. The benefit of supporting this is also unclear, and there are many open issues related, e.g., how many different frequencies are supported, the time location relation or spatial relation among these multiple OD-SSB on the different frequencies. Thus, we suggest not considering multiple OD-SSBs with the different frequencies for a given SCell in this release.

- Not support multiple OD-SSBs with the different frequencies for a given SCell in this release.

How to trigger the L3 measurement report of OD-SSB:

Regarding to the OD-SSB deactivation, RAN1 has made the below agreements.

RAN1#119 Agreement on OD-SSB For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

From these agreements, we understand both periodic OD-SSB transmission and one-shot OD-SSB transmission are possible. For example, with the above option 1, the network can activate the OD-SSB of an SCell when activating the SCell in scenario #2A case 1, and keep this OD-SSB transmission when the SCell remains activated and send the explicit OD-SSB deactivation indication via OD-SSB MAC CE when deactivating the SCell. This can be treated as periodic OD-SSB transmission. With the above option 2, after the OD-SSB is activated, the OD-SSB transmission will continue for a given period of time which won’t be long.

We think periodic OD-SSB is only useful in case 1, and one-shot OD-SSB is useful for both case 1 and case 2. The reporting of periodic OD-SSB transmission in case 1 is similar to that of AO-SSB in legacy. The potential specification impact to support periodic OD-SSB transmission in case 1 is shown below, where the measID that triggered the measurement reporting can be any measID, or the measID linked to A1/2 event or periodic reporting, which is up to network implementation.

5.5.5 Measurement reporting 5.5.5.1 General <Omit> For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows: 1> set the measId to the measurement identity that triggered the measurement reporting; 1> for each serving cell configured with servingCellMO-onDemandSSB: 2> if the reportConfig associated with the measId that triggered the measurement reporting includes rsType and rsType is set to ssb: 3> if the serving cell measurements based on the activated OD-SSB configuration within the MO indicated by servingCellMO-onDemandSSB is available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the activated OD-SSB; 1> set the servCellId within measResultServingMOList to include each NR serving cell that is configured with servingCellMO, if any; <Omit>

Regarding to the one-shot OD-SSB transmission for both case 1 and case 2, the report can be triggered by a new measurement report type (e.g., when the measurement requirement defined in RAN4 is fulfilled following the reception of the MAC CE used for activating the OD-SSB). In this way, the UE reports the available results to the network immediately when OD-SSB MAC CE is received and the RAN4 requirement is fulfilled, and the report can change the SCell status from ‘unknown’ to ‘known’, therefore can speed up the SCell activation. Moreover, such a method can support triggering a single MR for multiple OD-SSB SCells, and we think the existing work adopted in FR2 SCell enhancements can be considered [R2-2313937/R2-2313936]. In their work, the network can configure a measID linked to a report configuration with report type set to reportOnActivation, and when the UE receives the SCell activation indication via SCell A/D MAC CE and if the activated SCell(s) fulfills the measurement requirement as specified in TS 38.133, the UE initiates the measurement reporting procedure.

The potential specification impact to support one-shot OD-SSB transmission for both case 1 and case 2 is shown below, where when the MR is triggered by the new report type, the specification impact for periodic OD-SSB transmission mentioned above is reused to set the content of the triggered MR.

5.5.4 Measurement report triggering 5.5.4.1 General <Omit> If AS security has been activated successfully and if SCell activation(s) indication is received from lower layer, the UE shall: 1> if reportType is set to reportOnActivation for any measId included in the measIdList within VarMeasConfig: 2> if the activated SCell(s) fulfills the measurement requirement as specified in TS 38.133 [14]: 3> include a measurement reporting entry within the VarMeasReportList for this measId: 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 4> initiate the measurement reporting procedure, as specified in 5.5.5. If AS security has been activated successfully and if OD-SSB activation(s) indication is received from lower layer, the UE shall: 1> if reportType is set to reportOnOD-SSB for any measId included in the measIdList within VarMeasConfig: 2> if the activated OD-SSB of SCell(s) fulfill the measurement requirement as specified in TS 38.133: 3> include a measurement reporting entry within the VarMeasReportList for this measId: 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 4> initiate the measurement reporting procedure, as specified in 5.5.5.

The reporting of OD-SSB is summarized in the below Table.

| Case | MO | MR for periodic OD-SSB/AO-SSB | MR for one-shot OD-SSB |
| --- | --- | --- | --- |
| Case 1 | servingCellMO-onDemandSSB is configured for OD-SSB | Existing A1/2 event, periodic reporting, or via any triggered MR. | New report type (e.g., when the measurement requirement defined in RAN4 is fulfilled following the reception of the MAC CE used for indicating OD-SSB activation) |
| Case 2 | servingCellMO-onDemandSSB is configured for OD-SSB | / |  |
|  | servingCellMO is configured for AO-SSB | Existing A1/2 event, periodic reporting, or via any triggered MR. |  |

Given the above, we have the following proposal:

- RAN2 to discuss the below options to trigger the L3 OD-SSB measurement report considering periodic OD-SSB transmission for case 1 and one-shot OD-SSB transmission for both case 1 and case 2:

- Option 1: report is triggered by the existing measurement report type (e.g., periodic, A1/2 event, via any triggered MR);

- Option 2: report is triggered by a new measurement report type (e.g., when the measurement requirement defined in RAN4 is fulfilled following the reception of the MAC CE used for indicating OD-SSB activation).

Whether dedicated SMTC for OD-SSB configuration is needed?

In RAN1#120bis meeting, RAN1 has agreed the following:

Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

So far, RAN4 has not discussed the new requirement for L3 measurement of activated SCell in case 1. As the current requirement is based on the SMTC, if the dedicated SMTC for OD-SSB configuration is configured, the current L3 measurement requirement can be reused, to reduce the specification impact in RAN4.

- Support configuring dedicated SMTC for OD-SSB transmission at least for activated SCell in case 1.

- Conclusion

In this paper, the following proposals are given:

- From signalling perspective, each OD-SSB SCell can be configured with a list of OD-SSB configurations, and each configuration can include the following parameters:

- od-ssb-Periodicity;

- od-ssb-nrofBurst;

- ssb-PositionsInBurst.

- No need for the OD-SSB MAC CE to include a dedicated bit for each OD-SSB SCell to indicate whether explicit or implicit OD-SSB deactivation is applied:

- If the activated OD-SSB configuration for an OD-SSB SCell includes od-ssb-nrofBurst, the activated OD-SSB can be either implicitly deactivated when od-ssb-nrofBurst SSBs are transmitted, or explicitly deactivated by the A/D bit in the OD-SSB MAC CE;

- Otherwise, the activated OD-SSB can only be explicitly deactivated by the A/D bit in the OD-SSB MAC CE.

- Wait for RAN1 further input about the cases where the OD-SSB transmission parameters can be updated or not.

- No specification impact is needed for the RRM measurement on intra-frequency neighbour cells when the OD-SSB of SCell is activated.

- RAN2 assumes the L3 measurement of neighbour cell OD-SSB is based on legacy SMTC.

- Not support multiple OD-SSBs with the different frequencies for a given SCell in this release.

- RAN2 to discuss the below options to trigger the L3 OD-SSB measurement report considering periodic OD-SSB transmission for case 1 and one-shot OD-SSB transmission for both case 1 and case 2:

- Option 1: report is triggered by the existing measurement report type (e.g., periodic, A1/2 event, via any triggered MR);

- Option 2: report is triggered by a new measurement report type (e.g., when the measurement requirement defined in RAN4 is fulfilled following the reception of the MAC CE used for indicating OD-SSB activation).

- Support configuring dedicated SMTC for OD-SSB transmission at least for activated SCell in case 1.

- References

[1] R2-2503314, LS on Rel-19 higher layers parameters list Post RAN1#120bis (R1-2503154; contact: Ericsson).

- Appendix (RAN1/2/4 Agreements)

RAN1 Agreements

RAN1#120bis

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}

  - N2= [8]

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement

Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

- Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

  - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

    - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - (Working assumption) T=T_min

  - RAN4 to confirm the above two working assumptions

  - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

LS to RAN4 on time instance B for on-demand SSB SCell operation is agreed. Final LS in R1-2503108.

Agreement

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

- od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap.

- od-ssb-PositionsInBurst can be absent for Case #2

  - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- A CSI report configuration can be associated with

  - both of AO-SSB and OD-SSB

    - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met

    - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst

    - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

  - OD-SSB only

    - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

RAN1#120

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following.

- Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-C: The specification has no restriction with regards to overlapping

Agreement

- For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC,

  - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e.,

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

    - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

      - By defining reference point as SFN where AO-SSB burst exists

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

Conclusion

The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19

- Scenario #3A and Case #1

- Scenario #3A and Case #2

Above does not impact discussion on SSB periodicity adaptation in time domain

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633.

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options

- Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication

- Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB”

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

RAN1#119

Agreement Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress. Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst. Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement - New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19. Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. Agreement Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1: - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

RAN1#118bis

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not Conclusion No consensus on the support of on-demand SSB SCell operation triggered by UE. Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

RAN1#118

Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 Agreement LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438. Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

RAN1#117

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not

RAN1#116bis

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreement The following agreement from RAN1#116 is modified (in red) - For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement) Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

RAN1#116

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE Agreement - For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

RAN2 Agreements

RAN2#129bis

Agreements on OD-SSB - For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). - A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. - For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. - OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. - RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS. - Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

RAN2#129

Agreements on OD-SSB

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

RAN2#127bis

Agreements on OD-SSB SCell 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

RAN2#127

Scenarios on OD-SSB: - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Measurement on OD-SSB: - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

RAN4 Agreements

RAN4#114bis

Agreement: New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation. Sub-issue 1: Lower bound for measurement in FMW Agreement: Introduce UE capability on whether a lower bound is needed, i.e., - The lower bound is not needed (default value), or, - A lower bound is needed and based on OD-SSB periodicity of [10] ms - Further confirm whether [10] is feasible, and only one value The additional processing timing is T_processing, the value of T_processing: Agreement: Intorduce different values with UE capability: [2 or 3] ms and 5 ms

RAN4#114

Agreement: - RAN4 not to support OD-SSB based L1 measurements for deactivated Scell. Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity - RAN4 to support OD-SSB L1 measurement after SCell activation. - FFS the requirement when DRX is configured. - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi) - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung) - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT) - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received. Agreement: - The legacy measurement accuracy is applied for OD-SSB based measurement. Agreement: - RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion. - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell - No RAN4 spec. change is expected. - UE does not need to differentiate the OD-SSB neighbour cell with legacy SSB neighbour cell. - UE follows legacy deactivated SCC measurement requirement. - The agreement can be revisited with other WG’s further progress. Agreement: - For Scenario 1a: No RAN4 spec. change. - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell - No RAN4 spec. change is expected. - UE follows legacy deactivated SCC measurement requirement. Agreement for Scenario 1b: - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell - No RAN4 spec. change is expected. - UE does not need to differentiate the OD-SSB neighbour cell with legacy SSB neighbour cell. - UE follows legacy deactivated SCC measurement requirement. - The agreement can be revisited with other WG’s further progress. Agreement for Scenario 1c: - For FR1, follow legacy requirement for neighbour cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

RAN4#113

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be ready able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement. Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - The wording can be further clarified in offline discussion. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue. Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequeny for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. Agreement: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

RAN4#112bis

Sub-topic 1-1: General OD-SSB requirements Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. - This agreement can be revisited if it is not aligned with further agreement in other WGs. Issue 1-1-3: OD-SSB type Agreement: - RAN4 to first take NCD-SSB as a start point for OD-SSB requirements. Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 1) Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell. Sub-topic 1-3: OD-SSB based deactivated SCell L3 measurement (Case 2) Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation - Background From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements. - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different frequencies. Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement. Issue 1-3-2: Deactivated SCell measurement before OD-SSB transmission activation Agreement: - The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) before UE receiving OD-SSB transmission indication. Issue 1-3-3: Deactivated SCell measurement after OD-SSB transmission deactivation Agreement: - The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) after the OD-SSB transmission deactivation. Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo) Sub-topic 1-4: OD-SSB based SCell activation Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

RAN4#112

Sub-topic 1-1: General OD-SSB requirements Issue 1-1-1: OD-SSB work plan - Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed. Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement. Issue 1-1-3: Deactivated SCell measurement requirement - Agreement: - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement. - Includes both FR1 and FR2-1 Sub-topic 1-2: OD-SSB requirements(Case 1) Deactivated SCell L3 measurement part Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion. Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. Other option is not precluded