3GPP TSG-RAN WG2 Meeting #130                                    R2-2504039

St.Julians, Malta, May 19th – 23rd , 2025

Agenda Item:	8.11.2

Source:	vivo

Title:	Remaining issues on RACH procedure in SBFD

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues on RACH procedure in SBFD.

Discussion

Msg1 repetition in SBFD

In the post RAN2#129bis email discussion on running CR of TS 38.321 [1], issues on the relation between SBFD and Msg1 repetition were raised by the rapporteur, including:

Editor’s Note: FFS whether RA resource set reselection can be performed or not, after the RO type switching, given that the applicable Msg1 repetition number(s) may become different. Editor’s Note: Whether Msg1 repetition number fallback can be supported with SBFD RO or not. Editor’s Note: FFS the order of RO type fallback and Msg1 repetition number fallback if both are supported.

In general, we should first figure out the order to treat the issues to minimize repetitive discussion. We suggest to discuss the editor’s notes in the following order:

- Whether Msg1 repetition number fallback can be supported with SBFD RO or not.

- FFS whether RA resource set reselection can be performed or not, after the RO type switching, given that the applicable Msg1 repetition number(s) may become different.

- FFS the order of RO type fallback and Msg1 repetition number fallback if both are supported.

Whether Msg1 repetition number fallback can be supported with SBFD RO or not.

Considering RAN1 has agreed on separate Msg1 repetition configuration for SBFD, including all the possible Msg1 repetition numbers 2/4/8 [2], we think it is natural to reach the first conclusion that Msg1 repetition number fallback can be supported with SBFD RO.

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

In other words, if the UE chooses SBFD RO and perform Msg1 repetition (e.g. Msg1 repetition number is 2) for the first RACH attempt, the UE can perform Msg1 repetition number fallback (e.g. from 2 to 4 and then 8) when PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1 or PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1 as legacy, if the corresponding Msg1 repetition number in SBFD is applicable.

Proposal 1: Msg1 repetition number fallback can be supported with SBFD RO, e.g. Msg1 repetition number can fallback from 2 to 4, or from 4 to 8 as legacy, if the corresponding Msg1 repetition number in SBFD is applicable.

FFS whether RA resource set reselection can be performed or not, after the RO type switching, given that the applicable Msg1 repetition number(s) may become different.

We think this is the core issue of this session. Rather than directly treating this issue, we think RAN2 should first discuss whether RO type fallback is allowed when the applicable maximum Msg1 repetition number of the target RO type after RO type fallback is smaller the current Msg1 repetition number of the source RO type. If this is allowed, then we observe that RA resource set reselection is inevitable because the original Msg1 repetition number is not applicable anymore. Therefore, there are two way-forwards to handle this issue:

-	Way-forward 1: RO type fallback is not allowed if the current Msg1 repetition number is not applicable after RO type fallback; Otherwise, the UE can perform RO type fallback.

-	Way-forward 2: RO type fallback is always allowed. If the current Msg1 repetition number is not applicable after RO type fallback, the UE reselect to the corresponding RA resource set of the highest applicable Msg1 repetition number of the target RO type.

We observe both way-forwards work, and it is only a matter of preference to follow which one. Therefore, we propose RAN2 to discuss and down-select from the 2 way-forwards. The corresponding draft running CRs for the 2 way-forwards are provided in the Annex.

Proposal 2: RAN2 to discuss and down-select from the following 2 way-forwards to address the issue of Msg1 repetition number inapplicability after RO type fallback:

- Way-forward 1: RO type fallback is not allowed if the current Msg1 repetition number is not applicable after RO type fallback; Otherwise, the UE can perform RO type fallback.

- Way-forward 2: RO type fallback is always allowed. If the current Msg1 repetition number is not applicable after RO type fallback, the UE select the corresponding RA resource set of the highest applicable Msg1 repetition number of the target RO type.

FFS the order of RO type fallback and Msg1 repetition number fallback if both are supported.

The corresponding text in the current running CR is as below for reference [1]:

2> if the Random Access procedure is not completed: 3> if preambleTransMaxRO-Type is applied, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1: 4> if RO_TYPE is set to SBFD-RO: 5> set the RO_TYPE to non-SBFD-RO. 4> else if RO_TYPE is set to non-SBFD-RO: 5> set the RO_TYPE to SBFD-RO. 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

As long as the conclusion for proposal 2 is determined, we think this issue is only a matter of preference. We propose to simply follow the current running CR.

Proposal 3: RO type fallback can be either before or after Msg1 repetition number fallback. Follow the current running CR that RO type fallback is before Msg1 repetition number fallback.

RO selection criteria configuration

After RAN2#129, the RRC rapporteur suggested in the post email discussion that SSB threshold may not be configured by the network. We think this is against what we have agreed in the past meeting, i.e. if the NW does not indicate RO type, the UE selects RO based on SSB RSRP threshold [3]. Therefore, we propose RAN2 to confirm that, if the NW does not indicate RO type, the NW will at least configure an SSB RSRP threshold for RO type selection.

Proposal 4: If the NW does not provide RO type indication, the NW will at least configure an SSB RSRP threshold for RO type selection in the SBFD RACH configuration.

SSB/CSI-RS selection

SSB/CSI-RS selection is a step in random access procedure. If not indicated by the NW, the UE compares the RSRP of the SSB/CSI-RS with the threshold and select an SSB/CSI-RS with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS. The UE then determines RO based on the selected SSB/CSI-RS [3].

In SBFD, legacy ROs and additional ROs can be both mapped to the same SSB/CSI-RS. Considering the NW can evaluate the CLI in SBFD resources based on the UE reports and adjust the SBFD configuration accordingly, the NW may configure different threshold for the UE to select additional ROs or legacy ROs to minimize the CLI in SBFD resources. However, we may just rely on the legacy SSB/CSI-RS selection method. Therefore, we propose RAN2 to confirm that SBFD operation has no impact on SSB/CSI-RS selection for random access procedure.

Proposal 5: Legacy SSB/CSI-RS selection procedure is reused for random access procedure with SBFD operation.

Msg3-based early identification

RAN2 discussed about Msg-3 based method for several meetings and yet did not achieve consensus. From our point of view, when the UE chooses additional RO for preamble transmission, the NW is able to identify SBFD UE and thus can schedule SBFD Msg3 resources so that the SBFD UE may have higher possibility to successfully complete the random access procedure. However, if the UE chooses legacy RO for preamble transmission, this may reflect that the current situation for the UE is not suitable for further using SBFD resources for the coming Msg3 transmission. Besides, the NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed. Therefore, we do not observe the benefit for Msg-3 based early identification if the UE chooses legacy RO.

Observation 1: If the SBFD-aware UE chooses legacy RO, it reflects the fact that SBFD-based transmission may not be a good choice from scheduling point of view within some upcoming duration. The NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed.

Proposal 6: Do not support Msg3-based early identification for SBFD-aware UE.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: Msg1 repetition number fallback can be supported with SBFD RO, e.g. Msg1 repetition number can fallback from 2 to 4, or from 4 to 8 as legacy, if the corresponding Msg1 repetition number in SBFD is applicable.

Proposal 2: RAN2 to discuss and down-select from the following 2 way-forwards to address the issue of Msg1 repetition number inapplicability after RO type fallback:

- Way-forward 1: RO type fallback is not allowed if the current Msg1 repetition number is not applicable after RO type fallback; Otherwise, the UE can perform RO type fallback.

- Way-forward 2: RO type fallback is always allowed. If the current Msg1 repetition number is not applicable after RO type fallback, the UE select the corresponding RA resource set of the highest applicable Msg1 repetition number of the target RO type.

Proposal 3: RO type fallback can be either before or after Msg1 repetition number fallback. Follow the current running CR that RO type fallback is before Msg1 repetition number fallback.

Proposal 4: If the NW does not provide RO type indication, the NW will at least configure an SSB RSRP threshold for RO type selection in the SBFD RACH configuration.

Proposal 5: Legacy SSB/CSI-RS selection procedure is reused for random access procedure with SBFD operation.

Observation 1: If the SBFD-aware UE chooses legacy RO, it reflects the fact that SBFD-based transmission may not be a good choice from scheduling point of view within some upcoming duration. The NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed.

Proposal 6: Do not support Msg3-based early identification for SBFD-aware UE.

References

- [Post129bis][213][SBFD] Running CR for 38.321 (Samsung)

- RAN1#119 meeting minutes

- TS 38.321, V18.5.0

Annex

Draft CR in TS 38.321 for Proposal2_Way-forward 1

- 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

<irrelevant text omitted>

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

<irrelevant text omitted>

2>	if the Random Access procedure is not completed:

3> if preambleTransMaxRO-Type is applied, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1:

4> if RO_TYPE is set to SBFD-RO:

5> if the Random Access Preamble is not transmitted with repetitions

6> set the RO_TYPE to non-SBFD-RO.

5> else if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure

6> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a same or higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available for non-SBFD-RO:

7> set the RO_TYPE to non-SBFD-RO.

4> else if RO_TYPE is set to non-SBFD-RO:

5> if the Random Access Preamble is not transmitted with repetitions

6> set the RO_TYPE to SBFD-RO.

5> else if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure

6> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a same or higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available for SBFD-RO:

7> set the RO_TYPE to SBFD-RO.

3>	if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

4>	if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or

4>	if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1:

5>	if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available:

6>	select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

<irrelevant text omitted>

Draft CR in TS 38.321 for Proposal2_Way-forward 2

- 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

<irrelevant text omitted>

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

<irrelevant text omitted>

2>	if the Random Access procedure is not completed:

3> if preambleTransMaxRO-Type is applied, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1:

4> if RO_TYPE is set to SBFD-RO:

5> set the RO_TYPE to non-SBFD-RO.

5> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

6> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is not available for non-SBFD-RO:

7> select the set of Random Access resources associated with the highest Msg1 repetition number of non-SBFD-RO with the same feature or feature combination for this Random Access procedure;

7> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

4> else if RO_TYPE is set to non-SBFD-RO:

5> set the RO_TYPE to SBFD-RO.

5> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

6> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is not available for SBFD-RO:

7> select the set of Random Access resources associated with the highest Msg1 repetition number of SBFD-RO with the same feature or feature combination for this Random Access procedure;

7> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

3>	if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

4>	if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or

4>	if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1:

5>	if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available:

6>	select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

<irrelevant text omitted>