3GPP TSG-RAN WG2 Meeting #130	R2-2504049

St. Julien, Malta, 19th – 23rd May 2025

| Agenda Item: | 8.6.4 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Remaining issues on CLTM |
| Document for: | Discussion and Decision |

1.	Introduction

This paper handles details of RACH-less CLTM execution procedure and coexsitence related issue. For RACH-less CLTM execution procedure, we discuss the following remaining issue, i.e., FFS whether there is no CG resource associated with the triggered beam for L1 based RACH-less CLTM cell switch, and beam selection procedure for L3 based CLTM. For coexistence, it is discussed FFS whether Rel-19 CLTM can coexist with (e)RedCap, CovEnh, DAPS, and L3 HO.

2.	Discussion

2.1.	RACH-less CLTM execution procedure

In the meeting RAN2#129, the followings were agreed for RACH-less CLTM [1]:

When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

Do not support DG-based first UL transmission for RACH-less conditional LTM.

Based on the above agreements, if the UE has a valid TA value for the candidate cell, the UE performs RACH-less CLTM cell switch toward the candidate cell when an execution condition for the candidate cell is met. This means that whether to perform RACH during CLTM cell switch execution depends only on the UE has a valid TA for the associated candidate cell. In the case of L1 event based RACH-less CLTM cell switch execution, UE performs RACH-less CLTM toward the candidate cell with the triggered beam (i.e., the candidate beam of a satisfied CLTM execution condition). That is, the UE autonomously selects the target beam based on evaluation of CLTM executions.

CG can be associated with subset of all beams (i.e., subset of all SSBs) for RACH-less LTM cell switch based on the RRC specification. Since the UE autonomously selects the target beam based on evaluation of CLTM executions, a CLTM candidate cell should associate one or more CGs with all candidate beams associated with CLTM execution conditions if the UE is allowed to access the candidate cell without RA procedure (i.e., RACH-less CLTM). In the case where CG is not associated with the triggered beam in CLTM execution, there is no proper UE action for RACH-less cell switch because DG-based approach is not supported for RACH-less CLTM.

To address the problem described above, the network should configure CGs for all candidate beams associated with candidate cells that allow for UE to perform RACH-less CLTM. But, it seems like too much burden on the network. Further, reserving many CGs for CLTM cell switch may lead to inefficient radio resource utilization. Alternatively, the UE performs RACH-based CLTM cell switch if CG is not associated with the triggered beam.

Proposal 1. For L1 based RACH-less CLTM, the UE performs RACH-based CLTM cell switch if CG is not associated with the triggered beam.

In L3 based CLTM, more than one LTM candidate cells can fulfill the LTM cell switch execution at the same time. It can be discussed how to handle this case as noted in the RRC open issue list (i.e., RRC-3: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution). In legacy CHO, it is up to UE implementation which one to select when multiple CHO candidate cells fulfill the execution condition. For L3 based CLTM, it is straightforward to follow the legacy principle. In other words, for L3 based CLTM, it is up to UE implementation to select a candidate cell when multiple candidate cells fulfill an L3 execution condition.

Proposal 2. For L3 based CLTM, it is up to UE implementation to select a candidate cell when multiple candidate cells fulfill an L3 execution condition.

In order for UE to evaluate an L3 execution condition, beam level measurement results of multiple beams in a candidate cell are combined into a cell level measurement. This implies that it is ambiguous for the UE to select which beam of a fulfilled candidate cell for L3 based RACH-less CLTM execution. In Rel-18, RACH-less L3 handover was introduced. To be specific, the UE select the beam for RACH-less handover based on SSB evaluation, i.e., a threshold (e.g. cg-RRC-RSRP-ThresholdSSB) is used for beam selection in RACH-less handover. We think the beam selection procedure of Rel-18 RACH-less handover can be applied to the beam selection in L3 based RACH-less CLTM.

The network configures the UE with one or more L3 execution conditions for L3 based CLTM and the corresponding CG resources and threshold. If an L3 execution condition is met and if a valid TA for the candidate has been obtained, the UE selects a beam above the threshold for L3 based RACH-less CLTM. Then, the UE transmits the first UL signal (i.e., RRCReconfigurationComplete message) on the selected beam.

Proposal 3. For L3 based CLTM, the UE selects a beam above the threshold if an L3 execution condition is met and if a valid TA for the candidate has been obtained. Then, the UE transmits the first UL signal on the selected beam.

In Rel-18 RACH-less handover, the UE performs the random access procedure if there is no beam above the threshold, which can also be applied to Rel-19 CLTM or if there is no valid configured uplink grant. That is, for L3 based LTM, the UE performs RACH-based CLTM if there is no beam above the threshold for the candidate fulfilling the L3 execution condition or if there is no valid configured uplink grant.

Proposal 4. For L3 based CLTM, the UE performs RACH-based CLTM if there is no beam above the threshold for the candidate fulfilling the L3 execution condition or if there is no valid configured uplink grant.

5.8.2 Uplink […unreleated part is omitted…] For the uplink grant configured for configured grant Type 1 for RACH-less handover, if the configured uplink grant is valid according to TS 38.214 [7] for which the above formula is satisfied and RACH-less handover is not successfully completed, the MAC entity shall: 1> if the initial transmission of RACH-less handover has been performed according to clause 5.4.1 and 5.33: 2> if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for the initial transmission of RACH-less handover (i.e., retransmission of initial transmission of RACH-less handover): 3> select this SSB; 3> indicate the SSB index corresponding to the configured uplink grant to the lower layer; 3> consider this configured uplink grant as valid. 1> else if at least one SSB corresponding to the configured uplink grant with SS-RSRP above cg-RRC-RSRP-ThresholdSSB is available: 2> select an SSB with SS-RSRP above cg-RRC-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant; 2> indicate the selected SSB index to the lower layer; 2> consider this configured uplink grant as valid. […unreleated part is omitted…]

Based on TS 38.321 (i.e., the procedural text given above), it is not specified to select which beam when multiple beams are above the threshold as shown in the yellow-highlighted part. This means that it is up to UE implementation to select a beam when multiple beams are above the threshold in Rel-18 RACH-less handover. Similarly, in RAN2#129, RAN2 agreed that for L1 based CLTM it is up to UE implementation to select a beam and perform CLTM when multiple candidate beams satisfy the CLTM condition. We think this principle can be applied to L3 based CLTM, i.e., it is up to UE implementation to select a beam when multiple beams are above the threshold for beam selection in L3 based RACH-less CLTM.

Proposal 5. For beam selection in L3 based RACH-less CLTM, it is up to UE implementation to select a beam when multiple beams are above the threshold.

2.2.	Coexistence related issues

In the meeting RAN2#129, RAN2 agreed that Rel-19 inter-CU LTM could coexist with CovEnh and (e)RedCap [1]. For CLTM, the coexistence with (e)RedCap and CovEnh is still FFS. In our view, there is no blocking issue to support the coexistence with (e)RedCap and CovEnh for CLTM. Therefore, same as Rel-18 LTM and Rel-19 inter-CU LTM, Rel-19 CLTM could coexist with CovEnh and (e)RedCap.

Proposal 6. Support coexistence between Rel-19 CLTM and (e)RedCap.

Proposal 7. Support coexistence between Rel-19 CLTM and CovEnh.

The coexistence between Rel-18 LTM and DAPS is not suppored. It is simple to follow the Rel-18 principle for the coexsitence between Rel-19 CLTM and DAPS.

Proposal 8. Do not support coexistence between Rel-19 CLTM and DAPS.

In the meeting RAN2#127, the following is agreed about the coexistence between L3 HO (including both the network triggered L3 HO and CHO) and Rel-19 inter-CU LTM [2]:

L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

-	When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

-	The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

-	For the execution order between CHO and LTM, Rel-18 principle is applied.

For Rel-19 CLTM, the coexistence with L3 HO is still FFS. The target scenario of L3 HO and that of CLTM are different, i.e., L3 HO can be used for all scenarios including both inter-CU and intra-CU cases while CLTM can be used only for intra-CU case. This means that it may be beneficial to support the coexistence between Rel-19 CLTM and L3 HO due to the differences in supporting scenarios. Hence, we think it would be good to support the coexistence and RAN2 can follow the agreement for the coexistence between L3 HO and Rel-19 inter-CU LTM in order to support the coexistence between L3 HO and Rel-19 CLTM.

Proposal 9. Support coexistence between Rel-19 CLTM and L3 HO (including both the network triggered L3 HO and CHO), and follow the principle of the coexistence between Rel-19 inter-CU LTM and L3 HO.

For the coexistence between CLTM and CHO, it is FFS whether to simultaneously configure CLTM and CHO for the same candidate cell. In our view, there is no reason and motivation to simultaneously configure CLTM and CHO for the same candidate cell but it can be handled by network implementation. For example, network implementation does not configure CLTM and CHO for the same candidate cell at the same time.

Proposal 10. It is up to network implementation whether to simultaneously configure CLTM and CHO for the same candidate cell.

3.	Conclusion

In this paper, we have made the following observations and proposals:

RACH-less CLTM execution procedure

Proposal 1. For L1 based RACH-less CLTM, the UE performs RACH-based CLTM cell switch if CG is not associated with the triggered beam.

Proposal 2. For L3 based CLTM, it is up to UE implementation to select a candidate cell when multiple candidate cells fulfill an L3 execution condition.

Proposal 3. For L3 based CLTM, the UE selects a beam above the threshold if an L3 execution condition is met and if a valid TA for the candidate has been obtained. Then, the UE transmits the first UL signal on the selected beam.

Proposal 4. For L3 based CLTM, the UE performs RACH-based CLTM if there is no beam above the threshold for the candidate fulfilling the L3 execution condition or if there is no valid configured uplink grant.

Proposal 5. For beam selection in L3 based RACH-less CLTM, it is up to UE implementation to select a beam when multiple beams are above the threshold.

Coexistence related issues

Proposal 6. Support coexistence between Rel-19 CLTM and (e)RedCap.

Proposal 7. Support coexistence between Rel-19 CLTM and CovEnh.

Proposal 8. Do not support coexistence between Rel-19 CLTM and DAPS.

Proposal 9. Support coexistence between Rel-19 CLTM and L3 HO (including both the network triggered L3 HO and CHO), and follow the principle of the coexistence between Rel-19 inter-CU LTM and L3 HO.

Proposal 10. It is up to network implementation whether to simultaneously configure CLTM and CHO for the same candidate cell.

4.	References

[1] R2-2501331, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung), 3GPP TSG RAN WG2#129, Athens, Greece, 17th - 21st February 2025

[2] R2-2407571, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung), 3GPP TSG RAN WG2#127, Maastricht, Netherlands, Aug 19th – 23rd, 2024