3GPP TSG-RAN2#130										                             R2-2504082

St.Julians, Malta, May 19th – 23rd , 2025

Source: 			ZTE Corporation, Sanechips

Title: 	                  Consideration on IoT- NTN TDD mode

Agenda item:	      8.17

Document for:       Discussion and Decision

- Introduction

In RAN#106, the revised WID on introduction of IoT-NTN TDD mode[1] has been agreed to support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period[2]. In the RAN2#129 and RAN2#129bis meeting, some agreements has been made for supporting the IoT-NTN TDD mode.

In this contribution, we will further discuss the IoT-NTN TDD mode related RAN2 impacts and give our proposals.

- Discussion

# Issue 1: Impact on higher layer Timers

In RAN2#129bis meeting[4], the following agreement has been made:

Agreements: 3. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

In the TS 38.321[8], the PDCCH period is defined as follows:

PDCCH period (pp): Refers to the interval between the start of two consecutive PDCCH occasions and depends on the currently used PDCCH search space, as specified in TS 36.213 [2]. A PDCCH occasion is the start of a search space and is defined by subframe k0 as specified in clause 16.6 of TS 36.213 [2]. The calculation of number of PDCCH-subframes for the timer configured in units of a PDCCH period is done by multiplying the number of PDCCH periods with npdcch-NumRepetitions-RA when the UE uses the common search space or by npdcch-NumRepetitions when the UE uses the UE specific search space. When counting a timer whose length is calculated in PDCCH-subframes, the UE shall include PDCCH-subframes that will be dropped or not required to be monitored as specified in clause 16.6 of TS 36.213 [2]. The calculation of number of subframes for the timer configured in units of a PDCCH period is done by multiplying the number of PDCCH periods with duration between two consecutive PDCCH occasions.

It can be see that the length of timer in unit of PDCCH periods is defined as number of PDCCH-subframes, e.g. the length of timer in unit of PDCCH periods = the number of PDCCH-subframes, where the number equals to (the number of PDCCH periods * npdcch-NumRepetitions-RA) or (the number of PDCCH periods * npdcch-NumRepetitions) .

Observation 1: In TS 36.321 specification, the length of timer in unit of PDCCH periods is defined as number of PDCCH-subframes.

In TS 36.321[8], the PDCCH-subframe for NB-IoT is defined as all NB-IoT downlink subframes as follows:

PDCCH-subframe: Refers to a subframe with PDCCH. This represents the union over PDCCH-subframes for all serving cells excluding cells configured with cross carrier scheduling for both uplink and downlink, as specified in TS 36.331 [8]; except if the UE is not capable of simultaneous reception and transmission in the aggregated cells where this instead represents the PDCCH-subframes of the SpCell. ... - For NB-IoT UE, all subframes that are part of the NPDCCH search space represent PDCCH-subframes among all NB-IoT downlink subframes, including those which the UE is not required to monitor as specified in clause 16.6 of TS 36.213 [2].

Observation 2: In TS 36.321 specification, the PDCCH-subframe for NB-IoT is defined as all NB-IoT downlink subframes.

And the IoT NTN TDD work item is specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, the PDCCH-subframe definition for NB-IoT UE also applies to IoT NTN TDD UE.

Observation 3: Based on the IoT NTN TDD work item, the PDCCH-subframe definition for NB-IoT UE also applies to IoT NTN TDD UE.

Thus, the length of timer in unit of PDCCH periods is clear and clarification for the impact of invalid subframes is not necessary.

Proposal 1: For timer in unit of PDCCH periods, the definition in existing specs is clear enough, not need for further clarification.

When determine the value range of rel-15 NB-IoT TDD timer in unit of ms or s, the maximal PUSCH repetition number(e.g. 128 repetitions) , the maximal PDCCH repetition number(e.g. 2048 repetitions) and the maximal PDSCH repetition number(e.g. 2048 repetitions) are considered, if coverage enhancement(e.g. transmission with repetitions) is not considered in IoT-NTN TDD pattern with a period of 9 radio frames(e.g. the repetition number of all channels will be 1), the existing value range of Timer with unit of ms or s is enough for IoT-NTN TDD; but if IoT NTN TDD supports same coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions), some of the existing value range of Timer with unit of ms or seconds(e.g. T300, T301, T311,T-PollRetransmit) will be insufficient and need to be extended(e.g. extended to 4 times of legacy TDD value).

Observation 4: If IoT NTN TDD supports same coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions), some of the existing value range of Timer with unit of ms or seconds(e.g. T300, T301, T311,T-PollRetransmit) will be insufficient and need to be extended(e.g. extended to 4 times of legacy TDD value).

Based on the current RRC specification[8], MIB transmission repetitions and SIB1-NB transmission repetitions are made for coverage enhancement, e.g.

MIB uses a fixed schedule with a periodicity of 640 ms and repetitions made within 640 ms, and the transmissions are arranged in 8 independently decodable blocks of 80 ms duration. UE can decode the MIB if it can receive any one of 64 MIB blocks in 640ms.

SIB1 transmission reliability has also been considered for coverage enhancement case, e.g. The starting frame for the first transmission of the SIB1-NB is derived from the cell PCID and the number of repetitions within the 2560 ms period and repetitions are made, equally spaced, within the 2560 ms period (see TS 36.213 [23]). TBS for SystemInformationBlockType1-NB and the repetitions made within the 2560 ms are indicated by schedulingInfoSIB1 field in the MIB-NB. If additionalTransmissionSIB1 is set to TRUE in the MIB-NB, additional SIB1-NB transmission occurs in subframe #3 of the same radio frames where SIB1-NB transmission occurs with the same number of repetitions.

But in RAN1#120 meeting[3], the following agreement has been made[4],

Agreement NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

In RAN1#120bis meeting[5], the following agreement has been made[4],

Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact.

Dropped some NPBCH and SIB1 repetition may impact MIB and SIB1 coverage range. With the limitation of NPBCH SIB1 coverage, the legacy coverage enhancement level(e.g. DL with maximal 2048 repetitions) can not be provided in IoT NTN TDD.

Observation 5: Based on the RAN1 agreement, the legacy coverage enhancement level (e.g. DL with maximal 2048 repetitions) can not be provided in IoT NTN TDD.

Based on RAN1 progress it seems RAN1 will not further discuss the maximum number of repetition number that can be supported in IoT TDD system considering the overall link budget but would rather to leave it to NW implementation. Considering that the maximal value of timer in unit of ms or s is very large in NB-IoT, it shall be safe for RAN2 to assume that there is no need to extend the value range of timer in unit of ms or s for IoT NTN TDD and the applicable repetition number will be up to NW implementation. This assumption can be revisit if RAN1 provides further updates on the the exact overage enhancement level(e.g. the maximal repetitions number) that is supported in IoT NTN TDD system.

Proposal 2: RAN2 assumes not extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD. This assumption can be revisited if RAN1 provides the exact overage enhancement level(e.g. the maximal repetitions number) that is supported in IoT NTN TDD system.

In the current TS 36.331 specification[7], the ra-ResponseWindowSize and mac-ContentionResolutionTimer has the maximal absolute value limitation(e.g. 10.24s for FDD and 20.48s for TDD) as follows:

RACH-ConfigCommon-NB field descriptions

...

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. mac-ContentionResolutionTimer-r15 is only applicable for EDT. UE performing EDT shall use mac-ContentionResolutionTimer-r15, if present. For FDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 20.48s).

...

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. For FDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 20.48s).

Considering that there are less valid DL subframes and valid UL subframes in IoT NTN TDD than in legacy NB-IoT system, a larger absolute value limitation(e.g. 20.48s) is preferred for IoT NTN TDD.

Proposal 3: For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for TDD(i.e., 20.48s) is used for IoT NTN TDD.

Issue 2: About NPRACH related issue

In RAN1#120bis meeting[5], the following agreement has been made:

Agreement NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU. - NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported

In 3GPP TS36.321[8], the RA-RNTI is calculated as following:

For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0. For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2) where SFN_id is the index of the first radio frame of the specified PRACH and H-SFN is the index of the first hyper frame of the specified PRACH. The PDCCH transmission and the PRACH resource are on the same carrier.

If proposal 3 is agreed, then tone RAR window could cross two H-SFNs. Therefore (H-SFN mod 2)  should be considered in RA-RNTI calculation to indicate which H-SFN the PRACH transmission is made,  similar as that for NB-IoT UEs operating in TDD mode. And considering that DL subframes and UL subframes are on the same carrier for IoT NTN TDD, carrier id need not be considered in RA-RNTI calculation for IoT NTN TDD.

Proposal 4: For RA-RNTI calculation in IoT NTN TDD, H-SFN is considered and carrier id is not considered.

Proposal 4a: For RA-RNTI calculation in IoT NTN TDD, the NB-IoT TDD RA-RNTI calculation is used as baseline.

In legacy NB-IoT system, the minimal NPRACH periodicity is 40ms, so floor(SFN_id/4) is used for RA-RNTI calculation[8]. But in IoT NTN TDD, the minimal NPRACH periodicity has been changed to 90ms, theoretically, floor(SFN_id/9) should be used for RA-RNTI calculation, which can decrease the RA-RNTI number and save RNTI occupation.

Proposal 5: RAN2 discusses whether to use floor (SFN_id/4) or floor(SFN_id/9) in RA-RNTI calculation. If floor(SFN_id/9) is used, then RA-RNTI = 1 + floor(SFN_id/9) + 114*(H-SFN mod 2), otherwise RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2).

In TS 36.211[6], the NPRACH transmission occasion is defined as follows:

NPRACH transmission can start only time units after the start of a radio frame fulfilling .

And in TS 36.321[8], “the first radio frame of the specified PRACH” and “the first hyper frame of the specified PRACH” are used for the RA-RNTI, we understand “the first radio frame of the specified PRACH” and “the first hyper frame of the specified PRACH” is the first SFN and first H-SFN where the NPRACH is transmitted, the same mechanism can be used for IoT NTN TDD. Since the NPRACH transmission occasion is described in RAN1(e.g. TS 36.211), RAN2 can still use the “the first radio frame of the specified PRACH” and “the first hyper frame of the specified PRACH” for RA-RNTI calculation.

Proposal 6: Use the existing terminology “the first radio frame of the specified PRACH” and “the first hyper frame of the specified PRACH” for RA-RNTI calculation.

Issue 3: About SI scheduling impact

In RAN1#120 meeting[4], the following conclusion has been made:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

In the current specification[7], the SIs are scheduled by SIB1 based on the following time domain related parameters:

si-WindowLength-r13 ENUMERATED {ms160, ms320, ms480, ms640, ms960, ms1280, ms1600, spare1}, si-RadioFrameOffset-r13 INTEGER (1..15) OPTIONAL, -- Need OP SchedulingInfo-NB-r13::= SEQUENCE { si-Periodicity-r13 ENUMERATED {rf64, rf128, rf256, rf512, rf1024, rf2048, rf4096, spare}, si-RepetitionPattern-r13 ENUMERATED {every2ndRF, every4thRF, every8thRF, every16thRF}, ... }

Considering there is only 8 consecutive DL radio subframes in 90ms TDD structure, the si-RepetitionPattern with every2ndRF, every4thRF or every8thRF will lead the SI repetition useless(e.g. SI repetition in non-DL subframes), which need to be enhanced to include value range with multiple 90ms, e.g. with value range of every9thRF, every18thRF, every36thRF etc.

Proposal 7: Define value range with multiple 90ms for si-RepetitionPattern, e.g. with value range of every9thRF, every18thRF, every36thRF etc.

In RAN1#120 meeting[3], the following agreement has been made,

Agreement NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

In RAN1#120bis meeting[5], the following agreement has been made,

Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact.

Similar as MIB and SIB1 mechanism, SI transmission can be dropped in non-D NB-IoT subframes, and the SI-Window determination can follow the legacy NB-IoT mechanism.

Proposal 8: SI transmission can be dropped in non-D NB-IoT subframes, and the SI-Window determination can follow the legacy NB-IoT mechanism.

When supporting TDD for NB-IoT in Rel-15, si-WindowLength has not been extended. Considering that maximal value of 1600ms is long enough to successfully decode one SI message, the si-WindowLength does not need be extended.

Observation 6: si-WindowLength extension is not necessary for IoT NTN TDD.

In the current TS36. 331 specification[7], it is based on parameter configuration(e.g. NW implementation) to avoid SI-window overlap, the same mechanism can apply to IoT NTN TDD.

#### 5.2.1.2a Scheduling for NB-IoT ... The SI messages are transmitted within periodically occurring time domain windows (referred to as SI-windows) using scheduling information provided in SystemInformationBlockType1-NB. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI is transmitted. The length of the SI-window is common for all SI messages, and is configurable. ...

Proposal 9: It is up to NW implementation to avoid SI-window overlap.

Issue 4: About SC-PTM impact

In SC-PTM, several timers and periodicity are used. But considering that SC-PTM are transmitted on common channel, e.g. SC-MCCH and SC-MTCH, and the control information associated with transmission of SC-PTM are transmitted in SIB-20, which can be transmitted periodically, UE can drop the non-DL subframes for SC-PTM reception .

Proposal 10: UE drops the non-DL subframes for SC-PTM reception.

Issue 5: About implications of Kmac extension to IoT NTN and/or NR NTN

In NR/IoT NTN, Kmac is a cell specific parameter(e.g. sent by SIB), which is used also for UE to adjust the UP timers, e.g., DRX RTT timers, RAR window, contention resolution timer etc. If there are legacy UEs exists in the cell, and extended Kmac is broadcasted, then legacy UEs will still use the old Kmac values (e.g., smaller Kmac values) to adjust the start of the timers, which will lead to failed transmission/scheduling because of the misunderstanding between NW and UE on the timer status.

Observation 7: Kmac is used by the UE for UP timers adjustment, which is a cell specific value for idle/inactive UEs. The extended Kmac values cannot be used by legacy UEs, which could lead to misunderstanding between NW and UE on the timers status (RAR window, contentionResolutionTimer and etc.), thus lead to failed scheduling/transmission.

Based on above, it is proposed that Kmac extension value of 512ms to 1023ms need not apply to NR NTN and IoT NTN.

Proposal 11: Kmac extension value of 512ms to 1023ms does not apply to NR NTN and legacy IoT NTN.

Issue 6: PDCCH monitoring issue

RAN1 has made the following agreements in RAN1#120bis meeting[5]

Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc. Working assumption DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is not present in a NB-IoT NTN TDD cell. - NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes. - NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Since the PDCCH monitoring issues is more relevant to RAN1, RAN2 can wait for more progress for this issue in RAN1 to avoid duplicated work in RAN1 and RAN2..

Proposal 12: NPDCCH monitoring issue can wait for more progress in RAN1.

- Conclusion

The following observations and proposals are made:

Observation 1: In TS 36.321 specification, the length of timer in unit of PDCCH periods is defined as number of PDCCH-subframes.

Observation 2: In TS 36.321 specification, the PDCCH-subframe for NB-IoT is defined as all NB-IoT downlink subframes.

Observation 3: Based on the IoT NTN TDD work item, the PDCCH-subframe definition for NB-IoT UE also applies to IoT NTN TDD UE.

Observation 4: If IoT NTN TDD supports same coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions), some of the existing value range of Timer with unit of ms or seconds(e.g. T300, T301, T311,T-PollRetransmit) will be insufficient and need to be extended(e.g. extended to 4 times of legacy TDD value).

Observation 5: Based on the RAN1 agreement, the legacy coverage enhancement level (e.g. DL with maximal 2048 repetitions) can not be provided in IoT NTN TDD.

Observation 6: si-WindowLength extension is not necessary for IoT NTN TDD.

Observation 7: Kmac is used by the UE for UP timers adjustment, which is a cell specific value for idle/inactive UEs. The extended Kmac values cannot be used by legacy UEs, which could lead to misunderstanding between NW and UE on the timers status (RAR window, contentionResolutionTimer and etc.), thus lead to failed scheduling/transmission.

Proposal 1: For timer in unit of PDCCH periods, the definition in existing specs is clear enough, not need for further clarification.

Proposal 2: RAN2 assumes not extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD. This assumption can be revisited if RAN1 provides the exact overage enhancement level(e.g. the maximal repetitions number) that is supported in IoT NTN TDD system.

Proposal 3: For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for TDD (i.e., 20.48s) is used for IoT NTN TDD.

Proposal 4: For RA-RNTI calculation in IoT NTN TDD, H-SFN is considered and carrier id is not considered.

Proposal 4a: For RA-RNTI calculation in IoT NTN TDD, the NB-IoT TDD RA-RNTI calculation is used as baseline.

Proposal 5: RAN2 discusses whether to use floor (SFN_id/4) or floor(SFN_id/9) in RA-RNTI calculation. If floor(SFN_id/9) is used, then RA-RNTI = 1 + floor(SFN_id/9) + 114*(H-SFN mod 2), otherwise RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2).

Proposal 6: Use the existing terminology “the first radio frame of the specified PRACH” and “the first hyper frame of the specified PRACH” for RA-RNTI calculation.

Proposal 7: Define value range with multiple 90ms for si-RepetitionPattern, e.g. with value range of every9thRF, every18thRF, every36thRF etc.

Proposal 8: SI transmission can be dropped in non-D NB-IoT subframes, and the SI-Window determination can follow the legacy NB-IoT mechanism.

Proposal 9: It is up to NW implementation to avoid SI-window overlap.

Proposal 10: UE drops the non-DL subframes for SC-PTM reception.

Proposal 11: Kmac extension value of 512ms to 1023ms does not apply to NR NTN and legacy IoT NTN.

Proposal 12: NPDCCH monitoring issue can wait for more progress in RAN1.

- References

- RP-243293 Revised WID on introduction of IoT-NTN TDD mode, RAN#106

- Draft_Minutes_report_RAN1#119_v030

- Draft_Minutes_report_RAN1#120_v010

- R2-129bis NR-NTN-IoT-NTN (Sergio)_ 2025-04-10_1300

- Draft_Minutes_report_RAN1#120b_v010

- 3GPP TS 36.211, v18.0.1(2024-04), Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation(Release 18)

- 3GPP TS 36.331, V18.4.0 (2024-12), Evolved Universal Terrestrial Radio Access (E-UTRA);Radio Resource Control (RRC);Protocol specification(Release 18)

- 3GPP T3GPP TS 36.321 V18.3.0 (2024-09), Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification(Release 18)

- 3GPP TS 38.413 V18.4.0 (2024-12), NG-RAN; NG Application Protocol (NGAP)(Release 18)