3GPP TSG-RAN WG2 Meeting #130	R2-2504120

St. Julian's, Malta, May 19th – 23rd, 2025

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon, Nokia, NTT Docomo, OPPO, ZTE Corporation

Title:	 Discussion on the support of mTRP in event-triggered LTM

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18 and Rel-19. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report for R18 and event-triggered L1/2 measurement report for R19.

In a Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, thereby enhancing coverage, reliability, and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. In the Rel-18 mTRP scenario, each TRP handles both downlink and uplink transmissions for the UE, as shown in the figure below. For mTRP scenarios, the control of uplink and downlink operations is managed by the physical layer and MAC layer, based on the configuration provided by the RRC layer.

This contribution provides a comprehensive analysis on the co-existence of event-triggered L1 measurement reports and  mTRP. In the last meeting, co-existence of event-triggered LTM and mTRP was discussed and it was agreed that the coexistence of L1 event-triggered MR with the mTRP scenario will be revisited in the August meeting.

Agreements on L1 event-triggered MR 8. For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

2	Discussion

2.0	Why the co-existence of mTRP and event triggered LTM should be supported?

The mTRP was introduced in R16 under the background of R15 NR that only ideal backhaul between different access points are supported that this might not be realistic in all the cases, especially for the scenarios for distributed antennas that extend the coverage of the cell. While for event-triggered LTM is also related to service continuity of the UE that addressed the related issue with mTRP.

Observation: Co-existence of mTRP and event-triggered measurement report should be supported since they are both related to the coverage issues of the UE.

Proposal0: Confirm that the co-existence between event-triggered measurement report and mTRP is supported.

2.1	Impact on LTM event evaluation

Agreements on measurement enhancements for LTM: 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Agreements on L1 MR event evaluation 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

According to the agreements made in previous meetings, the current beam is defined as the beam corresponding to the indicated TCI state. If the serving cell is configured with two TRPs, there will be two indicated TCI states, and as a result, the UE should have two current beams. During the event evaluation procedure, for LTM events involving the serving cell the UE needs to evaluate the current beam for LTM2, LTM3, and LTM5. The multiple TRP configuration of the serving cell should impact the evaluation of these events.

In events LTM2, LTM3, and LTM5, if the serving cell has a two-TRP configuration, there will be two current beams in the event evaluation procedure. We need to discuss which current beam should be used in the event evaluation or which beam can be used to determine that whether the entering condition is satisfied. Obviously, there can be two options to down-select:

- One option is to use both current beams in the event evaluation procedure. The UE should measure the RSRP of both current beams and evaluate them separately to see if they meet the entering condition. In this case, the definition of the beam for the serving cell should be clarified, specifying that if the serving cell is configured with two TRPs, both current beams should be considered as the beam of the serving cell.

- Another option is to use the best beam of the serving cell in the event evaluation procedure. This means that the UE should measure the RSRP of both current beams and choose the best one for the event evaluation. This option is simpler than the previous one, as it requires the use of only one current beam during the event evaluation, which aligns more closely with the previous agreement for scenarios without a multiple TRP configuration.

Based on the analysis above, we think that the simplest solution can be that we only use the best beam of the two current beams when mTRP is configured for the serving cell for event-triggered LTM evaluation.

Proposal 1: When mTRP is configured for the serving cell, UE should use the best beam of the two current beams for event evaluation for LTM2/3/5.

If the candidate cell has a multiple TRP configuration, the UE can initially access one of the TRPs during the LTM cell switch and then access the other through beam management after the LTM cell switch. Therefore, there is no need to consider this during the event evaluation procedure.

Observation: There is no need to consider mTRP operations for candidate cell for event-triggered LTM.

2.2	Impact on MR MAC CE

Agreements on L1 event triggered MR 14. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration.

In addition to the event evaluation procedure, it is also crucial to consider how the multiple TRP configuration of the serving cell should impact the event reporting procedure. It has been agreed that the information regarding the current beam and its quantity should be included in the MR MAC CE. In cases where the serving cell is configured with two TRPs, the UE will have two current beams and it needs to be further discussed how to report the beam measurement for the serving cell.

- One approach is to include both the two current beams in the MR MAC CE. This would allow the network to assess the quality of both beams and provide a comprehensive overview of the two indicated TCI states of the UE. As a result, the network would be able to make a more stable decision based on the measurement results of both beams.

- An alternative approach is to report only the best current beam in the MR MAC CE. In this case, the UE would compare the measurement results of both current beams and select the one with the highest quality for inclusion in the MR MAC CE. This method requires an additional byte in the MR MAC CE, which would increase the overhead for uplink transmission. However, this approach is more aligned with the legacy MR MAC CE, which does not require as many bits as the previous approach. The downside, however, is that the network would not receive the beam quality of the current beam with the lower RSRP value.

The latter approach appears to be more advantageous, that (a) there is no impact for the currently agreed format for MR report MAC CE (b) it is enough that the NW knows the quality of the best beam for the serving cell. While in order to differentiate between the two beams, a beam index for the measurement report might be required. It can be further discussed whether beam index for the current beam is reported as well.

Proposal 2: When mTRP is configured for the serving cell, only the best current beam is reported in the MR MAC CE. FFS whether beam index for the current beam is reported.

# 3	Conclusion

In this paper, we have discussed how event-triggering LTM reports coexist with the multiple TRP configuration. The following proposals are presented:

Proposal 0: Confirm that the co-existence between event-triggered measurement report and mTRP is supported.

Proposal 1: When mTRP is configured for the serving cell, UE should use the best beam of the two current beams for event evaluation for LTM2/3/5.

Proposal 2: When mTRP is configured for the serving cell, only the best current beam is reported in the MR MAC CE. FFS whether beam index for the current beam is reported.

# 4	Reference

- RAN2 #128 chairman notes;

- RAN2 #129-bis chairman notes;