3GPP TSG-RAN WG2 Meeting #130	R2-2504133

St. Julians, Malta, 19th - 23rd of May 2025

Agenda item:	7.0.2.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Further Thoughts on IDC issue between GNSS and NTN UL

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#129 (February 2025) it was discussed how to ensure GNSS reception and UL NTN transmission in the band adjacent to GNSS can coexist [1]. As a result, the LS to RAN4 has been sent [2]. In this paper we outline the next steps, considering the unlikely (or very distant) response from RAN4.

# 2	Discussion

## 2.1	Background Information (what happened in the previous RAN2 and RAN4 meetings)

At RAN2#129 RAN2 has intensely discussed the IDC issue and concluded the following [1]:

- RAN2 will not work on enhancements to the existing IDC TDM solution or IDC autonomous denial solution for Rel-18 - We send a reply LS to RAN4 saying that RAN2 will not work on a solution for Rel-18 but can consider this for Rel-19. RAN2 would like to receive requirements from RAN4 on periodicity of IDC free time to fix GNSS

And the aforementioned LS to RAN4 contained the following action to RAN4 [2]:

RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g. the periodicity and duration of IDC free time to help the UE read the GNSS.

As can be seen above, RAN2 made an attempt to understand what kind of periodicity and duration of IDC free time could be required for the UE to measure GNSS. RAN2 could not conclude, so decided to re-involve RAN4 in this discussion.

Observation 1: RAN2 attempted to understand what kind of periodicity and duration of IDC free time could be required for the UE to measure GNSS.

Observation 2: RAN2 could not conclude without consulting other WGs, so sent the LS to RAN4, asking this working group on the periodicity and duration of IDC free time to help the UE read the GNSS.

At RAN4#114bis the LS from RAN2 has been discussed. However, RAN4 could not reach a consensus and indicate solid requirements. Thus, the topic might be continued and finalized at RAN4#115 (May 2025), but equally probable is also that RAN4 will never provide solid numbers in response to RAN2 LS [2].

Observation 3: The LS from RAN2 has been discussed at RAN4#114bis, but without any consensus regarding the requirements to be indicated to RAN2.

Observation 4: It might turn out, RAN2 needs to proceed without the feedback from RAN4.

## 2.2 	Existing solutions and way forward

As concluded in RAN2#129, we have decided to consider only the following approaches:

- IDC TDM solution or

- IDC autonomous denial solution

The first of those listed solutions is defined in NR RRC (ASN.1) as follows [3]:

IDC-TDM-Assistance-r18 ::= SEQUENCE { cycleLength-r18 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240}, startOffset-r18 INTEGER (0..10239), slotOffset-r18 INTEGER (0..31), activeDuration-r18 CHOICE { subMilliSeconds-r18 INTEGER (1..31), milliSeconds-r18 ENUMERATED { ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } }, ... }

whereas the second of the considered solutions is defined in NR RRC (ASN.1) as follows [3]:

AutonomousDenialParameters-r18 ::= SEQUENCE { autonomousDenialSlots-r18 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r18 ENUMERATED {n200, n500, n1000, n2000} }

As can be seen above, the maximum length of activeDuration in IDC-TDM approach is 1600 ms within a maximum cycle length of 10240 ms. We acknowledge the points raised by some companies (e.g. in [4]) that such value of activeDuration might be insufficient to acquire GNSS information. On the other hand, if multiple UEs request such IDC mitigation scheme and maximum values are used then 15% of UL transmissions are allowed to be avoided. This brings a serious complexity to NW’s scheduler.

Observation 5: When multiple UEs use IDC-TDM scheme with maximum values of activeDuration and cycle length, it results in ~15% of UL transmissions that might be avoided. This brings a serious complexity to NW’s scheduler.

The Autonomous Denial solution has significantly lower available values when it comes to the number of slots that can be denied (i.e. up to 30 slots). There are two spare values in autonomousDenialSlots which could be tempting for introducing larger values (as proposed in [4]). However, we prefer to avoid extending the solution where the NW has no awareness which slots will be denied, and the decision is left to the UE.

Proposal 1: Do not consider an extension of autonomousDenialSlots which would increase the number of slots the UE can autonomously deny and bring additional complexity to UL scheduling.

If companies in RAN2 are convinced a specification change to address the IDC issue is needed, then we suggest to work on extending the length of the activeDuration in IDC-TDM solution.

Proposal 2: If a specification change to address the IDC issue is needed, consider extending the length of the activeDuration in IDC-TDM solution.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: RAN2 attempted to understand what kind of periodicity and duration of IDC free time could be required for the UE to measure GNSS.

Observation 2: RAN2 could not conclude without consulting other WGs, so sent the LS to RAN4, asking this group on the periodicity and duration of IDC free time to help the UE read the GNSS.

Observation 3: The LS from RAN2 has been discussed at RAN4#114bis, but without any consensus regarding the requirements to be indicated to RAN2.

Observation 4: It might turn out, RAN2 needs to proceed without the feedback from RAN4.

Observation 5: When multiple UEs use IDC-TDM scheme with maximum values of activeDuration and cycle length, it results in 15% of UL transmissions that might be avoided. This brings a serious complexity to NW’s scheduler.

Proposal 1: Do not consider an extension of autonomousDenialSlots which would increase the number of slots the UE can autonomously deny and bring additional complexity to UL scheduling.

Proposal 2: If a specification change to address the IDC issue is needed, consider extending the length of the activeDuration in IDC-TDM solution.

# References

- R2-2501333 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb. 17th – 21st, 2025

- R2-2501568 Reply LS simultaneous operation between GNSS and NR NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb. 17th – 21st, 2025

- 3GPP TS 38.331 NR Radio Resource Control (RRC); Protocol specification, version 18.5.1, March 2025

- R2-2500537 Discussion on RAN4 LS on IDC issue in NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb. 17th – 21st, 2025