3GPP TSG-RAN WG2 Meeting #130	Tdoc R2-2504188

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	8.20.1

Source:	Ericsson

Title:	Simultaneous Tx-Rx capability for TDD-SDL and TDD-FDD

Document for:	Discussion, Decision

# 1	Introduction

RAN4 has sent an LS to RAN2 (R4-2505123) to capture aspects related to TDD-SDL and TDD-FDD for simultaneous Rx-Tx for inter-band CA. This contribution discusses the RAN2 impacts and further aspects from the related capabilities.

# 2	Discussion

The LS informed the following RAN4 agreement:

- To clarify the handling of simultaneous Rx-Tx interaction between network and UE for band combinations with different types of duplex mode - For TDD-SDL band combination, simultaneous Rx-Tx is mandatory with signalling. Consider to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD.

As mentioned in the agreement, simultaneousRxTxInterBandCA could be reused also for TDD-SDL CA case. We think a note should be sufficient to be added in the field description of simultaneousRxTxInterBandCA to indicate that this capability is also applicable to TDD-SDL in the same way as for TDD-FDD, since for SDL also no TDD pattern is applicable so it could be considered similar to FDD.

simultaneousRxTxInterBandCA

Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4].

This capability does not apply to the following components within TDD-TDD and TDD-FDD inter-band NR-CA or NR-DC combinations:

-	Intra-band NR-CA or NR-DC component

-	Inter-band NR-CA or NR-DC component where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band (as specified in TS 38.101-1 [2]).

Note: TDD-FDD also includes the case of TDD-SDL.

Since the RAN4 decision is from Rel-19, the change should also be applicable starting from Rel-19.

A note is added to simultaneousRxTxInterBandCA (from Rel-19) to clarify that TDD-FDD also includes the case of TDD-SDL.

Similarly, SDL information is missing from simultaneousRxTxInterBandCAPerBandPair. But in the case of this field, it should be noted that the UE (when including this field) should provide the band pairs for all bands in the band combination (as highlighted below in yellow). Therefore, this capability should be more generic and not refer to any particular band pair, since all of them should be indicated and not only TDD-TDD and TDD-FDD pairs.

simultaneousRxTxInterBandCAPerBandPair

Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA for each band pair in the band combination.

Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1.

If this field is included in ca-ParametersNR-ForDC, each bit of this field indicates whether the UE supports simultaneous transmission and reception between each band pair, within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC.

The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair.

Therefore, it should be discussed how to update simultaneousRxTxInterBandCAPerBandPair to be applicable to all band pairs, which may also affect the text in simultaneousRxTxInterBandCA, so further discussion is needed.

Discuss how to update simultaneousRxTxInterBandCAPerBandPair to be applicable to all band pairs and if any update is also needed for simultaneousRxTxInterBandCA.

RAN4 has also clarified the following for FR1+FR1 FDD-TDD CA:

Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD CA combinations.

In our understanding, there is no impact of the agreement above in RAN2 since this mandatory aspect is already captured in RAN4.

No impact in RAN2 from RAN4 agreements on simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD CA combinations.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	No impact in RAN2 from RAN4 agreements on simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD CA combinations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A note is added to simultaneousRxTxInterBandCA (from Rel-19) to clarify that TDD-FDD also includes the case of TDD-SDL.

Proposal 2	Discuss how to update simultaneousRxTxInterBandCAPerBandPair to be applicable to all band pairs and if any update is also needed for simultaneousRxTxInterBandCA.