3GPP TSG-RAN WG2 Meeting #130	R2-2504236

St Julian's, Malta, 19 - 23 May, 2025

Title: 	Discussion on MRO for mobility

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2

Document for:	Discussion and Decision

- Introduction

In this paper, we firstly address open issues for running 38.331 CR and running 36.331 CR, and then we analyze possible RAN2 impacts regarding MRO for LTM and CHO with candidate SCGs.

- Discussion

Open issues in running 38.331 CR and 36.331 CR

During the post-meeting email discussion [1][2], some open issues have been identified and they are to be discussed at RAN2#130.

For running 38.331 CR, there is one open issue:

Proposal 1. Companies are invited to discuss the Open issue RRC-1 (i.e., the correlation of SHR and SPR and avoiding duplication of the information) in their contribution for the meeting RAN2#130.

For this open issue, we understand that possible redundant measurements are mainly about neighbour cell measurements, which may need to be optimized from UE point of view.

We also realize that there are pros/cons for this direction:

pros: reduce UE memory requirement

cons: specify more UE behaviours, e.g. UE needs to check the duplicated parts and then filter the measurements in reports. NW behaviours need to be enhanced as well

We observe that only three companies provided their comments to the email discussion, so we understand that more companies are not so interested in this direction. In other words, there may not be much interests on solving this open issue. In addition, so far the solution details are unclear, and more UE behaviours may lead to more complexities on UE side. In this case, we are also fine to not do anything about it.

For running 36.331 CR, there are three open issues:

Proposal 1: It is proposed RAN2 to discuss the open issue LTERRC-1, i.e. whether to keep the following condition or not (it has been captured in section 5.6.13a.3 in running 36.331 CR):

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

Proposal 2: It is proposed RAN2 to discuss the open issue LTERRC-2, i.e. when the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure and this should be captured in running 36.331 CR.

Proposal 3: It is proposed RAN2 to discuss the open issue LTERRC-3, i.e. whether to have a single container to carry all the fields (PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800) or not.

For open issue LTERRC-1, we tend to agree with Samsung's view, i.e. the condition "RRCReconfiguration message including the reconfigurationWithSync for the SCG" will always happen before SCG failure happens. And then we are ok to remove the condition.

For open issue LTERRC-2, we note that the following CRs were agreed in principle at RAN2#129bis, which are to clarify the IE timeUntilReconnection in RLF report.

R2-2503116 	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-16	36.331	16.X.Y	5113	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2503102	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-17	36.331	17.12.0	5108	1	F	NR_ENDC_SON_MDT_enh-Core

R2-2503103	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-18	36.331	18.5.0	5109	1	A	NR_ENDC_SON_MDT_enh-Core

The above three are in-principle agreed

Similarly, if the UE supports RLF-Report for LTM, there could be similar changes to TS 36.331, e.g.:

5>	if the UE supports RLF-Report for LTM as defined in TS 38.306 [87] and if LTMCellId in VarRLF-Report of TS 38.331 [82] is set:

6>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the radio link failure or handover failure experienced in the failedPCellID stored in VarRLF-Report of TS 38.331 [82];

5>	else:

6>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the last radio link failure or handover failure;

For open issue LTERRC-3, we think that separate containers should be acceptable and workable. If we just use one container for carrying three different IEs from TS 38.331, it is unclear how it works, and how to reflect the presence of each IE.

Proposal 1: (LTERRC-1) It is proposed to remove the condition below:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

Proposal 2: (LTERRC-2) When the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure.

Proposal 3: (LTERRC-3) It is proposed to have separate containers for all the fields i.e. PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800.

Other issues on MRO for LTM

RAN2#129bis meeting has made the following agreements on MRO for LTM:

Agreements We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases. Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

To facilitate the further discussion, we illustrate the steps during the LTM procedure to have a retro on the design of Rel-18 LTM feature, compared to those during legacy L3 handover.

Figure 1: Handover procedures for Legacy L3 HO and LTM

The division of failure cases for LTM is similar as those for handover, since the LTM is built and augmented on top of L3 handover. For handovers, the SON features including the radio link failure (RLF) report and successful handover report (SHR) provides the UE feedback information. The RLF and SHR could be augmented to also provide useful feedback information to address the LTM mobility issues.

We will discuss the possible MRO enhancement for those SON reports.

Early sync info in RLF report

Early sync is the key to achieve the goal of reducing the interruption time. If it is RACH-less, RAN2 discuss whether to include early sync attempts towards multiple LTM candidate Cell in one RA report to identify the issues of network-based TA procedure, e.g., how frequent and which is the proper triggering timing the source DU should trigger the PDCCH order to maintain the TA validity, and which of candidate cells should be involved to acquire the TA so as to have a target cell with a valid TA.

As shown in Figure 1, for possible LTM procedure-1 (RACH-less access based on network TA), the time period between the reception of the PDCCH order and the reception of HO command MAC CE can be logged at UE and reported to NW, and then NW can optimize early UL sync procedure. For example, if the time period is large, the estimated TA may not be good enough when performing LTM cell switch, and NW can take it into account.

In summary, we suggest that RAN2 consider to enhance the early sync info to RLF report.

Proposal 4: For RACH-less access based on network TA, include the following information in RLF report:

- Time period between the reception of the PDCCH order and the reception of HO command MAC CE related to the failed LTM cell switch

TCI state/Beam Info

We notice that only the existence of L1 measurement is not sufficient. L1 measurement is the result of the UE movement at the time of failure occurrence.  The beam indicated with a TCI state included in the CSC is also a determining factor impacting whether the RA-less access succeeds or not, i.e., the first UL transmission can be received on the indicated beam before T304 expires or not. It is suggested that UE logs the beam info in the cell switch command, for the network to know how the UE movement affects the beam selection of the target cell. Combining this beam info with the L1 measurement result, and the time gap since the CSC till the failure, the network could perform analysis to optimize the choice of target beam.

Proposal 5: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Other issues on MRO for CHO with candidate SCGs

RAN2#129bis meeting has made the following agreements on MRO for CHO with candidate SCGs:

Agreements No new triggering conditions such as time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure. No new triggering conditions for SHR/SPR procedure for CHO with candidate SCG. UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs. UE includes the target PCell ID in SPR for successful CHO with candidate SCG. In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

In previous meetings, the joint configuration scenario of CHO-only and CHO with candidate SCGs has been mentioned. For a given candidate PCell, CHO-only is definitely more easily fulfilled than CHO with candidate SCGs. That is also the reason why CHO-only is used as a supplement to CHO with candidate SCGs. If CHO-only is fulfilled and CHO with candidate SCG is not, the legacy MRO for CHO is reused for triggered CHO-only in terms of RLF and SHR report, i.e. no special handling for CHO-only.

Proposal 6: It is proposed RAN2 to agree that the legacy MRO for CHO is reused for triggered CHO-only in terms of RLF and SHR report, i.e. no special handling for CHO-only.

- Conclusion

Based on the discussions above, we have the following proposals:

Proposal 1: (LTERRC-1) It is proposed to remove the condition below:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

Proposal 2: (LTERRC-2) When the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure.

Proposal 3: (LTERRC-3) It is proposed to have separate containers for all the fields i.e. PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800.

Proposal 4: For RACH-less access based on network TA, include the following information in RLF report:

- Time period between the reception of the PDCCH order and the reception of HO command MAC CE related to the failed LTM cell switch

Proposal 5: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Proposal 6: It is proposed RAN2 to agree that the legacy MRO for CHO is reused for triggered CHO-only in terms of RLF and SHR report, i.e. no special handling for CHO-only.

- Reference

[1] [Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)

[2] [Post129bis][604][SONMDT] Running LTE RRC CR for SONMDT (Huawei)