3GPP TSG RAN WG2 Meeting #130                          	   R2-2504242

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SSB SCell Operation

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives of on-demand SSB SCell are outlined with the following [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss the design details of OD-SSB transmission MAC CE and L3 RRM measurement for OD-SSB Case #2.

# Discussion

## MAC CE for On-demand SSB transmission

At RAN1 #120bis, the following agreements were made for OD-SSB MAC CE [4].

Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE]. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

At RAN2 #129bis, the following agreements were made for OD-SSB MAC CE [5].

Agreement 1. For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). 2. A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. 3. For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. 4. OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. 5. RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

Based on the above RAN1’s and RAN2’s agreements on OD-SSB transmission MAC CE, we conclude the following observations.

Observation 1. The MAC CE for OD-SSB transmission contains an N value for implicit deactivation of an OD-SSB transmission if at least one value of N is configured by RRC.

Observation 2. The MAC CE for OD-SSB transmission contains the SSB position of an OD-SSB for both Case #1 (without always-on SSB) and for Case #2 if the always-on SSB and OD-SSB have different center frequencies.

Observation 3. The MAC CE for OD-SSB transmission updates the transmission parameters of a transmitted OD-SSB for both Case #1 and Case #2 after an Scell is activated. No new field of the MAC CE is needed for such an update indication.

## L3 measurement for OD-SSB Case #2

At RAN1 #120, the following agreement was made for Case #2 when the always-on SSB (AO-SSB) and OD-SSB have different center frequencies [2].

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SSB

Observation 4. For Case #2, it’s UE implementation to measure OA-SSB or OD-SSB when AO-SSB and OD-SSB have different center frequencies.

At RAN2 #129, the following agreement was made for Case #2 [3]. However, this agreement was not included in the LS to RAN4.

Agreements on OD-SSB RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

At RAN2 #129bis, the following working assumption was made for Case #2 [5], and an LS was sent to RAN4.

Based on the observation and agreement, we can wait for RAN4’s feedback before deciding to confirm the working assumption.

Proposal 1. RAN2 waits for RAN4’s response before RAN2 decides to confirm the following working assumption:

When A-SSB and OD-SSB have different center frequencies, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

It was brought up during online discussion at RAN2 #129bis, if multiple OD-SSBs with different frequencies are supported for an Scell. In our view, supporting multiple OD-SSBs with different frequencies in a Scell complicates UE’s behavior without any benefit for the UE. Therefore, for a UE, only an OD-SSB with a frequency is supported in an Scell.

Proposal 2. From a UE’s aspect, only one OD-SSB is supported when AO-SSB and OD-SSB have different center frequencies in an Scell.

# Conclusion

In this contribution, we discussed the design details of OD-SSB transmission MAC CE and L3 RRM measurement for OD-SSB Case #2 and concluded with the following observations and proposals.

MAC CE for On-demand SSB transmission

Observation 1. The MAC CE for OD-SSB transmission contains an N value for implicit deactivation of an OD-SSB transmission if at least one value of N is configured by RRC.

Observation 2. The MAC CE for OD-SSB transmission contains the SSB position of an OD-SSB for both Case #1 (without always-on SSB) and for Case #2 if the always-on SSB and OD-SSB have different center frequencies.

Observation 3. The MAC CE for OD-SSB transmission updates the transmission parameters of a transmitted OD-SSB for both Case #1 and Case #2 after an Scell is activated. No new field of the MAC CE is needed for such an update indication.

L3 measurement for OD-SSB Case #1

Observation 4. For Case #2, it’s UE implementation to measure OA-SSB or OD-SSB when AO-SSB and OD-SSB have different center frequencies.

Proposal 1. RAN2 waits for RAN4’s response before RAN2 decides to confirm the following working assumption:

When A-SSB and OD-SSB have different center frequencies, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Proposal 2. From a UE’s aspect, only one OD-SSB is supported when AO-SSB and OD-SSB have different center frequencies.

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- “Chair notes RAN1#120 eom0”, RAN1 #120, Orlando, US, November 18th – 22nd, 2024

- “R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129, Athens, Greece, February 17th – 21st, 2025

- “Chair notes RAN1#120bis eom0”, RAN1 #120bis, Wuhan, China, Apr. 7th – 11th, 2025

- “R2_129bis_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129bis, Wuhan, China, Apr. 7th – 11th, 2025