3GPP TSG RAN WG2 Meeting #130                          	   R2-2504243

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.3

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

At RAN #105, it was agreed that on-demand SIB1 is specified for Case #2. The updated objects are outlined with the follows [1].

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective. - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1. - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE. - RAN1 specification impact to support this feature should be minimized

In this contribution, we further discuss design details of UL-WUS configuration for on-demand SIB1.

# Discussion

At RAN1 #120bis, the following agreements were made for UL-WUS configuration [2].

Agreement The following agreement is updated as follows: The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. - Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions - Alt1: The information is provided in UL-WUS config - Alt2: The information is provided in RAR in response to UL-WUS transmission - Alt3: No additional information is provided Agreement Indication of K_SSB value in the UL-WUS configuration should be 5 bits for FR1 and 4 bits for FR2. - Note: there is no change to current PBCH structure. - UE does not expect K_SSB to be larger than 23 for FR1 and larger than 11 for FR2 Agreement From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings: - PhysCellId and ARFCN-ValueNR are mandatory - frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification) - K_SSB is mandatory - searchSpaceZero and controlResourceSetZero are mandatory - ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory Agreement The parameter ra-SearchSpace in the UL-WUS configuration is of type SearchSpace. Agreement Include CarrierBandwidth and locationAndBandwidth in the UL-WUS configuration. - Previous RAN1 #120 conclusion is reverted Agreement - RAN1 clarifies frequencyBandList in UL WUS configuration refers to the IE within FrequencyInfoUL-SIB.

Based on the above RAN1 agreements, some UL-WUS parameters can be NES cell specific which make it hard to share UL-WUS configuration among NES cells.

Observation 1. Some UL-WUS parameters in an UL-WUS configuration can be cell specific which may limit the option of sharing an UL-WUS configuration among multiple NES cells.

At RAN2 #129bis, the following agreement was made for UL-WUS configuration [3].

Agreements on OD-SIB1 6. The UL WUS configuration includes the IE totalNumberOfRA-Preambles. 10. WUS configuration can be associated with a list of cells if the whole WUS configuration is same. 11. Area id will be used as legacy.

During RAN2 #129bis, access related information in UL-WUS configuration was discussed online and offline with the following parameters for consideration:

- plmn-IdentityList	(list of PLMN IDs each can be 3 digits for MCC and 2-3 digits for MNC)

- trackingAreaCode		(24 bits)

- cellReservedForOperatorUse	(1 bit)

- cellReservedForOtherUse	(1 bit)

- cellReservedForFutureUse	(1 bit)

- cellBarredATG		(1 bit)

- cellBarred-eRedCap1Rx	(1 bit)

- Barred-eRedCap2Rx		(1 bit)

- cellBarredNES		(1 bit)

- cellBarredNTN		(1 bit)

- cellBarred-RedCap1Rx	(1 bit)

- cellBarred-RedCap2Rx	(1 bit)

The size of plmn-IdentityList and trackingAreaCode can be significant compared with the other barring parameters. With plmn-IdentityList and trackingAreaCode in UL-WUS configuration, the SIBx containing UL-WUS configurations for multiple NES cells can be too large for some UEs.

Observation 2. plmn-IdentityList and trackingAreaCode contain large number of bits (especially plmn-IdentityList) which may increase UL-WUS configuration significantly.

Based on the observations, we propose the following proposals.

Proposal 1. RAN2 supports the following parameters to be included in UL-WUS configuration.

- cellReservedForOtherUse

- cellReservedForFutureUse

- cellBarredATG

- cellBarred-eRedCap1Rx

- Barred-eRedCap2Rx

- cellBarredNES

- cellBarredNTN

- cellBarred-RedCap1Rx

- cellBarred-RedCap2Rx

Proposal 2. It is network implementation to include the following parameters in UL-WUS configuration.

- plmn-IdentityList

- trackingAreaCode

# Conclusion

In this contribution, we discussed design details of UL-WUS configuration for on-demand SIB1 observations and proposals.

Observation 1. Some UL-WUS parameters in an UL-WUS configuration can be cell specific which may limit the option of sharing an UL-WUS configuration among multiple NES cells.

Observation 2. plmn-IdentityList and trackingAreaCode contain large number of bits (especially plmn-IdentityList) which may increase UL-WUS configuration significantly.

Proposal 1. RAN2 supports the following parameters to be included in UL-WUS configuration.

- cellReservedForOtherUse

- cellReservedForFutureUse

- cellBarredATG

- cellBarred-eRedCap1Rx

- Barred-eRedCap2Rx

- cellBarredNES

- cellBarredNTN

- cellBarred-RedCap1Rx

- cellBarred-RedCap2Rx

Proposal 2. It is network implementation to include the following parameters in UL-WUS configuration.

- plmn-IdentityList

- trackingAreaCode

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- “Chair notes RAN1#120bis eom0”, RAN1 #120bis, Wuhan, China, Apr. 7th – 11th, 2025

- “R2_129bis_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129bis, Wuhan, China, Apr. 7th – 11th, 2025