# 3GPP TSG-RAN2 Meeting #130	R2-2504260

# Malta, May 19-23, 2025

Agenda Item:	8.18.2

Work Item:	LTE_terr_bcast_Ph2-Core

Source: 	Qualcomm Incorporated, EBU

Title: 	RAN2 impacts to support time-frequency interleavers for LTE-based 5G Broadcast Phase 2

Document for:	Discussion/Decision

# Introduction

RAN-approved WI description for LTE-based 5G Broadcast Phase 2 is given in [1]. The objective is to specify time-frequency interleavers for MBMS-dedicated cells. RAN1 has made several agreements and now RAN2 should discuss about the signalling impacts to support the feature.

# Background

## Overview

The purpose time-interleaving is to interleave M Transport Blocks (TBs) together, in a uniform pattern, where each of these interleaved TBs (having their native TBS scaled by N) span N equispaced (by M subframes) non-contiguous subframes. This is depicted in Fig. 1, where the subframe mapping for TBi is highlighted in red. As a result, two parameters N and M are the main parameters forming the M×N subframes basic unit of time interleaving. Consequently, the two parameters (N and M) determine a host of associated quantities that are pertinent to the specification of time interleaving.

Figure 1: Subframe mapping of TBs for time interleaved PMCH transmission.

The above is reflected by the following RAN1#120 agreement:

Agreement The basic transmission pattern of PMCH with time interleaving consists of a set of (M x N) consecutive MBSFN subframes, excluding MCCH and MSI, where two transmissions of the same TB are separated by (M-1) MBSFN subframes, excluding MCCH and MSI.

Further, RAN1#120b agreed the following:

Agreement Support at least the following values of time interleaver depth: - Values M={4,8,16,32} and N={1,2,4,8,16} are supported. - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI. - FFS: larger values, including M=64, N={32,64}.

## Scheduling

As described in TS 36.321, the MSI MAC-CE is used to schedule the MTCHs for a set of sessions configured by the higher layer parameter MBMS-SessionInfoList, provided in TS 36.331.

The MSI MAC-CE is depicted in Section 6.1.3.7 in TS 36.321, which (in accordance with Section 5.12 on MCH reception) indicate that the MTCHs corresponding to the above-mentioned sessions are scheduled consecutively (one immediately after the other), immediately following the MSI subframe, within a MCH scheduling period, defined by the configured periodicity of the MSI. This means the MTCH subframes corresponding to each session do not have any overlap.

This (existing) scheduling mechanism above, for the MTCHs corresponding to different sessions, naturally lends itself to certain scheduling decisions when MTCHs use time-interleaved subframe mapping.

RAN1#120 discussed the above and agreed the following:

Agreement Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH.

The above agreement means the time-interleaved MTCH transmission is to be contained “within a session”—i.e., there will be no “across-sessions” interleaving. This design choice simplifies the MSI-based scheduling, in the presence of different types of services coexisting (e.g., services with different interleaving parameters M, N, as well as services without time-interleaving). Further, it is beneficial to retain the consecutive scheduling by sessions, without any overlap across sessions, as is currently done by the MSI.

Agreement Regarding configuration for time interleaving transmission, the following options can be considered: - Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config - Option 2: Per PMCH configuration, e.g., via pmch-Config - Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Furthermore, RAN1#120b updated the above agreement as follows:

Agreement The agreement in the previous meeting is updated as follows: Regarding configuration for time interleaving transmission, the following options can be considered: - Option 1: Per PMCH configuration, e.g., via pmch-Config - Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE - Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching. - Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

RAN1 still needs to down-select among the above two remaining options. So, we propose the following:

Wait for RAN1 for the down-selection regarding the configuration for time interleaving transmission.

## MCH scheduling periodicity

Above section discussed how the MSI may be used to schedule session(s) with time-interleaving. RAN1 also examined potential issues with how the current MSI is specified, and whether it can seamlessly accommodate the scheduling of services with time-interleaving. This is described in detail in [2] and [3]. In a nutshell, it can be observed that the MSI periodicity (as configured by mch-SchedulingPeriod) may not be divisible (M x N) units. As an example, N=4 and M=16. The MTCHs from each session would thus span 16×4=64 ms (as a minimum). Per the existing specified values of MSI periodicity, the values immediately proximal to 64 ms is 80ms. As a result, this configuration of time-interleaved sessions cannot be perfectly fit within a current MSI periodicity and leads to 18% resource wastage.

RAN1#120b discussed the above issues made the following agreements:

Agreement In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern: - Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations. - May include the case of scheduling different M/N within the same MSI periodicity - FFS the values of the new periodicities. - RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity - Option 2: Drop the time-interleaved TBs which could not be successfully decoded. - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour) - Other Options not precluded

Therefore, we propose to wait for RAN1 to discuss the above further and make progress.

Wait for RAN1 for the candidate values of the new MCH scheduling period(s).

# Signalling Impacts

## Configuration for time interleaving

Based on the discussion above and the RAN1 agreements so far, we identify the following signalling impacts. Note that formal list of RRC parameters were not included in the RAN1 LS to RAN2, however for quick reference, the draft of the RAN1 parameters can be found in [4] from the WI rapporteur.

### Signalling of M and N

A field to indicate TBS scaling factor N needs to be signalled. As shown in section 2.1, some values are already agreed by RAN1 while other values are still FFS. However, the value of N = 1 can be implicit on the presence of the parent IE that contains this field (as described later). Therefore, we propose:

Introduce a field to indicate TBS scaling factor (i.e. N) with candidate values {2,4,8,16,ffs}.

Similarly, a field to indicate the separation between two transmissions of the same TB (i.e. M) needs to be signalled to the UE. We propose to introduce a field for this with the candidate values as agreed by RAN1 so far.

Introduce a field to indicate the separation between two transmissions of the same TB (i.e. M) with candidate values {4,8,16,32,ffs}.

In addition, to indicate whether time interleaving is enabled or not, the signalling can be implicit upon the presence of the parent IE that contains the fields for N and M.

Presence of the parent IE containing fields for N and M indicates time interleaving is enabled.

## Configuration for frequency interleaving

Based on the RAN1 discussion so far, it seems there are no specific parameters to be signalled other than a flag indicating whether frequency interleaving is enabled or not.

Introduce a flag to indicate whether frequency interleaving for MTCH is enabled.

# UE capability signalling

According to the LS from RAN1 [5], there will be a few UE capabilities that needs to be defined. However, since RAN1 has not finalized the details yet, capabilities can be discussed later (or as part of the mega CR depending on the final RAN1 LS).

Postpone the discussion about UE capabilities to next meeting.

# Example ASN.1 signalling

An example ASN.1 signalling is shown below. The exact details on where these fields will be included can be further discussed based on RAN1 progress related to the down-selection of options (see section 2.1 and proposal 1 above). Additionally, the exact structure, field names, candidate values, etc. can be further discussed based on RAN1 and RAN2 progress. The below is only shown to illustrate example of potential impact to LTE RRC specification (TS 36.331).

ParentIE-TBD ::=	SEQUENCE { -- which IE is used to include the new configuration is TBD pending RAN1 progress, see section 2.1 above

<<skip existing fields>>

...,

timeInterleavingConfig-r19	SEQUENCE {

tbs-ScalingFactor-r19	ENUMERATED {n2, n4, n8, n16, ffs} 	OPTIONAL, -- Need OR

tb-Separation-r19	ENUMERATED {sf4, sf8, sf16, sf32, ffs} OPTIONAL -- Need OR

}							OPTIONAL, -- Need OR

freqInterleavingConfig-r19	ENUMERATED {enabled}		OPTIONAL, -- Need OR

mch-SchedulingPeriod-XX	ENUMERATED {ffs}	OPTIONAL -- Need OR

-- Exact values to be decided based on RAN1 further progress and whether this is extension to existing field(s) containing only new values (-v19xy), or as a critical extension (-r19) including all the values.

}

PMCH-InfoList field descriptions

freqInterleavingConfig Presence of the field indicates frequency interleaving is enabled as specified in TS 36.211 [21].

timeInterleavingConfig Presence of the field indicates time interleaving is enabled as specified in TS 36.213 [23].

tbs-ScalingFactor Indicates the TBS scaling factor as specified in TS 36.213 [23] when time interleaving is enabled. Value n2 indicates scaling factor 2, value n4 indicates scaling factor 4 and so on.

tb-Separation Indicates the separation between two successive transmissions of the same TB as specified in TS 36.213 [23] when time interleaving is enabled. Value sf4 indicates 4 subframes, value sf8 indicates 8 subframes and so on.

mch-SchedulingPeriod Indicates the MCH scheduling period i.e. the periodicity used for providing MCH scheduling information at lower layers (MAC) applicable for an MCH. Value rf8 corresponds to 8 radio frames, rf16 corresponds to 16 radio frames and so on. The mch-SchedulingPeriod starts in the radio frames for which: SFN mod mch-SchedulingPeriod = 0. E-UTRAN configures mch-SchedulingPeriod of the (P)MCH listed first in PMCH-InfoList to be smaller than or equal to mcch-RepetitionPeriod. In case mch-SchedulingPeriod-v1430 is configured, the UE shall ignore mch-SchedulingPeriod-r12. <<changes here TBD based on new values from RAN1>>

ASN.1 example given in section 5 can be taken as baseline for LTE RRC CR.

# Summary

Based on the discussion above, we have the following proposals:

Proposal 1:	Wait for RAN1 for the down-selection regarding the configuration for time interleaving transmission.

Proposal 2:	Wait for RAN1 for the candidate values of the new MCH scheduling period(s).

Proposal 3:	Introduce a field to indicate TBS scaling factor (i.e. N) with candidate values {2,4,8,16,ffs}.

Proposal 4:	Introduce a field to indicate the separation between two transmissions of the same TB (i.e. M) with candidate values {4,8,16,32,ffs}.

Proposal 5:	Presence of the parent IE containing fields for N and M indicates time interleaving is enabled.

Proposal 6:	Introduce a flag to indicate whether frequency interleaving for MTCH is enabled.

Proposal 7:	Postpone the discussion about UE capabilities to next meeting.

Proposal 8:	ASN.1 example given in section 5 can be taken as baseline for LTE RRC CR.

# References

[1] RP-250794, Revised WID on LTE-based 5G Broadcast Phase 2, RAN#107, March 2025

[2] R1-2501179, Time and Frequency Interleaving for 5G Broadcast, Qualcomm, RAN1#120, Athens, Feb 2025

[3] R1-2502861, Time and Frequency Interleaving for 5G Broadcast, Qualcomm, RAN1#120b, Wuhan, Apr 2025

[4] R1-2502548, Higher layer parameters for Rel-19 LTE-based 5G Broadcast after RAN1#120, EBU, Apr 2025

[5] R2-2503309/R1-2502982, LS on initial Rel-19 RAN1 UE features lists for LTE after RAN1#120bis, Apr 2025