3GPP TSG-WG2 Meeting # 130	R2-2504305

# Malta, MT,  19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0187 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sidelink Positioning, LPHAP, Carrier aggreation and frequency hopping correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 supports UL and DL carrier aggregation, UL and DL frequency hopping for Redcap devices, however these features are not described in TS 38.305. Further LPHAP UEs can be configured with eDRX beyond 10.24s, this is not captured in the specification. SL communication resource allocation is described in detail in TS 38.300 and part of it is copied in TS 38.305 for describing sidelink positioning resource allocation. However a reference for TS 38.300 is needed is TS 38.305 so that any future updates do not have to be captured in both specs. Editorial correction on LPHAP Abbrevation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new section for carrier aggregation, frequency hopping have been added in TS 38.305. LPHAP UEs can be configured with eDRX beyond 10.24s has been added. Missing reference of TS 38.300 for SL Positioning resource allocation has been added. Editorial Correction for LPHAP abbrevation. Impacted functionality: BW Aggregation, Frequency Hopping, LPHAP, SL Positioning Inter-operability: No Inter-Operability issue foreseen between UE and NW. Inter-operability: No Inter-Operability issue foreseen between UE and UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Feature description, incomplete specification, risk of future duplicate updates need between TS 38.305 and TS 38.300 for SL Resouce allocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 7.X (new), 7.Y (new), 7.14.1, 8.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501 "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 22.071: "Location Services (LCS); Service description, Stage 1".

[4]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[5]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[6]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[7]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Draft 0, Galileo Joint Undertaking, May 23rd, 2006.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5, 2002.

[10]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.0, June 17, 2008.

[11]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[12]	RTCM 10402.3, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[13]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 38.331: "NR Radio Resource Control (RRC) protocol specification".

[15]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[16]	OMA-TS-ULP-V2_0_6: "UserPlane Location Protocol Approved Version 2.0.6".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[19]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[20]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[21]	IEEE 802.11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

[22]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[23]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[24]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[25]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRA".

[26]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[27]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[28]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[29]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[30]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[31]	RTCM 10403.3, "RTCM Recommended Standards for Differential GNSS Services (v.3.3)", October 7, 2016.

[32]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[33]	3GPP TS 29.572: "Location Management Services; Stage 3".

[34]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017

[35]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[36]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[37]	3GPP TS 38.215: "NR; Physical layer – Measurements".

[38]	3GPP TS 38.401: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NG-RAN; Architecture description".

[39]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[40]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[41]	3GPP TS 24.571: "Control plane Location Services (LCS) procedures".

[42]	3GPP TS 37.355: "Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[43]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, August 2017.

[44]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[45]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

[46]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[47]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol specification".

[48]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[49]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[50]	3GPP TS 38.211: "NR; Physical channels and modulation".

[51]	Bluetooth Special Interest Group: "Bluetooth Core Specification v5.4", February 2023.

[52]	3GPP TS 38.300: "NR; NR and NR-RAN Overall Description; Stage 2".

[53]	3GPP TS 24.080: "Supplementary services specification; Formats and coding".

[54]	3GPP TS 24.514: "Ranging based services and sidelink positioning in 5G system (5GS); Stage 3".

[55]	3GPP TR 38.901: "Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz".

[56]	3GPP TS 24.572: "User Plane Location Services (LCS) Protocols And Procedures".

[XX]	3GPP TS 38.214: "NR; Physical layer procedures for data".

Next Change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

A-AoA	Azimuth-Angle of Arrival

ADR	Accumulated Delta Range

AL	Alert Limit

AoA	Angle of Arrival

AP	Access Point

APC	Antenna Phase Center

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

CID	Cell-ID (positioning method)

CLAS	Centimetre Level Augmentation Service

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-RSCP	Downlink Reference Signal Carrier Phase

DL-RSCPD	Downlink Reference Signal Carrier Phase Difference

DL-TDOA	Downlink Time Difference Of Arrival

DNU	Do Not Use

DRX	Discontinuous Reception

E-SMLC	Enhanced Serving Mobile Location Centre

E-CID	Enhanced Cell-ID (positioning method)

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered-Inertial

eDRX	Extended Discontinuous Reception

EGNOS	European Geostationary Navigation Overlay Service

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDMA	Frequency Division Multiple Access

FH	Frequency Hopping

FKP	Flächenkorrekturparameter (Engl: Area Correction Parameters)

GAGAN	GPS Aided Geo Augmented Navigation

GCS	Global Coordinate System (as defined in TR 38.901 [55])

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GMLC	Gateway Mobile Location Centre

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GRS80	Geodetic Reference System 1980

HESSID	Homogeneous Extended Service Set Identifier

IOD	Issue of Data

LCS	LoCation Services

Local Coordinate System (as defined in TR 38.901 [55])

LCS-UP	Location Services User Plane

LCS-UPP	Location Services User Plane Protocol

LMF	Location Management Function

LPHAP	Low Power High Accuracy Positioning

LPP	LTE Positioning Protocol

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

Multi-RTT	Multi-Round Trip Time

NavIC	NAVigation with Indian Constellation

NG-C	NG Control plane

NG-AP	NG Application Protocol

NI-LR	Network Induced Location Request

N-RTK	Network – Real-Time Kinematic

NRPPa	NR Positioning Protocol A

NTN	Non-Terrestrial Network

OTDOA	Observed Time Difference Of Arrival

PCO	Phase Center Offset

PCV	Phase Center Variation

PDU	Protocol Data Unit

posSI	Positioning System Information

posSIB	Positioning SIB

PPP	Precise Point Positioning

PPP-RTK	Precise Point Positioning – Real-Time Kinematic

PRS	Positioning Reference Signal (for E-UTRA)

PRU	Positioning Reference Unit

QZSS	Quasi-Zenith Satellite System

RP	Reception Point

RRM	Radio Resource Management

RSPP	Ranging/SL Positioning Protocol

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTD	Relative Time Difference

RTK	Real-Time Kinematic

SBAS	Space Based Augmentation System

SDT	Small Data Transmission

SET	SUPL Enabled Terminal

SIB	System Information Block

SL	Sidelink

SL-PRS	Sidelink Positioning Reference Signal

SL-PRS-RSRP	Sidelink PRS Reference Signal Received Power

SL-PRS-RSRPP	Sidelink PRS Reference Signal Received Path Power

SL-RSTD	Sidelink Reference Signal Time Difference

SL-RTOA	Sidelink Relative Time of Arrival

SLP	SUPL Location Platform

SLPP	Sidelink Positioning Protocol

SP	Semi-Persistent

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

SSID	Service Set Identifier

SSR	State Space Representation

STEC	Slant TEC

SUPL	Secure User Plane Location

TADV	Timing Advance

TBS	Terrestrial Beacon System

TEC	Total Electron Content

TEG	Timing Error Group

TP	Transmission Point

TRP	Transmission-Reception Point

TTA	Time To Alert

TxTEG	Tx Timing Error Group

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RSCP	Uplink Reference Signal Carrier Phase

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL-TDOA	Uplink Time Difference of Arrival

UPF	User Plane Function

URA	User Range Accuracy

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

Z-AoA	Zenith Angles of Arrival

Next Change

## 7.X	Bandwidth Aggregation for Positioning Measurements

DL-PRS/UL-SRS bandwidth aggregation for intra-band contiguous carriers is supported as specified in TS 38.133 [32], TS 38.214 [XX]. LMF provides assistance data to UE to perform bandwidth aggregated DL-PRS measurements. The assistance data contains up to 2 or 3 DL-PRS Resource sets which are linked to transmit DL-PRS in an aggregated fashion (see TS 37.355 [42]). gNB configures UE to transmit UL-SRS by linking up to 2 or 3 UL-SRS resource sets (see TS 38.331 [14]) from different carriers. This allows RPs to perform bandwidth aggregated UL measurements. Transmission of UL-SRS with bandwidth aggregation is supported in both RRC_CONNECTED and RRC_INACTIVE modes (see TS 38.331 [14]) whereas bandwidth aggregated measurement of DL-PRS is supported in all RRC modes (see TS 37.355 [42]).

## 7.Y	DL-PRS UL-SRS Frequency Hopping

For Reduced Capabilities (RedCap) UEs, FH for measurement of DL-PRS and transmission of UL-SRS for positioning is supported. The UE is able to measure DL-PRS resources with multiple hops at least over the BW as defined in TS 38.133 [xx] and transmit UL-SRS with multiple hops as defined in TS 38.214 [xx]. Transmission of UL-SRS with frequency hopping is supported in both RRC_CONNECTED and RRC_INACTIVE modes TS 38.331 [14] whereas measurement of DL-PRS with frequeny hopping is supported in all the RRC modes TS 37.355 [42].

Next Change

## 7.14	Procedures for Area-specific SRS Configuration

### 7.14.1	General

For LPHAP UEs, eDRX cycle beyond 10.24s TS 38.300 [52] is supported. To support LPHAP as defined in TS 23.273 [35], area-specific SRS configuration is used to enable SRS transmission by the UE in RRC_INACTIVE state, within positioning validity cell list(s).

### 7.14.2	Area-specific SRS (Pre-)configuration Allocation Procedure

Figure 7.14.2-1 shows the Area-specific SRS (Pre-)configuration Allocation procedure.

Next Change

## 8.15	SL positioning and ranging

### 8.15.1	SL-PRS transmission and reception

For SL positioning and ranging using SL-RTT, SL-AoA, SL-TDOA, and SL-TOA methods UEs need to transmit and/or receive SL-PRS as specified in TS 38.211 [50] over the NR PC5 interface.

A UE can be configured with one or more sidelink resource pools via system information or dedicated signalling while inside NG-RAN coverage or pre-configuration while outside NG-RAN coverage as specified in TS 38.331 [14].

A sidelink resource pool which can be used for transmission of both, SL-PRS and SL data is referred to as SL-PRS shared resource pool. A sidelink resource pool which can be used for transmission of SL-PRS and cannot be used for transmission of SL data is referred to as SL-PRS dedicated resource pool.

Two sidelink resource allocation schemes for SL-PRS are supported as specified in 5.7.2 of TS 38.300 [52]: scheme 1 and scheme 2. In scheme 1, the SL-PRS resource allocation is provided by the network. In scheme 2, the UE decides the SL-PRS transmission resources in the resource pool(s).

-	Scheme 1 resource allocation is characterized by:

-	The UE needs to be RRC_CONNECTED state in order to transmit SL-PRS;

-	NG-RAN schedules transmission resources.

-	Scheme 2 resource selection is characterized by:

-	The UE can transmit SL-PRS when inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when outside NG-RAN coverage;

-	the UE autonomously selects transmission resources from resource pool(s).

End of Changes