3GPP TSG-RAN WG2 Meeting #130                                    	R2-2504312

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Discussion on beam hopping with multiple SMTC offsets

Document for:	Discussion and Decision

# Introduction

In RAN2#129bis meeting, following agreements were made.

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this document, we further discuss the RAN2 aspects of the default SSB periodicity that is larger than 20ms and multiple SMTCs offsets beyond 4.

# Discussion

Currently the SSB extension periodicity of 160ms is already supported as specified in TS 38.331. However, it is to note that for initial access, the UE would not be aware of such extension from a cell and by default the assumption for UE is that the SSB periodicity is 20ms. This also impacts the MIB periodicity and default SIB1 repetition periodicity. Following is specified in TS 38.213.

- For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

This would mean by default the UE would be searching the SSB with the assumption of 20ms SSB periodicity and it can impact UE’s action for cell search is the SSB is not detected in the next 20ms.

Even though the UE can access a cell that is operating with default SSB periodicity of 160ms, the UE might be unnecessarily performing frequency search or even trying to measure the neighbor cell at wrong occasions. To avoid this, a cell should indicate the default SSB periodicity in SIB1. This helps UE to optimize the measurements and SSB detection. In addition, information of neighbor cells on the periodicity of larger than 20ms may also be considered to assist UE in neighbor cell measurements, however, this can be implicit from the SMTC periodicity configured for the neighbor cell measurements.

Currently, network can indicate SSB periodicity in SIB1. This would be sufficient for the extended SSB periodicity.

#### –	ServingCellConfigCommonSIB

The IE ServingCellConfigCommonSIB is used to configure cell specific parameters of a UE's serving cell in SIB1.

ServingCellConfigCommonSIB information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=      SEQUENCE {

downlinkConfigCommon                DownlinkConfigCommonSIB,

uplinkConfigCommon                  UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

supplementaryUplink                 UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                           OPTIONAL, -- Need S

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

ssb-PeriodicityServingCell          ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

## 5.2 System information ### 5.2.1 Introduction System Information (SI) is divided into the MIB and a number of SIBs and posSIBs where: - the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB; NOTE 1: If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB. - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

Existing signaling ssb-PeriodicityServingCell in serving cell common configuration is sufficient to indicate the default SSB periodicity of a cell.

Multiple SMTCs

In the last meeting, an LS [1] is sent to RAN4 on checking feasibility of 2 SMTC periodicities in the same frequency layer as follows:

Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases:

- RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements.

- RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

We think RAN2 can simply wait RAN4 to understand the complexity of multiple periodicities on the same frequency layer. However, more than 4 SMTCs can be considered, and we should define a mechanism for UEs not to select more than 4 SMTCs.

Specially there needs to be a way for UEs to select correct SMTCs/beams to measure (i.e., right SMTCs for the closest beams). There are some solutions discussed such as:

- SSB index based SMTC selection

- Location-based SMTC selection

In our view, how multi-beam operation works properly in NTN has never been discussed. In the context of RACH-less HO, RAN2 sent LS to RAN1 asking question on multiple beams and RAN2 received response from RAN1 in [2] with following response.

2. To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)?

RAN1 response: If single beam is indicated, UE will monitor the target cell PDCCH scheduling the first PUSCH based on the indicated beam. RAN1 will further discuss the case where multiple beams are indicated.

Figure 1 An Example of beam hopping with extended multiple SMTC offsets (i.e., 6 offsets)

However, RAN1 never discussed how multi-beam operation works in NTN as a single beam can be of very larger size. In RACH-less handover in NTN, we agreed there will be just one single target beam exactly for this reason.

In addition, multi-beam operation is just one of the many deployment choices. For this reason, SSB index based SMTC selection is not guaranteed to solve the issue. Therefore, we suggest defining a location-based SMTC selection procedure.

Introduce location-based SMTC selection procedure.

In location-based SMTC selection, each SMTC can be associated with a reference location of the intended neghbor beams that need to be measured by the UE. Then UE will select the supported set of SMTCs that are closest to the UE’s location. We think UE should calculate the distance from its position to the reference location, and choose the 4 SMTCs with the shortest distances. The UE will perform measurements using existing procedure and RRM requirements based on 4 SMTCs.

There can be ways to reduce signaling overhead, such as, indication of direction w.r.t. a reference location. However, this may not be necessary given sufficient TBS size in NR.

Each SMTC can be associated with a reference location of the intended neighbor beams that need to be measured by the UE.

The UE chooses 4 SMTCs based on the shortest distances from UE’s location to reference locations and perform RRM measurements using existing procedure as if smtc4list-r17 is configured.

Currently UE can support 4 SMTCs signalled via smtc4list-r17. In our view, the legacy UEs will not understand the new signaling so extension of SMTCs beyond 4 will not be applicable to them. It means the network has to keep signaling the smtc4list-r17.

It is possible to signal the delta using smtc4listExt-v19xy to add additional SMTC offsets and list of cells. The new UEs will concatenate the list smtc4list-r17 and smtc4listExt-v19xy.

It is also possible signal new smtc5list-r19 for supporting UEs in which other necessary information such as associated SMTC reference locations can also be provided. This works for configuring UEs in connected mode. However, for IDLE mode UEs, both smtc4list-r17 and smtc5list-r19 would have to be broadcast with smtc4list-r17 being redundant. But this can simplify the solutions as UEs supporting smtc5list-r19 can ignore the smtc4list-r17.

SSB-MTC5List-r19::=  SEQUENCE (SIZE(1..5)) OF SSB-MTC5-r19

SSB-MTC5-r19 ::= SEQUENCE {

pci-List-r19 	SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId  OPTIONAL, -- Need M

referenceLocation-r19 	ReferenceLocation-r17

offset-r19 		INTEGER (0..159)

}

The legacy UEs use smtc4list-r17 and new capable UEs ignore smtc4list-r17. For new capable UEs (i.e., UEs supporting location-based SMTC), new list is introduced to include PCI list, offset and associated reference location for each SMTC.

# Conclusion

Following proposals are made.

Proposal 1	Existing signaling ssb-PeriodicityServingCell in serving cell common configuration is sufficient to indicate the default SSB periodicity of a cell.

Proposal 2	Introduce location-based SMTC selection procedure.

Proposal 3	Each SMTC can be associated with a reference location of the intended neighbor beams that need to be measured by the UE.

Proposal 4	The UE chooses 4 SMTCs based on the shortest distances from UE’s location to reference locations and perform RRM measurements using existing procedure as if smtc4list-r17 is configured.

Proposal 5	The legacy UEs use smtc4list-r17 and new capable UEs ignore smtc4list-r17. For new capable UEs (i.e., UEs supporting location-based SMTC), new list is introduced to include PCI list, offset and associated reference location for each SMTC.

# References

[1] R2-2503068, “LS on SMTC enhancements” RAN2#129bis, Source RAN2, To RAN4.

[2] R2-2307011, “Reply LS on RACH-less Handover”, RAN2#123, Source RAN1, To RAN2.