3GPP TSG-RAN WG2 Meeting #130                                    	R2-2504314

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	6.1.2

Source:	Qualcomm Incorporated

Title:	Corrections on P-CSI report in NTN

Document for:	Discussion and Decision

# Introduction

CSI masking is setup for power saving such that UE is limited to send the periodic CSI report only during C-DRX ON duration. However, due to UE’s TA pre-compensation, UE may not be able to send the report if the UE would be in C-DRX sleep. In this document we provide details on how to address the issue.

# Discussion

In NTN, the logical DL symbol “n” and logical UL symbol “n” are not aligned. Network would only know the symbol “n” according to its timing and wouldn’t exactly know the UE’s timing.

When CSI masking is configured, UE’s periodic CSI reporting is limited to only when drx-onDurationTimer is running. As per current procedural text in TS 38.321, if the CURRENT symbol “n” (i.e., when UE checks this symbol now) is not within the ON duration, the UE will not send CSI report. However, when the CURRENT symbol “n” is within the ON duration, there will not be enough time for UE to be able to send the CSI report due to large TA compensation in NTN.

Figure 1 P-SCI reporting issue in NTN due to large TA

We see there can be multiple options to address the issue.

Option#1: Network allocates the resource with respect to Koffset

If network wants UE to send CSI report, network can do so by allocating the P-CSI report Koffset away from the start of the ON duration as shown in figure 2. This means if the symbol (n-UE’s TA) is within ON duration, UE can send the periodic CSI. If the symbol (n-UE’s TA) is outside the ON duration, the UE is not required to send the P-CSI report. However, this requires change in the procedural text to clarify the interpretation of symbol “n”.

Figure 2 Solution (option#2) where UE checks the CSI resource Koffset away

Text proposal for Option#1.

1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause: 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> not report semi-persistent CSI on PUCCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH. 1> else: 2> in current symbol n or for NTN, in current symbol (n-UE’s TA), if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n or for NTN, prior to symbol (n-UE’s TA) when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group; 3> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group. 2> if CSI masking (csi-Mask) is setup by upper layers: 3> in current symbol n or for NTN, in current symbol (n-UE’s TA), if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n or for NTN, prior to symbol (n-UE’s TA) when evaluating all DRX Active Time conditions as specified in this clause; and 3> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 4> not report CSI on PUCCH in this DRX group. NOTE 4: If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

Option#2: Leave it to UE

In this option, it can be left to UE implementation to wake up from sleep mode to be able to send the CSI report for symbol “n” which falls during ON duration because waking up too early will drain UE’s battery. This option requires just addition of clarification, and a note could be sufficient.

Text proposal for Option#2.

1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause: 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> not report semi-persistent CSI on PUCCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH. 1> else: 2> in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group; 3> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group. 2> if CSI masking (csi-Mask) is setup by upper layers: 3> in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 3> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 4> not report CSI on PUCCH in this DRX group. NOTE 4: If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s). NOTE 5: In NTN, if drx-onDurationTimer would not be running prior to symbol n, it is up to UE implementation whether to wakeup from C-DRX inactive time prior to symbol n to report periodic CSI or SRS.

Similar issue exists for periodic SRS and same clarification would be needed.

RAN2 decide whether to adopt option#1 or option#2:

- Option#1: In current symbol (n-UE’s TA), if drx-onDurationTimer of a DRX group would not be running, the UE does not report P-CSI on PUCCH in this DRX group.

- Option#2: if drx-onDurationTimer would not be running prior to symbol n (i.e., UE is in DRX sleep prior to symbol n), it is up to UE implementation whether to wakeup from C-DRX sleep prior to symbol n to report periodic CSI.

Same as in P-SCI, clarification is added for periodic SRS.

# Conclusion

Following proposals are made.

Proposal 1	RAN2 decide whether to adopt option#1 or option#2:

Option#1: In current symbol (n-UE’s TA), if drx-onDurationTimer of a DRX group would not be running, the UE does not report P-CSI on PUCCH in this DRX group.

Option#2: if drx-onDurationTimer would not be running prior to symbol n (i.e., UE is in DRX sleep prior to symbol n), it is up to UE implementation whether to wakeup from C-DRX sleep prior to symbol n to report periodic CSI.

Proposal 2	Same as in P-SCI, clarification is added for periodic SRS.