3GPP TSG-RAN WG2 Meeting#130                               	R2-2504347

Malta, May 19-23	2025

Agenda item:	8.19 (TEI-19)

Source: 	EchoStar, Boost Mobile, TerreStar, Qualcomm, TTP Plc

Title: 	NR-NTN to NB-IoT NTN mobility

Document for:	Discussion and Decision

# Introduction

In RAN#107 we proposed as part of Rel-20 enhancement a topic on “Co-existence of NTN-NR/NB-IoT NTN in Smartphone” [1]. Enhancing the availability of NTN-NR (or TN-NR) and NTN-IoT NTN chipsets within smartphones can enable smooth fallback to IoT-NTN services such as for text messaging services. This can be especially valuable for emergency text messaging when NR-NTN coverage is no longer available.

In this paper, sourcing companies propose a way forward to enable a mechanism for smooth NR-NTN (or NR-TN) to NB-IoT NTN mobility by providing inter-RAT cell selection information to ensure that UEs with NR-NTN (or NR-TN) and NTN-IoT capabilities can perform the necessary cell selection as needed.

# Discussion

Given the NB-IoT NTN is becoming very popular and will likely co-exist in the smartphones capable of NR-NTN (or NR-TN), it is beneficial for these UEs capable of both NB-IoT NTN and NR-NTN (or NR-TN) to use the satellite assistance information to search an NB-IoT NTN cell during when the NR-NTN (or NR-TN) signal is weak and vice versa.

- Observation 1: SIB19 is currently the most specific and dedicated SIB for NR-NTN-related system information, especially useful for measurement and mobility across NTN cells.

- Observation 2: NR currently does not provide inter-RAT NB-IoT cell selection or reselection information.

Therefore, in TS 38.331 there is a need to extend SIB (System Information Blocks) to indicate available NB-IoT RATs information including NB-IoT carrier frequency and NB-IoT satellite information. For example, SIB19 can be broadcast by both NR NTN and NR TN and SIB19 can be extended to include NB-IoT satellite information.

- Observation 3: In TS 36.331 there is a need to define a new NR carrier frequency and NR-NTN satellite information. As part of LTE to NR NTN mobility, the non-NB-IoT version of SIB33 has been extended to include the NR NTN satellite information. Therefore, in TS 36.331, same change can apply to NB-IoT version of SIB33. For NB-IoT, SIB27-NB can be extended to include NR carrier frequency and satellite ID.

In view of the growing interest and market need to enable smooth mobility from NR-NTN (or NR-TN) to NB-IoT NTN within an NR and NB-IoT capable device, we respectfully ask RAN2 to define a mechanism to provide a list of assistance information for smooth cell selection between NR-NTN (or NR-TN) and NB-IoT NTN.

- Proposal 1: For mobility from NB-IoT NTN to NR-NTN, RAN2 introduce a solution to provide a list of assistance information of NR-NTN carrier frequency and the NR-NTN satellite.

- Proposal 2: For mobility from NR-NTN to NB-IoT NTN, define a new IE ntn-NeighCellConfigList-NB-r19 in SIB19 to provide the list of NB-IoT satellite information and NB-IoT carrier information.

# Conclusion

In this paper, we discussed the need to enable a mechanism for inter-RAT selection to ensure that UEs with NR-NTN (or NR-TN) and NTN-IoT capabilities can perform the necessary selection as needed.

Based on the observation and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposal.

- Proposal 1: For mobility from NB-IoT NTN to NR-NTN, RAN2 introduce a solution to provide a list of assistance information of NR-NTN carrier frequency and the NR-NTN satellite.

- Proposal 2: For mobility from NR-NTN to NB-IoT NTN, define a new IE ntn-NeighCellConfigList-NB-r19 in SIB19 to provide the list of NB-IoT satellite information and NB-IoT carrier information.

# Reference

RP-250323 - Operators View NTN-IoT Rel-20

# Annex (TP for TS 36.331)

#### –	SystemInformationBlockType27-NB

The IE SystemInformationBlockType27-NB contains information relevant only for inter-RAT cell selection i.e. assistance information about E-UTRA frequencies and/or GERAN frequencies and/or NR-NTN frequencies for cell selection and/or cell reselection.

SystemInformationBlockType27-NB information element

-- ASN1START

SystemInformationBlockType27-NB-r16 ::=	SEQUENCE {

carrierFreqListEUTRA-r16				CarrierFreqListEUTRA-NB-r16		OPTIONAL,	-- Need OR

carrierFreqsListGERAN-r16				CarrierFreqsListGERAN-NB-r16	OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...,

[[	carrierFreqListNR-v19xy			CarrierFreqListNR-NB-v19xy		OPTIONAL		-- Need OR

]]

}

CarrierFreqListEUTRA-NB-r16 ::=			SEQUENCE (SIZE (1..maxFreqEUTRA-NB-r16)) OF

CarrierFreqEUTRA-NB-r16

CarrierFreqsListGERAN-NB-r16 ::=		SEQUENCE (SIZE (1..maxFreqsGERAN-NB-r16)) OF

CarrierFreqsGERAN-NB-r16

CarrierFreqListNR-NB-v19xy ::=		SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-NB-v19xy

CarrierFreqEUTRA-NB-r16 ::=				SEQUENCE {

carrierFreq-r16							ARFCN-ValueEUTRA-r9,

sib1-r16								ENUMERATED {supported}		OPTIONAL,	-- Need OR

sib1-BR-r16								ENUMERATED {supported}		OPTIONAL,	-- Need OR

...

}

CarrierFreqsGERAN-NB-r16 ::=			SEQUENCE {

carrierFreqs-r16						CarrierFreqsGERAN,

ec-GSM-IOT-r16							ENUMERATED {supported}		OPTIONAL,	-- Need OR

peo-r16									ENUMERATED {supported}		OPTIONAL,	-- Need OR

...

}

CarrierFreqNR-NB-v19xy ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

multiBandInfoList-r15				MultiFrequencyBandListNR-r15		OPTIONAL,	-- Need OR

satAssistanceInfoList-r19		SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18	OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType27-NB field descriptions

carrierFreq E-UTRAN carrier frequency.

carrierFreqListEUTRA Provides a list of neighbouring E-UTRA carrier frequencies, which may be searched for neighbouring E-UTRAN cells.

carrierFreqs The list of GERAN carrier frequencies organised into one group of GERAN carrier frequencies.

carrierFreqsListGERAN Provides a list of neighbouring GERAN carrier frequencies, which may be searched for neighbouring GERAN cells. The GERAN carrier frequencies are organised in groups and the parameters are indicated per group of GERAN carrier frequencies.

carrierFreqListNR Provides a list of neighbouring NR carrier frequencies, which may be searched for neighbouring NR cells.

ec-GSM-IOT Indicates that the GERAN carrier frequencies support EC-GSM-IOT.

peo Indicates that the GERAN carrier frequencies support Power Efficient Operation (PEO).

sib1 Indicates that SIB1 is scheduled in the E-UTRAN cells.

sib1-BR Indicates that SIB1-BR is scheduled in the E-UTRAN cells.

<<skipped>>

#### –	SystemInformationBlockType33-NB

The IE SystemInformationBlockType33-NB contains satellite assistance information for neighbour cells.

SystemInformationBlockType33-NB information element

-- ASN1START

SystemInformationBlockType33-NB-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18		NeighSatelliteInfoList-r18		OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[

neighSatelliteInfoListNR-r19	NeighSatelliteInfoListNR-r19			OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

# Annex (TP for TS 38.331)

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

ntn-CovEnh-r18                           NTN-CovEnh-r18                                  OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]],

[[

ntn-NeighCellConfigList-NB-r19         NTN-NeighCellConfigList-NB-r19                     OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

NTN-NeighCellConfigList-NB-r19 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-NB-r19

NTN-NeighCellConfig-NB-r19 ::=              SEQUENCE {

ntn-Config-r19                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-NB-r19                       CarrierFreq-NB-r19                                   OPTIONAL,       -- Need R

t-ServiceStartNeigh-r19 					  TimeOffsetUTC-r17 OPTIONAL -- Need OR

}

NTN-CovEnh-r18 ::=                       SEQUENCE {

numberOfMsg4HARQ-ACK-Repetitions-r18     BIT STRING (SIZE(4)),

rsrp-ThresholdMsg4HARQ-ACK-r18           RSRP-Range                                      OPTIONAL        -- Need R

}

SatSwitchWithReSync-r18 ::=              SEQUENCE {

ntn-Config-r18                           NTN-Config-r17,

t-ServiceStart-r18                       INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

ssb-TimeOffset-r18                       INTEGER (0..159)                                OPTIONAL        -- Need R

}

CarrierFreq-NB-r19 ::= 				SEQUENCE {

carrierFreq-r19 					ARFCN-ValueEUTRA,

carrierFreqOffset-r19 				ENUMERATED { v-10, v-9, v-8, v-7, v-6, v-5, v-4, v-3, v-2, v-1, v-0dot5, v0, v1, v2, v3, v4, v5, v6, v7, v8, v9 } OPTIONAL -- Need ON

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m. This field is only present in an NTN cell.

movingReferenceLocation Reference location of the serving cell of an NTN Earth-moving cell at a time reference. It is used in the evaluation of eventD2 and condEventD2 criteria for the serving cell in RRC_CONNECTED, and location-based measurement initiation in RRC_IDLE and RRC_INACTIVE when distanceThresh is also configured, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell. This field is excluded when determining changes in system information, i.e., changes to movingReferenceLocation should neither result in system information change notifications nor in a modification of valueTag in SIB1. This field is only present in an NTN cell.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch time. In a TN cell, this field is only present in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList and all elements of ntn-NeighCellConfigListExt. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. Network provides ntn-Config for the first entry of ntn-NeighCellConfigList. If the ntn-Config is absent for any other entry in ntn-NeighCellConfigList, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList applies.

ntn-NeighCellConfigList-NB Provides a list of NB-IoT NTN neighbour satellites including their ntn-Config, carrier frequency and earliest time when the area covered by the current serving cell is going to be covered by the neighbour satellite (t-ServiceStartNeigh).

referenceLocation Reference location of the serving cell provided via NTN (quasi)-Earth fixed cell and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. This field is only present in an NTN cell.

satSwitchWithReSync Provides parameters for the target satellite required to perform satellite switch with resynchronization. This field is only present in an NTN cell and its presence indicates that satellite switch without PCI change is supported in the cell.

t-Service Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-Service is the uplink time synchronization reference point of the cell. This field is only present in an NTN cell.

NTN-CovEnh field descriptions

numberOfMsg4HARQ-ACK-Repetitions The number of repetition slots for PUCCH transmission with HARQ-ACK information for Msg4, see clause 9.2.6 in TS 38.213 [13]. The first/leftmost bit corresponds to the repetition factor 1, the second bit corresponds to the repetition factor 2, the third bit corresponds to the repetition factor 4, and the last/rightmost bit corresponds to the repetition factor 8. The repetition factor 1 shall be indicated together with at least one other repetition factor.

rsrp-ThresholdMsg4HARQ-ACK This threshold is used by the UE for determining the configuration of the MAC entity for PUCCH repetition for Msg4 HARQ-ACK, as specified in clause 6.2.1 in TS 38.321 [3].

SatSwitchWithReSync field descriptions

ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes.

t-ServiceStart Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-ServiceStart is the uplink time synchronization reference point of the serving satellite.