3GPP TSG-RAN WG2 Meeting #130		     R2-2504354

Malta, MT: 19th May – 23rd May 2025

Source:	Qualcomm Incorporated

Title:	Open Issues on Network Side Data Collection

Agenda Item:	8.1.3 NW Side Data Collection

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, in RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved [3].

This contribution paper discusses the open issues on data collection for the network side model training for beam management and positioning enhancements use cases.

# Data Collection for Network Side Models

This section discusses the following:

- Data collection for network-side model training for the beam management use case,

- Data collection for network-side model training for the positioning enhancements use case.

## 2.1   Data Collection for Network-side Models for the Beam Management

In this section, considering RAN2 agreements for NW-side data collection, we discuss the following open issues

- UE capability considerations

- Further discussion on event-triggered data collection,

- Multiple configurations and dynamic activation/deactivation,

- Handling data collection configuration and report during UE mobility.

This section discusses the FFS issues mentioned above.

### 2.1.1	UE Capability Considerations

The training data configuration consists of measurement targets and logging configuration (e.g., periodicity, event triggers for logging). The configuration/activation of the data collection configuration for NW-side training should be subject to UE capability. If the UE supports the training data collection and logging for the network side model training, the network should ensure that SSB/CSI-RS is configured for measurements across different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring) and different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes) lie within the maximum number of SSB/CSI-RS measurements supported at the UE.

Proposal 1: If the UE supports the training data collection and logging for the network side model training, the network should continue to ensure that the number of SSB/CSI-RS configured/activated for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. This includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring).

### 2.1.2	Reporting Assistance Information related to Logged Measurements

In RAN2#129bis meeting [4], RAN2 agreed on the following

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Note that the UE may need to send the low-power indication, irrespective of whether the UE has logged measurements or not. Therefore, low power indication should be considered independently from data availability indication. On the other hand, for the data availability indication, indicating the cause should be sufficient, as UE should set the cause or reason for trigger UAI only when UE has available data. Therefore, for data availability, a single bit indication is used to indicate the reason for the trigger (full buffer, threshold).

Proposal 2: Introduce a one-bit indication for low power indication.

Proposal 3: For the data availability, a one-bit indication is used to indicate the reason for the trigger (full buffer, threshold). Data availability is implicitly indicated by the reason for the trigger (full buffer, threshold).

### 2.1.3	Further Procedure for UE Assistance Information related to Logging

The UE should be able to indicate to the network if the configured/activated CSI-ResourceConfig is not suitable, i.e., if the UE cannot detect the configured/activated CSI-ResourceConfig for the network side data collection, the UE should indicate this to the network.

Proposal 4: If the UE cannot detect the configured/activated CSI Resource (CSI-RSs or SSBs) for the network side data collection, the UE indicates to the network that the configured/activated CSI-ResourceConfig is not suitable.

### 2.1.4	CellID Reporting in the Logged Data for NW-side Data Collection

In the running CR [5], for configuring logging of L1 measurement, CSI-LoggedMeasurementConfig is introduced. The CSI-LoggedMeasurementConfig for logging L1 measurements should be provided per serving cell to collect measurements on PCell and its SCell, or PSCell and its SCells. Therefore, the CSI-LoggedMeasurementConfig should contain the serving cell index. In the logged L1 measurement report, the UE includes the CSI-LoggedMeasurementConfigID. The serving cell index can be derived based on the CSI-LoggedMeasurementConfigID reported in the Logged measurement report. Furthermore, when the CSI-logMeasInfo is retrieved by the configuring node, the cell can be determined by the MN unambiguously based on the reported CSI-LoggedMeasurementConfigID in the CSI-logMeasInfo. Additional reporting of cellID together with CSI-LoggedMeasurementConfigID may be needed only when the CSI-logMeasInfo is retrieved by the target cell after the handover. The UE may report the CGI of the cell if available; otherwise reports the PCI and ARFCN.

Proposal 5: The CSI-LoggedMeasurementConfig for logging L1 measurements should be provided per serving cell to collect measurements on PCell and its SCell, or PSCell and its SCells.

Proposal 6: The CSI-LoggedMeasurementConfig should contain the serving cell index.

Observation 1: The CSI-logMeasInfo (containing L1 measurement for network-side model training) is primarily retrieved by the configuration node. Therefore, based on the reported CSI-LoggedMeasurementConfigID in the CSI-logMeasInfo, the network should be able to determine the CellID unambiguously.

Proposal 7: No need to report cellID additionally in the CSI-logMeasInfo. The network determines the CellID unambiguously based on the reported CSI-LoggedMeasurementConfigID in the CSI-logMeasInfo.

Proposal 8: Additional reporting of cellID together with CSI-LoggedMeasurementConfigID may be needed only when the CSI-logMeasInfo is retrieved by the target cell after the handover.

Proposal 9: The UE may report the CGI of the cell if available; otherwise reports the PCI and ARFCN.

### 2.1.5	Multiplexing Legacy Logged Data and AI/ML Logged Data in New SRB

RAN2 has agreed on UEInformationRequest and UEInformationResponse messages for retrieving CSI-logMeasInfo, which is the same for requesting and reporting existing SON and logged MDT report. We believe that the new SRB can be used to report existing SON and logged MDT report, which have been traditionally reported over SRB2.

Proposal 10: The new SRB introduced to report data collection for network side model training can be used to report the existing SON and logged MDT report.

### 2.1.6	Handling Data Collection Configuration and Reporting during UE Mobility

In RAN2#129bis meeting [4], RAN2 agreed on the following for handling the logged measurement for the network-side model training,

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

##### 2.1.6.1 Handling of logged training data during mobility

For 1-bit indication, there are two possible solutions discussed in [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson) [5] as follows:

- The serving cell preconfigures the UE with a 1-bit indication on whether to release or retain unretrieved data,

- The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. No RAN3 impact is expected if the transmission of the 1-bit indication is limited to the case in which the target gNB is from the same vendor as the source gNB.

The purpose of indicating a 1-bit indication on whether to release or retain unretrieved data in RRCReconfiguration is to avoid inter-vendor information leakage. The serving cell may not even know the suitable target cell beforehand, therefore, it may not know whether the target cell belongs to the same vendor or a different one. Therefore, the serving cell cannot preconfigure the UE with a 1-bit indication on whether to release or retain unretrieved data.

Observation 2: The purpose of indicating a 1-bit indication on whether to release or retain unretrieved data in RRCReconfiguration is to avoid inter-vendor information leakage.

Observation 3: The serving cell may not even know the suitable target cell beforehand, therefore, it may not know whether the target cell belongs to the same vendor or a different one.

Proposal 11: The serving cell cannot preconfigure the UE with a 1-bit indication on whether to release or retain unretrieved data, as it cannot know the target cell beforehand.

On the other hand, a rogue target cell may send the 1-bit indication without an indication from the serving cell, which may result in inter-vendor information leakage. Therefore, to avoid inter-vendor information leakage, the serving cell can configure the UE with cell IDs (among the neighbor cell list), where the UE can send availability indications and data collection reports, is more suitable.

Proposal 12: Receiving the 1-bit indication from the target gNB is not a suitable solution, as a rogue target cell may send the 1-bit indication irrespective of whether it is configured to do so or not by the serving cell.

Proposal 13: To avoid inter-vendor information leakage, the serving cell can configure the UE with cell IDs (among the neighbor cell list), where the UE can send availability indications and data collection reports, is more suitable.

Observation 4: Since the scope is only for the handover, the cell ID list can be significantly smaller, resulting in less overhead.

# Conclusion

Data collection and reporting for the beam management use case

Proposal 1: If the UE supports the training data collection and logging for the network side model training, the network should continue to ensure that the maximum number of SSB/CSI-RS configured/activated for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring).

Proposal 2: Introduce a one-bit indication for low power indication.

Proposal 3: For the data availability, a one-bit indication is used to indicate the reason for the trigger (full buffer, threshold). Data availability is implicitly indicated by the reason for the trigger (full buffer, threshold).

Proposal 4: If the UE cannot detect the configured/activated CSI Resource (CSI-RSs or SSBs) for the network side data collection, the UE indicates to the network that the configured/activated CSI-ResourceConfig is not suitable.

Proposal 5: The CSI-LoggedMeasurementConfig for logging L1 measurements should be provided per serving cell to collect measurements on PCell and its SCell, or PSCell and its SCells.

Proposal 6: The CSI-LoggedMeasurementConfig should contain the serving cell index.

Observation 1: The CSI-logMeasInfo (containing L1 measurement for network-side model training) is primarily retrieved by the configuration node. Therefore, based on the reported CSI-LoggedMeasurementConfigID in the CSI-logMeasInfo, the network should be able to determine the CellID unambiguously.

Proposal 7: No need to report cellID additionally in the CSI-logMeasInfo. The network determines the CellID unambiguously based on the reported CSI-LoggedMeasurementConfigID in the CSI-logMeasInfo.

Proposal 8: Additional reporting of cellID together with CSI-LoggedMeasurementConfigID may be needed only when the CSI-logMeasInfo is retrieved by the target cell after the handover.

Proposal 9: The UE may report the CGI of the cell if available; otherwise reports the PCI and ARFCN.

Proposal 10: The new SRB introduced to report data collection for network side model training can be used to report the existing SON and logged MDT report.

Observation 2: The purpose of indicating a 1-bit indication on whether to release or retain unretrieved data in RRCReconfiguration is to avoid inter-vendor information leakage.

Observation 3: The serving cell may not even know the suitable target cell beforehand, therefore, it may not know whether the target cell belongs to the same vendor or a different one.

Proposal 11: The serving cell cannot preconfigure the UE with a 1-bit indication on whether to release or retain unretrieved data, as it cannot know the target cell beforehand.

Proposal 12: Receiving the 1-bit indication from the target gNB is not a suitable solution, as a rogue target cell may send the 1-bit indication irrespective of whether it is configured to do so or not by the serving cell.

Proposal 13: To avoid inter-vendor information leakage, the serving cell can configure the UE with cell IDs (among the neighbor cell list), where the UE can send availability indications and data collection reports, is more suitable.

Observation 4: Since the scope is only for the handover, the cell ID list can be significantly smaller, resulting in less overhead.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-250792, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #107, March 2025

- Meeting Report, “RAN2#129bis meeting report”

- [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)

- Meeting Report, “RAN2#127bis meeting report”

- Meeting Report, “RAN2#129 meeting report”

- [POST128][019][AI PHY] NW side data collection (Nokia) email discussion report

- TS 38.331, RRC Specification

- TS 38.214

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".