3GPP TSG-RAN2 Meeting #130			R2-2504375

St. Julian, Malta, 19th-23rd May

Agenda item:	8.7.6

Title:	Further consideration on Rate control

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting [1], RAN2 discusses some issues related to rate control and formulates agreements as follows.

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

This contribution mainly discusses some remaining issues on rate control.

Discussion

  - Bit rate table for UL rate control, DL rate control, and SRB

Regarding to the agreements on the last meeting, the rate table for Rate Control MAC CE can be formulated by:

Rk=Rmin×1+pk

p=Rmax/Rmin1/N-1

With a rate range from 0.1 Mbps to 40 Mbps and 8 code points, the parameter p≈0.048, where p is also the quantization error.

Such a 4.8% quantization error is considered acceptable for rate control on XR applications, where rate control tends to be adaptive and misalignment can be mitigated in subsequent adjustments. Indeed, in rate control mechanisms that operate periodically or upon event triggers, the immediate precision of a single update is less critical than the long-term responsiveness and scheduling robustness.

Observation 1: With 8 codepoints and bit rate ranges from 0.1 to 40 Mbps, the expected QE is about 4.8%, which is acceptable for XR applications as accuracy in rate control is not a critical concern.

Proposal 1: The rate control table ranges from 0.1 to 40Mbps.

Consider the scope of WID and the fact that DL rate control is totally up to gNB implementation, DL rate control and DL rate control query is not in the scope, therefore it’s suggested that DL rate control and query is not supported in RAN2 at least in Rel-19.

Proposal 2: RAN2 will not specify DL rate control and DL rate control query.

It is also noted that SRBs are not subject to rate control via the Rate Control MAC CE. This reflects their distinct role and criticality in control plane operations, where data transmission must remain uninhibited by rate-limiting procedures.

Proposal 3: SRB is not affected by rate control.

  - UL Rate Control MAC CE

Still, challenges arise in associating rate control operations with appropriate granularity. In legacy, MAC layer rate control is typically applied per DRB by PBR and Bj, leveraging the DRB as the granularity point for MAC-layer scheduling.

In 5G NR, particularly under a CU-DU split architecture, the MAC entity may not possess awareness of the mapping between DRBs and their constituent QoS flows. This disconnect necessitates explicit signaling that enables proper alignment of rate control decisions with both MAC-layer scheduler and higher-layer QoS profile adjustment.

Observation 2a: Capture DRB identity in Rate Control MAC CE is beneficial for LCP and fast rate control at UE side, as LCP is on LCH granularity.

To address this, it is proposed that each Rate Control MAC CE include both a DRB ID and a QFI. This dual-indication approach ensures compatibility across layers: the DRB ID enables immediate applicability at the MAC layer, while the QFI provides context for QoS-aware decisions in upper layer and, if needed, at the application layer.

Another reason to capture DRB ID in Rate Control MAC CE is that QFI across different PDU session may be the same, therefore, indicating QFI alone may cause some ambiguity.

Observation 2b: Capture DRB identity in Rate Control MAC CE can also avoid potential ambiguity in case two PDU sessions have QoS flows with the same QFI.

Proposal 4: The QoS flow in the Rate control MAC CE is indicated via the combination of DRB ID and QFI.

The situation becomes more complex when multiple QoS flows are multiplexed within a single DRB. In such cases, the signaling must support multiple DRB-QFI pairs. A deterministic ordering is recommended for encoding multiple entries, and an ascending or descending order can be preferred, we don’t have strong preference. This allows for predictable parsing and ensures that flows with higher priority receive rate control decisions that minimize adverse impact, thus aligning with QoS expectations.

Proposal 5: Support rate control over multiple QoS flows by assembling the DRB-QFI pairs in an ascending or descending order with regarding to QoS flow priority in the Rate Control MAC CE, FFS which one.

Given that the maximum number of DRB identities is 32, or 29 which is more precise regarding the maximum number of DRBs that can be added, and the number of QFIs may reach up to 64, a 4-bit field for DRB ID and another 6-bit filed QFI should be captured in the Rate Control MAC CE.

Proposal 6: The Rate Control MAC CE should consist of one or multiple set(s) of 5-bit LCH ID, 6-bit QFI, and 8-bit (physical) bit rate.

Figure 2.2-1: An example of proposed Rate Control MAC CE

As can be seen, there are many padding bits in the proposed MAC CE. RAN2 may consider if there are any optimization for overhead if time allows, e.g., there are only one or a few QoS flow in each DRB.

  - Handling of triggered UL rate queries

In the last meeting, an uplink rate query mechanism is introduced, allowing the UE to request suitable UL bit rate from the network. This query may be triggered by upper layer, e.g., the application layer, and could have implications for SA4/CT1 if application behavior influences the triggering procedure.

Proposal 7a: The UL Rate Query MAC CE is triggered by upper layer, e.g., UE application layer. FFS there will be SA4/CT1 impact on indication of QoS flow or it’s up to UE implementation.

Because the query may not be always transmitted immediately, pending uplink rate queries are maintained at the MAC layer. These pending entries reflect outstanding requests for network rate indication. However, their presence does not trigger a scheduling request, as they typically relate to QoS flow(s) that are already scheduled or expected to become active soon. This design avoids unnecessary signaling and prevents SR overload.

Proposal 7b: The triggered UL Rate Query will not trigger the SR.

A pending UL rate query is cleared when the corresponding Rate Query MAC CE is transmitted, or when a Rate Control MAC CE is received for the queried QoS flow, indicating that the network has already provided updated rate guidance. Temporary unavailability of data does not cancel the query, ensuring that intent to align on rate is preserved across traffic gaps.

Proposal 7c: The triggered UL Rate Query is cancelled when:

- A MAC PDU is transmitted and this PDU includes a UL Rate Query MAC CE.

- The MAC entity receives an UL Rate Control MAC CE inducing all the LCH(s) and QoS flow(s) reported in the UL Rate Query MAC CE.

- FFS any other case.

To ensure proper multiplexing, the Rate Query MAC CE is given a lower transmission priority than DSR, but higher than Padding BSRs. This ensures timely handling without interfering with more urgent control messages. From our perspective, the legacy RBR query MAC CE is good reference.

Proposal 8: The priority of Rate Query MAC CE should be the same as the MAC CE for Recommended bit rate query.

Given that the query mechanism inherently generates additional signalling, mechanisms to limit its frequency are essential. Drawing from legacy practices, a prohibit timer configured per DRB has proven effective in constraining the periodicity of similar control messages. Applying a similar approach to the Rate Query MAC CE allows the network to regulate how often a UE may initiate a query for a specific DRB, thereby reducing signalling overhead while preserving responsiveness in dynamic scenarios.

Proposal 9: Introduce a per LCH prohibit timer for UL rate query to avoid signaling overhead.

  - UL Rate Query MAC CE

In XR uplink transmission, the UE often lacks sufficient information to determine an appropriate and efficient target rate without network assistance. While downlink traffic may benefit from centralized scheduling and network-side optimization, uplink adaptation relies more heavily on UE-side estimations and prediction.

This misalignment between UE and gNB becomes particularly pronounced in unstable radio environments or when the UE supports multiple applications with differing QoS characteristics. Consequently, there arises a need for a standardized mechanism through which the UE can query the network regarding a suitable transmission rate, thereby improving alignment between demand and schedulable capacity.

To address this, the introduction of the Rate Query MAC CE has been considered. It is observed that uplink rate queries must be constrained to minimize signalling overhead, especially in scenarios involving multiple QoS flows or DRBs. Supporting both DRB-level and QoS flow-level granularity within the Rate Query MAC CE ensures compatibility with a range of deployment configurations and application requirements. In particular, DRB-level granularity provides backward compatibility and immediate applicability for MAC scheduling, while QoS flow-level querying offers finer control for differentiated service scenarios.

Proposal 10: The Rate Query MAC CE should support both DRB and QoS flow granularity.

Similar to the Rate Control MAC CE, the Rate Query MAC CE should also support multiplexing and include both DRB ID and QFI.

Proposal 11: The Rate Query MAC CE should consist of one or multiple pair(s) of 5-bit LCH ID and 6-bit QFI.

Figure 2.3-1: An example of proposed Query Control MAC CE

  - Other WG impact

RAN3 hasn’t determine whether it’s per QoS flow or per DRB, but RAN3 agrees to introduce an indicator over F1AP. DU may need to be indicated on how QoS flow is mapped into DRB.

For Specify uplink congestion signalling: Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control. Introduce the same indication in XnAP to inform target gNB. FFS on per QoS flow indication or per DRB indication over F1AP?

Also, rate query may have impact on SA4 and CT1.

Proposal 12: Send an LS to RAN3/SA4/CT1 to synchronize RAN2’s agreements on rate control and query.

Conclusion

Based on the abovementioned analysis and observations, the following is proposed.

Overall:

Observation 1: With 8 codepoints and bit rate ranges from 0.1 to 40 Mbps, the expected QE is about 4.8%, which is acceptable for XR applications as accuracy in rate control is not a critical concern.

Proposal 1: The rate control table ranges from 0.1 to 40Mbps.

Proposal 2: RAN2 will not specify DL rate control and DL rate control query.

Proposal 3: SRB is not affected by rate control.

UL Rate Control MAC CE;

Observation 2a: Capture DRB identity in Rate Control MAC CE is beneficial for LCP and fast rate control at UE side, as LCP is on LCH granularity.

Observation 2b: Capture DRB identity in Rate Control MAC CE can also avoid potential ambiguity in case two PDU sessions have QoS flows with the same QFI.

Proposal 4: The QoS flow in the Rate control MAC CE is indicated via the combination of DRB ID and QFI.

Proposal 5: Support rate control over multiple QoS flows by assembling the DRB-QFI pairs in an ascending or descending order with regarding to QoS flow priority in the Rate Control MAC CE, FFS which one.

Proposal 6: The Rate Control MAC CE should consist of one or multiple set(s) of 5-bit LCH ID, 6-bit QFI, and 8-bit (physical) bit rate.

Handling of UL Rate Queries:

Proposal 7a: The UL Rate Query MAC CE is triggered by upper layer, e.g., UE application layer. FFS there will be SA4/CT1 impact on indication of QoS flow or it’s up to UE implementation.

Proposal 7b: The triggered UL Rate Query will not trigger the SR.

Proposal 7c: The triggered UL Rate Query is cancelled when:

- A MAC PDU is transmitted and this PDU includes a UL Rate Query MAC CE.

- The MAC entity receives an UL Rate Control MAC CE inducing all the LCH(s) and QoS flow(s) reported in the UL Rate Query MAC CE.

- FFS any other case.

Proposal 8: The priority of Rate Query MAC CE should be the same as the MAC CE for Recommended bit rate query.

Proposal 9: Introduce a per LCH prohibit timer for UL rate query to avoid signaling overhead.

UL Rate Query MAC CE:

Proposal 10: The Rate Query MAC CE should support both DRB and QoS flow granularity.

Proposal 11: The Rate Query MAC CE should consist of one or multiple pair(s) of 5-bit LCH ID and 6-bit QFI.

Impact on other WGs:

Proposal 12: Send an LS to RAN3/SA4/CT1 to synchronize RAN2’s agreements on rate control and query.

References

R2-2502985, Report from session on R19 XR, Session chair (Huawei)