3GPP TSG RAN WG2 Meeting #130	R2-2504391

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.3

Source:	CMCC

Title:	Discussion on uplink capacity/throughput enhancement for NR NTN

Document for:	Discussion, Discussion

Introduction

In last RAN2 meeting, there were some discussions on Uplink Capacity/Throughput Enhancement and some agreements were achieved as follows [1]:

Agreements;

1.	There is no need to introduce dedicated access control mechanism for OCC capable UEs

And according to RAN1 discussion progress, two OCC code lengths are support at least and related agreements are achieved as following:

- RAN1 #119:

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

- RAN1 #120bis:

Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

In this contribution, we would like to provide some discussion on RAN2 aspects about uplink capacity/throughput enhancement based on above agreements reached in RAN1.

Discussion

UE capability

In RAN1 #119 meeting, the UE capability aspects is confirmed as follows:

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Then in RAN2, we need to introduce new UE capability to support OCC to keep alignment. Define one UE capability with OCC length 2 and other one capability with OCC length 4 which means also support OCC length 2 simultaneously.

Proposal 1: Introduce 2 new UE capabilities to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory.

OCC configuration

Per the RAN1 #120bis agreements, for configured grant PUSCH type 1, the OCC length and sequence are agreed to be provided by RRC higher layers, and up to RAN2 to configure one or two RRC parameters. Then for DG PUSCH and CG PUSCH Type 2, the OCC sequence is determined to be indicated dynamically in DCI while the OCC length is needed to be further discussed among 5 candidate options mentioned above. And existing OCC related configuration in PUCCH-Config is as following:

<<omitted>> PUCCH-Resource ::= SEQUENCE { pucch-ResourceId PUCCH-ResourceId, startingPRB PRB-Id, intraSlotFrequencyHopping ENUMERATED { enabled } OPTIONAL, -- Need R secondHopPRB PRB-Id OPTIONAL, -- Need R format CHOICE { format0 PUCCH-format0, format1 PUCCH-format1, format2 PUCCH-format2, format3 PUCCH-format3, format4 PUCCH-format4 } } PUCCH-ResourceExt-v1610 ::= SEQUENCE { interlaceAllocation-r16 SEQUENCE { rb-SetIndex-r16 INTEGER (0..4), interlace0-r16 CHOICE { scs15 INTEGER (0..9), scs30 INTEGER (0..4) } } OPTIONAL, --Need R format-v1610 CHOICE { interlace1-v1610 INTEGER (0..9), occ-v1610 SEQUENCE { occ-Length-v1610 ENUMERATED {n2,n4} OPTIONAL, -- Need M occ-Index-v1610 ENUMERATED {n0,n1,n2,n3} OPTIONAL -- Need M } } OPTIONAL, -- Need R ..., [[ format-v1700 SEQUENCE { nrofPRBs-r17 INTEGER (1..16) } OPTIONAL, -- Need R <<omitted>> PUCCH-format4 ::= SEQUENCE { nrofSymbols INTEGER (4..14), occ-Length ENUMERATED {n2,n4}, occ-Index ENUMERATED {n0,n1,n2,n3}, startingSymbolIndex INTEGER(0..10) } <<omitted>>

PUCCH-Resource, PUCCH-ResourceExt field descriptions

<<omitted>>

format Selection of the PUCCH format (format 0 – 4) and format-specific parameters, see TS 38.213 [13], clause 9.2. format0 and format1 are only allowed for a resource in a first PUCCH resource set. format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. The network can only configure format-v1610 when format is set to format2 or format3. The network only configures format-v1700 when format is set to format0, format1 or format4.

<<omitted>>

occ-Index Indicates the orthogonal cover code index (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

occ-Length Indicates the orthogonal cover code length (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

Similarly, at least for CG PUSCH type 1, to provide OCC configuration, we could consider to introduce a new IE (e.g. occ-v19) to include OCC length and OCC sequence related configuration in PUSCH-Config and specific parameters could wait for RAN1. Then network could indicate specific configuration based on UE capability reporting.

For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, we need to wait for RAN1 to make down selection among the 5 options.

Observation 1: RAN1 has agreed that OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. And for the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI.

Proposal 2: For CG PUSCH type 1, it is proposed to introduce a new IE (e.g. occ-v19) to include OCC length and OCC sequence related configuration in PUSCH-Config.

Proposal 3: Wait for RAN1 discussion for the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on RAN2 aspects about uplink capacity/throughput enhancement for NR NTN and have the following proposals:

Proposal 1: Introduce 2 new UE capabilities to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory.

Observation 1: RAN1 has agreed that OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. And for the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI.

Proposal 2: For CG PUSCH type 1, it is proposed to introduce a new IE (e.g. occ-v19) to include OCC length and OCC sequence related configuration in PUSCH-Config.

Proposal 3: Wait for RAN1 discussion for the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2.

Reference

- R2-129bis NR-NTN-IoT-NTN (Sergio)_ EOM

- Chair notes RAN1#119 eom0

- Chair notes RAN1#120bis eom0

- 3GPP TS 38.331 V18.3.0 NR; Radio Resource Control (RRC) protocol specification (Release 18)