3GPP TSG RAN WG2 Meeting #130	R2-2504395

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.17

Source:	CMCC

Title:	Support of IoT-NTN TDD mode

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of IoT-NTN TDD mode and some agreements were achieved as follows [1]:

Agreements:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

According to RAN1 discussion progress, the latest agreements are reached as following[2]:

Agreement NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU. - NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be 50ms at the ULSRP. - NOTE: This roughly corresponds to the largest separation between DL and UL Iridium slot pairs. - Note: Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA. Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode, down-select among the following options: - Option 1: DL gaps do not apply for NB-IoT NTN TDD mode. - Option 2: DL gaps apply based on current specifications. - Option 3: DL gaps apply, where new periodicity(ies) are introduced (either implicity e.g. the periodicity is scaled by a fixed factor F, or explicitly) Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc. Working assumption DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is not present in a NB-IoT NTN TDD cell. - NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes. - NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact. Agreement For GNSS measurement gap, RAN1 to further discuss at least the following options: - Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap - Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe - Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

In this contribution, we would like to provide some further considerations on support of IoT-NTN TDD mode.

Discussion

User plane impact

  - 2.1.1 Times with unit of pp (PDCCH periods)

In last RAN2 meeting, we agreed to reuse the existing value ranges of the timers in unit of pp. FFS on the possible clarification to take into account the impact of invalid subframes. Some companies indicate online that the invalid subframes is a general issue for NB-IoT, which is not a IoT-NTN TDD specific issue. However, compared to the new IoT-NTN TDD pattern, there might be much shorter/fewer invalid subframes for legacy NB-IoT. And maybe we could discuss to add some clarification to reduce the impact of invalid subframes. For example, add clarification that it is up to UE implementation to suspend the timers in invalid DL subframes duration and resume when valid subframes occur.

Observation 1: Compared to the new IoT-NTN TDD pattern, there might be much shorter/fewer invalid subframes for legacy NB-IoT.

Proposal 1: Kindly suggest RAN2 to add clarification that it is up to UE implementation to suspend the timers in invalid DL subframes duration and resume when valid subframes occur.

  - 2.1.2 k-Mac

It is supported to extend k-Mac up to 1023ms for IoT-NTN TDD, and FFS extending this to IoT NTN and NR NTN. From our perspective, we need to limit the discussion to the IoT-NTN TDD, and at least for now, there is no need to extend the k-Mac for IoT NTN and NR NTN considering that we have considered both transparent and regenerative architecture related scenarios in R17 and R18 to define the value ranges of k-Mac.

Proposal 2: No need to extend the k-Mac for IoT NTN and NR NTN.

  - 2.1.3 HARQ

And for HARQ of NB-IoT NTN FDD, in R18, we supported DL HARQ feedback disabled to support long propagation delay based on network configuration. For NB-IoT NTN TDD, considering the characteristic of the new TDD structure, for one HARQ process, the interval between a new transmission and a retransmission or two retransmission would be too long. That is a feedback used to determine whether a retransmission is needed may be not important. And we could consider an solution (e.g. add related description in TS 36.300) that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled as default.

Proposal 3: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Control plane impact

  - 2.2.1 System information scheduling

Based on the scope, there are only 8 consecutive DL subframes in the TDD pattern, then SIB1-NB only could be transmitted at most once in one TDD pattern period, which could not even support the repetition mechanism against the poor coverage that some IoT devices deployed. Then for other SIB-NB information scheduled by SIB1-NB, the corresponding SI window may be also affected. As in the current TS 36.331, for NB-IoT UE, the related SI window determination is as following:

- 5.2.3a Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall: - determine the start of the SI-window for the concerned SI message as follows: 2> if the concerned SI message is configured in the schedulingInfoList, schedulingInfoListExt (if present) or if the concerned SI message is configured in the posSchedulingInfoList and si-posOffset is not configured; 3> for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList, schedulingInfoListExt (if present) in SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and posSchedulingInfoList in SystemInformationBlockType1-BR; 3> determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT); 3> if the UE is a NB-IoT UE: 4> the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset); - <<omitted>>

However, for NB-IoT TDD mode, the start calculation of SI-window needs to take the new TDD pattern into account. For example, based on the agreement of RAN1 #120, the downlink subframes within D=8 are fixed to [3 4 5 6 7 8 9 0] (across two consecutive radio frames), which would lead to that the actual calculated subframe #0 may not be used. Then for si-WindowLength, the current maximum value is 1600ms for NB-IoT, which is enough for the new TDD pattern. Therefore we need to discuss the impacts for starting time of SI-window based on the new TDD pattern at least.

Proposal 4: Starting time of SI-window needs to consider the new TDD pattern.

  - 2.2.2 Measurement Gap

Based on the current TS 36.133, in E-UTRA-NR dual connectivity mode, NR - E-UTRA dual connectivity mode and E-UTRA standalone mode, all gap patterns #0~11 in Table 8.1.2.1-1 can be configured  for measurements of NR carriers only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurements of E-UTRA carriers with the applicability as specified in Table 8.1.2.1-1.

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 2 | 3 | 40 | 24NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR |
| 3 | 3 | 80 | 12NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR |
| 4 | 6 | 20 | 120 Note 1 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 5 | 6 | 160 | Note 3 | inter-RAT NR |
| 6 | 4 | 20 | 72 Note 1, 5, 7 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 7 | 4 | 40 | 36 Note 1, 5, 8 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 8 | 4 | 80 | 18Note 1, 5, 9 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 9 | 4 | 160 | Note 3 | inter-RAT NR |
| 10 | 3 | 20 | 48 Note 1, 5 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 11 | 3 | 160 | Note 3 | inter-RAT NR |
| 24 | 10 | 80 | 54Note 1 | Inter-Frequency E-UTRAN FDD and TDD Note 11 |
| 25 | 20 | 160 | Note 3 | Note 11 |
| NOTE 1: When determing UE requirements using Tinter1 for GP2, 3, 4, 6, 7, 8, 10, Tinter1 = 60 for GP2, GP4, GP6, GP7, GP10 and Tinter1 = 30 for GP3, GP8 and GP24 shall be used. NOTE 2: Void. NOTE 3: This gap pattern can be used only for measurement of NR carriers, and Tinter is not applicable. NOTE 4: Void NOTE 5: Void. NOTE 6: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 7: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 48ms corresponding to the first 3ms of the 4ms gap NOTE 8: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 24ms corresponding to the first 3ms of the 4ms gap NOTE 9: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 12ms corresponding to the first 3ms of the 4ms gap NOTE 10: For UE only supporting measGapPatterns-NRonly-r16 for the corresponding gap patterns among GP2-11, the corresponding gap patterns are only applicable to measurement of inter-RAT NR. NOTE 11: Measurement gap patterns #24 and #25 can be requested by NR PCell [38] only when the UE is configured at least with any of RSTD, UE Rx-Tx, or PRS-RSRP measurements requiring such gaps and can only be used during the corresponding positioning measurement period |  |  |  |  |

From our perspective, the parameters MeasurementGap Length and measurement gap repetition period would be affected by the available DL subframes. And based on the current measurement gap configuration, the value range of MeasurementGap Length may be suitable to the new TDD pattern based on network configuration. While for the measurement gap repetition period, some new values could be considered based on the available DL subframes in one new TDD pattern period, for example, 90ms (i.e. corresponding to the period of the new TDD pattern) and multiples of 90ms. Therefore, measurement gap configuration enhancements could be discussed to match the new TDD pattern.

Proposal 5: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on IoT-NTN TDD mode and have the following proposals:

Observation 1: Compared to the new IoT-NTN TDD pattern, there might be much shorter/fewer invalid subframes for legacy NB-IoT.

Proposal 1: Kindly suggest RAN2 to add clarification that it is up to UE implementation to suspend the timers in invalid DL subframes duration and resume when valid subframes occur.

Proposal 2: No need to extend the k-Mac for IoT NTN and NR NTN.

Proposal 3: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Proposal 4: Starting time of SI-window needs to consider the new TDD pattern.

Proposal 5: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Reference

- R2-129bis NR-NTN-IoT-NTN (Sergio)_ EOM

- Chair notes RAN1#120bis eom0

- 3GPP TS 36.300 V18.4.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.4.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)

- 3GPP TS 36.304 V18.3.0 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode (Release 18)

- 3GPP TS 36.133 V18.8.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18)