3GPP TSG-RAN WG2 Meeting #130	R2-2504402

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.6.3

Source:	CMCC

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion

Introduction

At the RAN2#129bis meeting we reached the following agreements [1]:

RAN2 #129bis: Agreements on L1 event triggered MR - Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource. - The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id. - In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to). - In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam. - RAN2 will consider only the following options: - Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} - Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting. Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE. Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE. Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before). Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info) - Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} - Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR. - For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR. - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. - When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE. - When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed. - The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE. - FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

Discussion

Number of beam measurements in the measurement report MAC CE

During the RAN1 120bis meeting, they reached the following conclusions and responded to the LS sent to RAN2.

Agreement Regarding the LS from RAN2 (R1-2501686) the following contents are included in the reply LS. - RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting - Regardless whether or not the report includes the current beam - Note: The maximum number of beams that can be reported by a UE subject to UE capability is under discussion in RAN1 Agreement Reply LS on number of beam measurements in the measurement report MAC CE is agreed. RAN1 would like to thank RAN2 for their LS on number of beam measurements in the measurement report MAC CE. RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting regardless whether or not the report includes the current beam. It is noted that the maximum number of beams that can be reported by a UE is subject to UE capability which is under discussion in RAN1. Final LS in R1-2503077.

From the conclusion, it seems that when the network is configured to report the current beam, the UE can only report 15 other beams. However, based on our understanding of the discussion at the RAN1 meeting at that time, the agreement expressed that the UE can report up to 16 other beams without considering the current beam. Considering that it has been agreed in RAN2 129bis that the current beam does not need RSRI, this saves one byte. Therefore, we recommend that RAN2 clarify that the current beam is not counted in the 16 beams, that is, when the MAC CE carries the measurement results of 16 beams, the UE can carry the current beam measurement results at the end.

Proposal 1: The current beam is not counted in the 16 beams, that is, when the MAC CE carries the measurement results of 16 beams, the UE can carry the current beam measurement results at the end.

LTM2 issue

RAN1 120bis agreed the following conclusion：

Agreement For the RS type determination for event triggered reporting with event LTM2, - Alt 1-2: At least one candidate RS shall be configured

Therefore, RAN2 can follow RAN1's ​​conclusion and make it clear that LTM2 also needs to configure at least one candidate beam resource.

Proposal 2: RAN2 follow RAN1's conclusion and make it clear that LTM2 also needs to configure at least one candidate beam resource.

In addition, when the current beam triggers measurement reporting because it meets the LTM2 conditions, if there are additional available uplink resources, the UE should be allowed to carry the measurement results of the LTM candidate beams in the MR MAC CE.

Proposal 3: If there are additional uplink resources available, the UE should be allowed to carry the measurement results of LTM candidate beams in the LTM2 MR MAC CE.

MR MAC CE issue

At the RAN2#129bis meeting we reached the following agreements of truncated MR MAC CE

Agreements - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. - When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

It should be pointed out that since RAN2 has agreed to the TTT of beam level, when the UE evaluates the triggering condition of a configured event, it is possible that multiple beams meet the triggering condition at the same time. Then, combining these two agreements, a special case should be considered, that is, how to select the reference beam when the UL grant can only carry triggered beams. In our opinion, the simplest way to deal with it is to use the triggered beam with the best quality as the reference beam in the truncated MR MAC CE.

Observation 1: Due to the beam level TTT, for a certain event, there may be multiple beams satisfying the entry/leaving condition at a certain moment.

Proposal 4: When multiple beams meet the triggering condition of a configured event at the same time, they will be included in one MR MAC CE.

Proposal 5: When the UL grant can only carry triggered beams, use the triggered beam with the best quality as the reference beam in the truncated MR MAC CE.

For a certain event, if multiple beams meet the entry/leaving condition at a certain moment, then it is necessary to consider the priority of the entry beam and the leaving beam when the UL grant is insufficient and the truncated MR MAC CE is sent. In our opinion, the entry beam is more important than the leaving beam because it lets the NW know that it is time to switch or early sync. Therefore, when the truncated MR MAC CE is not enough to carry all triggered beams, the beam that meets the entry condition should be reported first.

Proposal 6: When the truncated MR MAC CE is not enough to carry all triggered beams, the beam that meets the entry condition should be reported first.

Dedicated SR issue

In our opinion, if there is no dedicated SR configuration assigned for L1 measurement report MAC CE transmission, then an uplink grant should be requested through the RACH procedure. In addition, similar to the BFR processing in SpCell, the UE should be allowed to carry the MR MAC CE in the transmitted Msg 3.

Proposal 7: If there is no dedicated SR configuration assigned for L1 measurement report MAC CE transmission, then an uplink grant should be requested through the RACH procedure. In addition, similar to the BFR processing in SpCell, the UE should be allowed to carry the MR MAC CE in the transmitted Msg 3.

Support of semi-persistent CSI-RS

In RAN1#120bis meeting, it is agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition.

R1-2503079 LS to RAN2, RAN3: Similarly to the semi-persistent CSI-RS resource supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN1 has also agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 As captured in the agreement, RAN1 sees the spec impact in RAN2 to define the MAC CE. Also, RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case.

For gNB scheduled L1-RSRP measurement and reporting, compared with periodic CSI-RS, semi-persistent CSI-RS can be activated and deactivated flexibly for UE to measure the CSI-RSs for a subset of candidate cells, which could reduce the measurement and report overhead for UE.

The signaling procedure can be summarized as follows:

Fig.1 Activation procedure

First, the serving cell sends an activation request for the semi-persistent CSI-RS resource to the candidate cell. Upon receiving the request, the candidate cell starts transmitting the corresponding semi-persistent CSI-RS according to the RRC configuration and sends a success confirmation back to the serving cell. Finally, after receiving the successful activation confirmation, the serving cell notifies the UE via a dedicated MAC CE, allowing it to begin the corresponding semi-persistent CSI-RS measurements.

Fig.2 Deactivation procedure

For deactivation, the serving cell does not need to wait for the candidate cell's feedback before notifying the UE to deactivate the semi-persistent CSI-RS resource. The process is considered complete once the serving cell receives the successful deactivation confirmation from the candidate cell.

It is worth discussing whether the activation/deactivation of the candidate cell semi-persistent CSI-RS resource should be performed per cell or per resource set. Since the network cannot know in advance which candidate cell the UE will switch to, the network may need to activate the semi-persistent CSI-RS resource of multiple LTM candidate cells at a certain moment, and vice versa. Therefore, a resource set containing multiple LTM candidate cells may be a better configuration granularity, similar to the CSI-RS resource configuration in current RRC running CR as shown below.

Proposal 8: Activate/deactivate semi-persistent CSI-RS resource based on the resource set containing multiple LTM candidate cells.

Conclusions

In this contribution, we analyse for measurement event evaluation, following are the observations and proposals.

Observations:

Observation 1: Due to the beam level TTT, for a certain event, there may be multiple beams satisfying the entry/leaving condition at a certain moment.

Proposals:

Proposal 1: The current beam is not counted in the 16 beams, that is, when the MAC CE carries the measurement results of 16 beams, the UE can carry the current beam measurement results at the end.

Proposal 2: RAN2 follow RAN1's conclusion and make it clear that LTM2 also needs to configure at least one candidate beam resource.

Proposal 3: If there are additional uplink resources available, the UE should be allowed to carry the measurement results of LTM candidate beams in the LTM2 MR MAC CE.

Proposal 4: When multiple beams meet the triggering condition of a configured event at the same time, they will be included in one MR MAC CE.

Proposal 5: When the UL grant can only carry triggered beams, use the triggered beam with the best quality as the reference beam in the truncated MR MAC CE.

Proposal 6: When the truncated MR MAC CE is not enough to carry all triggered beams, the beam that meets the entry condition should be reported first.

Proposal 7: If there is no dedicated SR configuration assigned for L1 measurement report MAC CE transmission, then an uplink grant should be requested through the RACH procedure. In addition, similar to the BFR processing in SpCell, the UE should be allowed to carry the MR MAC CE in the transmitted Msg 3.

Proposal 8: Activate/deactivate semi-persistent CSI-RS resource based on the resource set containing multiple LTM candidate cells.

References

- Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES.