3GPP TSG-RAN WG2 Meeting #130	R2- 2504514

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	LP-WUS in RRC_CONNECTED

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

So far, RAN2 discussed the LP-WUS in RRC_CONNECTED aiming at supporting RAN1 decision with minimal specification impact. At RAN2#129bis, RAN2 made the following agreements with some remaining FFS:

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used.

Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback.

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously.

If configured, the UE can signal a preferred time offset via UAI signalling.

Ask RAN1 for further information regarding their conclusions.

FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

In this contribution, we discuss the remaining issues including FFS from the last meeting. Also, we think some of the uplink aspects need to be clarified or confirmed from RAN2 perspective since we see that LP-WUS is not totally the same as DCP due to separate receiver, i.e., MR and LR.

# 2	Discussions

## 2.1	Secondary DRX group

It remains FFS whether and how to support LP-WUS in conjunction with dual DRX group. At RAN2#129bis, two options were briefly discussed:

Option 1: LP-WUS is configured only on PCell. LP-WUS indication wakes up all serving cells of all DRX groups at once.

Option 2: LP-WUS is configured separately per DRX group, e.g., on PCell in primary DRX group and/or an activated SCell on secondary DRX group. LP-WUS indication wakes up all serving cells within the relevant DRX group.

Option 1 offers simple implementation, as LP-WUS is commonly applied to both DRX groups. However, this simplicity comes at the cost of limited power-saving benefits because the UE wakes-up on all cells simultaneously and the UE monitors LP-WUS only after the active time ends on all cells, which undermines the advantages of dual DRX grouping.

Option 2 assumes that the UE independent monitors PDCCH and LP-WUS for each DRX group, enabling selective wake-up. For example, FR2 cells in secondary DRX group may remain inactive, i.e., monitoring LP-WUS for FR2 cells, while FR1 cells in primary DRX group are active, i.e., monitoring PDCCH. This also allows FR2 cells to enter DRX inactive state earlier than FR1 cells. However, this requires multiple LP-WUS configurations for a UE, and the agreements from RAN1#120bis restrict LP-WUS configuration to PCell/PSCell:

Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group

Although these agreements are limited to configuration without dual DRX groups, they reflect RAN1’s preference for simplicity. Furthermore, there was no consensus in RAN1 on supporting LP-WUS with dual DRX group, due to differing views on its configuration and interaction with DRX.

While option 2 is preferable from power saving perspective, it requires RAN1 confirmation – specifically, whether the UE can monitor PDCCH for the primary DRX group while simultaneously monitoring LP-WUS for the secondary DRX group.

In contrast, option 1 minimizes the specification impact and simplifies the implementation by enabling one-shot handling of both DRX groups. It may be argued that, under option 1, the gNB cannot initiate scheduling on one DRX group that is not in DRX active time by sending LP-WUS, as long as the other DRX group remains in active time due to on-going PDCCH monitoring. However, in generally, the downlink traffic is not highly latency-sensitive, and the gNB can continue DL transmission using the cells on the currently active DRX group. Therefore, we do not see this as a critical limitation of option 1. Considering the time constraints and implementation simplicity, we propose to proceed with Option 1.

Proposal 1: LP-WUS can be configured with secondary DRX group.

Proposal 2: When a secondary DRX group configured, the LP-WUS is configured only on the PCell.

Proposal 3: With a secondary DRX group, the UE starts PDCCH monitoring in both primary and secondary DRX group upon receiving LP-WUS on the PCell.

Proposal 4: With a secondary DRX group, the UE starts LP-WUS monitoring when active time ends in both primary and secondary DRX groups.

One remaining question is whether lpwus-PDCCHMonitoringTimer can be separately configured for the primary and secondary DRX groups. Considering that drx-InactivityTimer is also separately configured for the secondary DRX group, it seems reasonable to allow separate configuration of lpwus-PDCCHMonitoringTimer as well.

Proposal 5: With secondary DRX group, lpwus-PDCCHMonitoringTimer can be separately configured for the secondary DRX group.

## 2.2	Uplink aspect

### 2.2.1	SR and LP-WUS

For both option 1-1 and 1-2, it is assumed that the UE does not monitor LP-WUS during C-DRX active time. This raises the question of whether the UE can automatically transition to active time by uplink traffic, such as when SR is pending, or the UE should continue waiting for LP-WUS from the network.

When SR is pending, the network may not yet be aware that the UE has entered C-DRX active time. As a result, the network might send LP-WUS for downlink scheduling, which would be missed if the UE has stopped monitoring of LP-WUS. Only after the gNB receives the SR, will it stop sending LP-WUS and schedule the UE via PDCCH.

Although downlink latency is generally considered non-critical, the potential failure of the initial SR transmission may motivate to maintain LP-WUS monitoring until the gNB successfully receives the SR and responds – the UE starts monitoring of PDCCH based on the new timer started by LP-WUS received SR is pending. It would imply also that the SR is responded by LP-WUS instead of PDCCH, which would need to be clarified in the specification.

Observation 1: If the UE monitors LP-WUS instead of PDCCH while SR is pending, reception of LP-WUS would need to be considered as a response to SR.

Alternatively, the UE could monitor both LP-WUS and PDCCH while the SR is pending; however, the feasibility of this may depend on a decision by RAN1.

Observation 2: Monitoring both LP-WUS and PDCCH simultaneously while SR is pending would need to be confirmed by RAN1.

In our view, the simplest solution is to begin PDCCH monitoring when SR is pending, as is done in legacy, unless there is good reason to keep the UE’s C-DRX active time synchronized with the gNB. Our understanding is that the current MAC running CR is already based on this principle and we see no specification impact.

Proposal 6: RAN2 confirms that the UE stops LP-WUS monitoring when SR is pending.

### 2.2.2	CG/SPS

It has not yet been discussed whether the MR keeps the SPS and CG activated or deactivated while the UE monitors the LP-WUS. One might assume it is straightforward to remain SPS/CG operation as it is, but the question is whether the UE can do it – since PDSCH monitoring is done by MR and LP-WUS monitoring is done by LR.

If SPS and CG are kept activated, transmission or reception of MAC PDU on SPS/CG will start the corresponding timers, i.e., drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL, which might then keep the MR awake. To save more power, it might be preferred to deactivate SPS and CG. As it is possible to deactivate SPS and CG by PDCCH and activation/deactivation MAC CE, respectively, it seems not necessary to define additional deactivation mechanism such as the UE by itself deactivate the SPS or CG when starting LP-WUS monitoring.

Proposal 7: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.

# 3	Conclusion

We discussed the secondary DRX group aspect and untouched issues for uplink, and proposed the followings:

Proposal 1: LP-WUS can be configured with secondary DRX group.

Proposal 2: When a secondary DRX group configured, the LP-WUS is configured only on the PCell.

Proposal 3: With a secondary DRX group, the UE starts PDCCH monitoring in both primary and secondary DRX group upon receiving LP-WUS on the PCell.

Proposal 4: With a secondary DRX group, the UE starts LP-WUS monitoring when active time ends in both primary and secondary DRX groups.

Proposal 5: With secondary DRX group, lpwus-PDCCHMonitoringTimer can be separately configured for the secondary DRX group.

Observation 1: If the UE monitors LP-WUS instead of PDCCH while SR is pending, reception of LP-WUS would need to be considered as a response to SR.

Observation 2: Monitoring both LP-WUS and PDCCH simultaneously while SR is pending would need to be confirmed by RAN1.

Proposal 6: RAN2 confirms that the UE stops LP-WUS monitoring when SR is pending.

Proposal 7: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.