3GPP TSG-RAN WG2 Meeting #130	R2-2504557

St.Julians, Malta,  May 19th – 23rd , 2025

Source:                Qualcomm Incorporated

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# Discussion

2.1 Exit conditions

In RAN2 #128 meeting, there is FFS on exist condition as following. There were some discussion in RAN2#129bis meeting, but no conclusion is achieved.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

If RRM relaxation exit shares the same condition with entry, then there could be a ping-pong switching between entry and exit. To avoid this ping-pong issue, it is preferred that a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation and offloading exit condition.

- To avoid ping-pong switching, a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation and offloading exit condition.

For RRM relaxation mode, one open issue is whether both MR and LR threshold to be configured to the UE. Network can configure both of MR and LR threshold as RRM relaxation mode exit condition, and when either MR or LR measurement is below the corresponding threshold, UE should exit from RRM relaxation mode.

- Network can configure both of MR and LR threshold as RRM relaxation mode exit condition.

- When any of LR or MR measurement is below a threshold configured for exit condition, UE exits from RRM relaxation mode and enters legacy measurement mode.

## 2.3 Legacy neighboring measurement relaxation criterion

Currently, for intra-frequency measurement, in order to save UE power, a serving cell quality threshold (SIntraSearchP/SIntraSearchQ) are introduced for trigger intra-frequency measurement. When in relaxed measurement mode to LP-WUS, UE will measure using both of MR and LR, but the MR measurement is in a relaxed way. That means there are two measurement results. One issue is which measurement result will be used to trigger intra-frequency measurement. One way is using existing serving cell MR measurement, but serving cell measurement is in a relaxed way, the measurement accuracy could not be enough to evaluate the condition; another way is to introduce LP-WUS specific parameters.

Similarly, for inter-frequency measurement relaxation, SIntraSearchP/SIntraSearchQ)  the criteria to determine ‘not at cell edge’, whether MR measurement or LR measurement is used needs to be determined.

Currently, for intra-frequency measurement, in order to save UE power, a serving cell quality threshold (SIntraSearchP/SIntraSearchQ/SnonIntraSearchP/SnonIntraSearchQ) are introduced for trigger intra-frequency measurement and inter-freqeuncy measurement of equal or lower priority, or inter-RAT frequency cells of lower priority . When in relaxed measurement mode to LP-WUS, UE will measure using both of MR and LR, but the MR measurement is in a relaxed way. That means there are two measurement results. One issue is which measurement result will be used to trigger intra-frequency measurement. One way is using existing serving cell MR measurement, but serving cell measurement is in a relaxed way, the measurement accuracy could not be enough to evaluate the condition; another way is to introduce LP-WUS specific parameters.

Similarly, for the criteria to determine ‘not at cell edge’ in Rel-16 neighboring measurement relaxation, whether MR measurement or LR measurement is used needs to be determined.

- Reuse existing MR serving cell measurement criterion, i.e. SIntraSearchP/SIntraSearchQ/SnonIntraSearchP/SnonIntraSearchQ) to trigger skipping measurement of intra-frequency and measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

- Reuse existing MR serving cell measurement criterion for Rel-16 “not at cell edge” evaluation for neighboring measurement relaxation.

## 2.4 Merge LP-WUS monitoring and RRM relaxation conditions

## In last RAN2 meeting, there were discussion on how to merge LP-WUS and RRM relaxation conditions, and two options are on the table.

      - Direction 3a. merge LPWUS monitoring and serving cell offloading

      - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

If merging LP-WUS monitoring and serving cell offloading entry/exit condition, then LP-WUS monitoring usage could be a limited scope; so merging LP-WUS monitoring and Rel-19 relaxation is preferable.

- Merge LPWUS monitoring and Rel-19 RRM relaxation entry/exit condition.

# Conclusion

- To avoid ping-pong mode switching, a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation/offloading exit condition.

- Network can configure both of MR and LR threshold as RRM relaxation mode exit condition.

- When any of LR or MR measurement is below a threshold configured for exit condition, UE exits from RRM relaxation mode and enters legacy measurement mode.

- Reuse existing MR serving cell measurement criterion, i.e. SIntraSearchP/SIntraSearchQ/SnonIntraSearchP/SnonIntraSearchQ) to trigger skipping measurement of intra-frequency and measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

- Reuse existing MR serving cell measurement criterion for Rel-16 “not at cell edge” evaluation for inter-frequency measurement relaxation.

- Merge LPWUS monitoring and Rel-19 RRM relaxation entry/exit conditions.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)