3GPP TSG-RAN WG2 Meeting #130	R2-2504559

St.Julians, Malta,  May 19th – 23rd , 2025

Source:                Qualcomm Incorporated

Title:	SON and MDT for LTM

Agenda Item:	8.10.3

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses UE capabilities for Rel-19 SON/MDT.

# Discussion

In Rel-19 SON/MDT, several features were discussed have progress, the following table sumarizes the contents.

| Features | Content |
| --- | --- |
| LTM | - RLF report for failure case, e.g. L1 measurement, cell ID, cause, RACH info, time info. - SHR report for near failure case, e.g L1 measurement, cell ID, cause, RACH info, time info. |
| CHO with candidate SCG | - RLF/ SCGFailureInformation report for CHO with candidate SCG - SHR/SPR report for CHO with candidate SCG |
| Subsequent CPAC | - TBC (no progress in RAN2) |
| NTN | - RLF report for time/location-based CHO - Area scope checking for MDT |
| Slicing | - TBC (no progress in RAN2) |
| Rel-18 leftover | - RA-report for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for EN-DC SCG failure |

For each feature, it should be enough that one UE capability is for one feature, that means no need to distinguish UE capabilities for RLF report and SHR/SPR report for MRO. So it is proposed that separate UE capabilities are introduced for different SON/MDT features.

- Introduce UE capability for SON/MDT for LTM.

- Introduce UE capability for SON/MDT for CHO with candidate SCGs.

- Introduce UE capability for RLF report for time/location-based CHO in NTN.

- Introduce UE capability on RA-report for SDT.

- Introduce UE capability on MHI enhancement for SCG deactivation/activation.

- Introduce UE capability on MRO for EN-DC SCG failure.

Whether those UE capabilities are needed to be reported depends on whether gNB needs to know UE capabilities. So far, there is no special configuration introduced for these features, then it can be FFS on whether radio access signalling is needed for above UE capabilities.

- FFS whether radio access signalling is needed for above UE capabilities.

In last RAN2 meeting, RAN2 agreed to introduce a UE capability to support geographic area scope checking, but it is FFS if with or without signalling.

- Introduce a UE capability to support geographic area scope checking. FFS if with or without signalling.

Since RAN2 agreed to use dedicated signalling to provide area scope configuration to the UE, then it is needed to let the gNB know whether UE can support geographic area scope checking. UE needs to report the capability to the gNB.

- UE reports the capability of  supporting geographic area scope checking to the gNB.

# Conclusion

- Introduce UE capability for SON/MDT for LTM.

- Introduce UE capability for SON/MDT for CHO with candidate SCGs.

- Introduce UE capability for RLF report for time/location-based CHO in NTN.

- Introduce UE capability on RA-report for SDT.

- Introduce UE capability on MHI enhancement for SCG deactivation/activation

- Introduce UE capability on MRO for EN-DC SCG failure

- FFS whether radio access signalling is needed for above UE capabilities.

- UE reports the capability of  supporting geographic area scope checking to the gNB.

# Reference