3GPP TSG RAN WG2 Meeting #130     													         R2-2504604

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda item:		8.5.3

Title:						Reducing On-demand SIB1 overhead

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The Release 19 WI for Network Energy Saving (NES) was revised in RAN#105 [1], where the specification of the studied case 2 for on-demand SIB1 was agreed.  The latest agreements are listed in [2].

One of the remaining issues of on-demand SIB1 is to complete the design of the signaling. This contribution discusses the aspect of overhead reduction for sending the UL WUS configuration on the Cell A.

# Reduction of UL WUS Configuration Overhead

In Rel-19 on-demand SIB1 operation, one cell (referred as Cell A) will carry UL WUS configuration for many NES cells. While the overhead for a single NES cell could be manageable, repeating this overhead for each NES cell may lead to a situation where the energy consumption of Cell A increases significantly. Furthermore, the amount of NES cells which could be supported depends directly on the overhead, as there are limits to the number of bits carried on SIBs. For these reasons, it is very important to reduce the UL WUS configuration overhead.

Observation 1: It is very important to reduce UL WUS configuration overhead in on-demand SIB1.

During email discussions [3], some points were identified to guide the remaining issues to complete on-demand SIB1 design. A core issue is the optionality of parameters in SIBxx which will carry the UL WUS configuration:

5.2.2.5 Editors notes: FFS if SIBxx is essential system information SIBxx Editor’s note: Only parameters in R1-2501645 that are in own rows are implemented and not all listed e.g. in cell 17P or 21P. FFS to group some parameters under subIEs like frequencyInfoUL FFS to separate IE UL-WUS as own IE, for review purposes it is here now. FFS: value for maxCells, maxSIB1-Message FFS: optionality of the parameters as there was no input on this FFS: “UL WUS transmission” in the field descriptions is not defined(it is used in the L1 input excel) FFS: Whether the UL-WSU configuration applies to one cell or several cells

Note, however that the parameters sent on UL-WUS configuration are to a great extent a subset of the parameters of SIB1. Therefore, it is possible to save a lot of the overhead with the following approach: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration must be sent on SIBxx by the gNB. The UE shall use the same parameters from Cell A SIB1 for any parameters of SIBxx which were not sent on SIBxx.

Proposal 1: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration must be sent on SIBxx by the gNB. The UE shall use the same parameters from Cell A SIB1 for any parameters of SIBxx which were not sent on SIBxx.

By using this approach, the overhead can be reduced drastically. Many configuration parameters are shared among different cells and only those which will differ would be broadcast. This would lead to better energy savings as well as supporting more NES cells per Cell A.

# Conclusions

In this contribution, we discussed some aspects of on-demand SIB1 for NES. The following observations and proposals have been made:

Observation 1: It is very important to reduce UL WUS configuration overhead in on-demand SIB1.

Proposal 1: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration must be sent on SIBxx by the gNB. The UE shall use the same parameters from Cell A SIB1 for any parameters of SIBxx which were not sent on SIBxx.

# References

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), 3GPP TSG RAN Meeting #105	 Melbourne, Australia, September 2024.

- R2-2502981, “Report from session on R18 SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), 3GPP TSG-RAN WG2 Meeting #129bis, Wuhan, China, April, 2025

- R2-2502150, “Summary of Comments to 38.331 CR for NES”, Ericsson (Rappertour), 3GPP TSG-RAN WG2 Meeting #129bis, Wuhan, China, April, 2025