3GPP TSG-RAN WG2 #130		                                      R2-2504693

Malta, MT, 19 - 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2087 | rev | 1 | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IAB related MAC CEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Samsung, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2025-5-9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R17, serveral MAC CEs are introduced for IAB which are sent from the parent node to an IAB-node, including Child IAB-DU Restricted Beam Indication, DL TX Power Adjustment and Timing Case Indication MAC CEs, based on TS 38.300 and the RAN1 LS as copied below. And according to the definition in TS 38.300, the parent node can be an IAB-node or IAB-donor-DU. R1-2202947 LS on upper layers parameters for Rel-17 eIAB Child IAB-DU Restricted Beam Indication：Signaling from an IAB-node/IAB-donor to a child node indicating beams of the child IAB-DU in the direction of which simultaneous operation is restricted. DL TX Power Adjustment: The parent-node indicates to the IAB-node an adjustment to the parent-node’s DL TX power (e.g., in response to receiving Desired DL TX Power Adjustment from the IAB-node). Timing Case Indication:The parent-node indicates to an IAB-node a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot).. TS 38.300： Parent node: IAB-MT's or mobile IAB-MT's next hop neighbour node; the parent node can be an IAB-node or IAB-donor-DU ...... Correspondingly, the parent node can provide information via MAC-CE to the IAB-node to facilitate enhanced multiplexing at the IAB-node and/or at the parent node: - restricted IAB-DU Tx beams; - actual parent node's IAB-DU Tx power adjustment; - IAB-MT's uplink transmission timing mode.. However, the case where IAB-donor-DU sends these three MAC CEs to its child IAB-MT is missing in current TS 38.321. Moreover, for the Child IAB-DU Restricted Beam Indication MAC CE, it indicates restricted beams of the child IAB-DU and/or child IAB-MT. However, the current text implies that it indicates the restricted beams of the parent IAB node, which is incorrect. |
| Summary of change: |  | - Add the case where IAB-donor-DU sends Child IAB-DU Restricted Beam Indication, DL TX Power Adjustment and Timing Case Indication MAC CEs to its child IAB-MT. - Clarify that Child IAB-DU Restricted Beam Indication MAC CE indicates restricted beams of child IAB-DU and child IAB-MT. - Some editorial corrections. Impact analysis: Impacted 5G architecture options: NR SA, (NG)EN-DC, NR-DC Impacted functionality: IAB-MT Inter-operability: No inter-operability issue is identified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The case where IAB-donor-DU sends Child IAB-DU Restricted Beam Indication, DL TX Power Adjustment and Timing Case Indication MAC CEs to its child IAB-MT is missing in current TS 38.321. - The Child IAB-DU Restricted Beam Indication MAC CE indicates the restricted beams of the parent IAB node, which is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.26, 5.18.27, 5.18.29, 6.1.3.39, 6.1.3.63 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | - v1 - Add “Impacted 5G architecture options” in the cover page, and re-submit to RAN2#130 meeting. |
| --- | --- |

FIRST CHANGE

5.18.26	Restricted and recommended beam indication for IAB

Child IAB-DU Restricted Beam Indication MAC CE is used by an IAB-DU or an IAB-donor-DU to indicate to its child node spatial resources and associated frequency information where simultaneous transmission/reception from the IAB-MT of the child node and transmission from the IAB-DU cells of the child node is restricted. IAB-MT Recommended Beam Indication MAC CE is used by an IAB-node to indicate to its parent node recommendations for spatial resources and associated frequency information for transmission/reception from and to the IAB-MT of the IAB-node. Time resources where these restrictions/recommendations can apply are indicated via RRC, while the MAC CE further selects from these time resources the specific time resource configuration (comprising a list of slot indices) to which information provided in the MAC CE applies.

Upon reception of a Child IAB-DU Restricted Beam Indication MAC CE the IAB-node shall:

-	apply the configuration signalled in the MAC CE to the time slots indicated in IAB-ResourceConfig (as specified in TS 38.331 [5]) which contains iab-ResourceConfigID parameter which matches the Resource Configuration ID field of the MAC CE.

The MAC entity may:

1>	if an IAB-MT Recommended Beam Indication query has not been triggered:

2>	trigger an IAB-MT Recommended Beam Indication query for this Serving Cell.

If the MAC entity has UL resources allocated for new transmission the MAC entity shall:

1>	for each IAB-MT Recommended Beam Indication query that has been triggered and not cancelled:

2>	if the allocated UL resources can accommodate a IAB-MT Recommended Beam Indication MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure to generate the IAB-MT Recommended Beam Indication MAC CE;

3>	cancel this IAB-MT Recommended Beam Indication query.

Next Change

5.18.27	DL TX power adjustment for IAB

DL TX Power Adjustment MAC CE is used by an IAB-DU or an IAB-donor-DU to indicate to its child node spatial resources and associated frequency information where the DL TX power adjustment contained in the MAC CE applies. Desired DL TX Power Adjustment MAC CE is used by an IAB-node to indicate to its parent node recommendations for such a restriction. Time resources where these restrictions/recommendations can apply are indicated via RRC, while the MAC CE further selects from these time resources the specific time resource configuration (comprising a list of slot indices) to which information provided in the MAC CE applies.

Upon reception of a DL TX Power Adjustment MAC CE the IAB-node shall:

-	apply the configuration signalled in the MAC CE to the time slots indicated in IAB-ResourceConfig (as specified in TS 38.331 [5]) which contains iab-ResourceConfigID parameter which matches the Resource Configuration ID field of the MAC CE.

The MAC entity may:

1>	if a Desired DL TX Power Adjustment query has not been triggered:

2>	trigger a Desired DL TX Power Adjustment query for this Serving Cell.

If the MAC entity has UL resources allocated for new transmission the MAC entity shall:

1>	for each Desired DL TX Power Adjustment query that has been triggered and not cancelled:

2>	if the allocated UL resources can accommodate a Desired DL TX Power Adjustment MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure to generate the Desired DL TX Power Adjustment MAC CE;

3>	cancel this Desired DL TX Power Adjustment query.

Next Change

5.18.29	Timing case indication for IAB

Timing Case Indication MAC CE is used by an IAB-DU or an IAB-donor-DU to indicate to its child node timing modes which should apply to time resources indicated via RRC.

Upon reception of a Timing Case Indication MAC CE the IAB-node shall:

-	apply the configuration signalled in the MAC CE to the time slots indicated in IAB-ResourceConfig which contains iab-ResourceConfigID parameter (as specified in TS 38.331 [5]) which matches the Resource Configuration ID field of the MAC CE.

Next Change

6.1.3.39	Case-6 Timing Request MAC CE

The Case-6 Timing Request MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-2b. This MAC CE is used by the IAB-MT of the node to inform its parent node whether Case-6 timing is required for simultaneous operation.

It has a fixed size of zero bits.

Next Change

6.1.3.63	DL TX Power Adjustment and Desired DL TX Power Adjustment MAC CEs

The DL TX Power Adjustment MAC CE and Desired DL TX Power Adjustment MAC CE are identified by MAC subheaders with eLCIDs as specified in Table 6.2.1-1b and Table 6.2.1-2b, respectively. Otherwise the format of the two MAC CEs is identical. Each has a variable size with following fields (Figure 6.1.3.63-1):

End of Changes