3GPP TSG-RAN WG2 Meeting #131	R2-2505117

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 Discussion on Event-triggered L1 reporting

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report.

For R19 mobility enhancements, one of the goals outlined in the WID is to define the required improvements for enabling event-triggered L1 measurements.

RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

In this contribution, we provide a comprehensive analysis of event-triggered L1 measurement reports, covering the discussion of the remaining open issues listed the previous online/offline discussion for the key aspects.

2	Discussion

2.1	Semi-persistent CSI-RS measurement for LTM

| Agreements on L1 measurement triggered MR 1. UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell. |  |  |
| --- | --- | --- |
| [CB session]: [Apple]: RAN1 has not made any decision regarding whether early CSI acquisition using SP-CSI resource is one-shot or is continued after LTM cell switch completion. We can postpone it to the next meeting. [Ericsson, ZTE]: Shouldn’t we distinguish two separate SP-CSI resources for RRM measurement and early CSI acquisition purposes? [Apple]: It is not decided in RAN1. Most likely we can use same SP-CSI resources for both purposes.  Postpone target cell case to the next meeting. |  |  |
| MAC-25 | Handling of the activated SP CSI-RS for target cell after LTM cell switch Editor’s NOTE: After reconfiguration with sync that is triggered by LTM, it is FFS how to handle the configured Semi-persistent CSI-RS/CSI-IM resource sets for target cell. Based on the following agreement: UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell | Issue Type: Essential How to address it: based on companies’ contribution |

In the previous RAN1 meeting, it was agreed that the UE should deactivate the semi-persistent CSI-RS resources of candidate cells (other than the target cell) after the LTM cell switch to avoid introducing extra overhead for both the UE and the network. However, it still FFS whether the activated SP CSI-RS resource of the target cell should remain activated after the LTM cell switch.

In RAN1, the introduction of semi-persistent CSI-RS/CSI-IM resources aims to support early CSI acquisition. For the target cell, the UE can continue measuring CSI-RS/CSI-IM resources associated with the configured CSI reporting after receiving the LTM CSC, provided that CSI acquisition is configured for the target cell. The UE can then transmit the latest measurement results of the SP CSI-RS/CSI-IM resources to the target cell.

Therefore, in alignment with the intention of RAN1, the UE should continue measuring the activated SP CSI-RS resources for the target cell after the LTM cell switch procedure.

Proposal 1: (MAC-25) The SP CSI-RS resource of the target cell remains activated after cell switch if the SP CSI-RS was activated before cell switch.

2.2	Truncated Measurement report MAC CE

| MAC-24 | It is still working assumption on the details for truncated L1 MR MAC CE Editor’s NOTE: It is working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

Based on the discussion in the last meeting, there is a working assumption that the minimum size of the truncated MR MAC CE should at least include {one triggered beam + the current serving beam}, in cases where the UE includes the current serving beam in the truncated MR MAC CE, when the NW configures it to do so.

Regardless of the triggered candidate beam, the RSRP value of the current serving beam is also essential for the NW to make the LTM cell switch decision. If the beam quality of the current serving beam is not poor, the NW may not immediately send the LTM CSC MAC CE to trigger the cell switch. Therefore, the truncated MR MAC CE should at least include both the one triggered beam and the current serving beam.

Proposal 2: (MAC-24) The working assumption should be confirmed: The minimum size of the truncated MR MAC CE should at least include {one triggered beam + the current serving beam}.

Due to the running MAC CR, it has been specified that the event-triggered L1 MR and truncated event-triggered L1 MR share a unified MAC CE format. However, in the MAC CE format of the truncated event-triggered L1 MR, it should at least include one triggered beam and the current serving beam if configured by the network, while in the non-truncated MAC CE, the current serving beam is placed at the bottom. If UE follows current running MAC CR, it will not include the current beam in the truncated MAC CE, which contradicts to the previous agreement of unified format for both truncated and non-truncated MAC CE format.

To solve this issue, we can add descriptive text explaining the placement of the current serving beam and other beams in the event-triggered L1 report MAC CE, as shown below:

- RSRIi: This field indicates the reference signaling resource index of the beam i of LTM candidate cell for the event triggered L1 measurement report (i.e. SS/PBCH Block Resource indicator (SSBRI) or CSI-RS resource indicator (CRI)). The maximum number of non-serving RS reported is configured by maxNumberOfReportedBeams if the measurement of current RS of serving cell is not included, or is maxNumberOfReportedBeams-1 if the measurement of current RS of serving cell is included. The first RS is the RS with the highest measured quality for non-serving RS in this measurement report MAC CE. At least one triggered beam should be present in the truncated MAC CE. The length of the RSRI index field is 9 bits; - RSRP1: This field indicates the measured quantity of the first beam based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) as described in TS 38.215 [24]. This field is optionally present in the truncated MAC CE. The length of the RSRP1 field is 7 bits; - DiffRSRPi: This field indicates the derived differential measured quantity for the beam i of LTM candidate cell based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) as described in TS 38.215 [24], with the reference of measured quality of the first beam. This field is optionally present in the truncated MAC CE. The length of the DiffRSRPi field is 4 bits; - RSRPserving: This field indicates the measured quantity based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) for current RS of serving cell as described in TS 38.215 [24], if UE is configured to report the measurement result of current RS of the serving cell by reportCurrentBeam. This field should be present in the truncated MAC CE. The length of the RSRPserving field is 7 bits; - R: Reserved bit, set to 0.

Based on the above description, the current serving beam result can be placed at the bottom of the truncated MAC CE while remaining in the same position in the non-truncated MAC CE. Regarding UE behavior, the UE generates the non-truncated MAC CE after receiving the UL grant, rather than first generating a non-truncated MAC CE and then truncating it after receiving the UL grant as specified in the current MAC specification. Thus, this approach appears unlikely to increase UE complexity significantly based on the current UE behaviors. This solution may neither require reverting previous agreements nor modifying the unified MAC CE format in the current MAC running CR, representing the simplest implementation method.

Proposal 3: (MAC-24) A unified format should be used for both the truncated MR MAC CE and non-truncated one. Descriptive text explaining the placement of the current serving beam and other beams should be added in the event-triggered L1 report MAC CE as shown above.

# 3	Conclusion

In this paper, we have addressed the objective regarding improvements in event-triggering reports for LTM. There are the following observations and proposals:

Semi-persistent CSI-RS measurement for LTM

Proposal 1: (MAC-25) The SP CSI-RS resource of the target cell remains activated after cell switch if the SP CSI-RS was activated before cell switch.

Trunacted measurement report MAC CE

Proposal 2: (MAC-24) The working assumption should be supported: The minimum size of the truncated MR MAC CE should at least include {one triggered beam + the current serving beam}.

Proposal 3: (MAC-24) A unified format should be used for both the truncated MR MAC CE and non-truncated one. Descriptive text explaining the placement of the current serving beam and other beams should be added in the event-triggered L1 report MAC CE as shown above.

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #120 chairman notes;

- R3-253084, (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC