3GPP TSG-RAN WG2 Meeting #131	R2-2505259

Bangalore, India, Aug 25th – 29th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues for PWS support

Agenda Item:	8.9.4

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. In RAN#105, the WID is further refined in [1] and a new objective for PWS support has been added as below:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: <skip the unrelated part> - Support of Capacity enhancements for uplink <skip the unrelated part> - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In RAN#107, the WID is further refined in [2] and a note was added “PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported”. According to this clarification for WID objective, it can be seen that the NB-IoT UEs cannot acquire PWS notifications when it’s in connected mode.

For this objective, in RAN2#127bis meeting, RAN2 focus the discussion on new SIBs for PWS and new PWS indication in direct indication information, and achieved the following agreements:

- RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS - The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. - For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields): - SystemInformationBlockType10-NB for primary ETWS notification; - SystemInformationBlockType11-NB for secondary ETWS notification; - SystemInformationBlockType12-NB for CMAS notification. - Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB): - etws-Indication; - cmas-Indication. - Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means) - Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In RAN2#128 meeting, the following agreements were achieved:

- We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - Send a LS to RAN1 asking whether it’s possible to specify support for PWS reception in RRC_CONNECTED for NB-IoT in Rel-19.

In RAN2#129 meeting, the following agreements were achieved:

- Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells - Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information. - Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB - We don’t introduce a new SIB for carrying geographical information for emergency areas

In RAN2#129bis meeting, the following agreements were achieved:

- In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

In RAN2#130 meeting, the following agreements were achieved:

- A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT) - Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case.

In this document, we will discuss the remaining issues for PWS support and give our proposals.

Discussion

  - RRC open issues

2.1.1 Inter-cell PWS reception for intra-eNB scenario

In last meeting, RAN2 has agreed not to discuss continued reception of PWS segmentation of a message from different cells, except for intra-eNB case. One of the reason is that, for other cases, e.g., inter-eNB case, the different eNBs likely have different segmentation rules for the PWS, so it’s not feasible/easy or maybe wrong for UE to assemble PWS segments from different cells in different eNBs. For intra-eNB case, this issue may not exist as the cells within a same eNB can have some internal coordination.

Even for intra-eNB case, there is no explicit agreement on whether and how to support inter-cell PWS reception. Further discussion is needed. According to the current specification description as following:

5.2.2.4 System information acquisition by the UE <skip the unrelated parts> 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11; 2> when the UE acquires SystemInformationBlockType1 following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> if schedulingInfoList indicates that SystemInformationBlockType10 is present: 4> if the UE is in CE: 5> start acquiring SystemInformationBlockType10; 4> else 5> start acquiring SystemInformationBlockType10 immediately; 3> if schedulingInfoList indicates that SystemInformationBlockType11 is present: 4> start acquiring SystemInformationBlockType11 immediately; NOTE 2: UEs shall start acquiring SystemInformationBlockType10 and SystemInformationBlockType11 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed. 1> if the UE is CMAS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment; 2> when the UE acquires SystemInformationBlockType1 following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment: 3> if schedulingInfoList indicates that SystemInformationBlockType12 is present: 4> acquire SystemInformationBlockType12; NOTE 3: UEs shall start acquiring SystemInformationBlockType12 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

It can be seen that for legacy LTE/eMTC, upon changing cell, the UE in idle mode will discard any previously buffered warningMessageSegment and clear the current values of messageIdentifier and serialNumber for the PWS messages. So if continued reception of PWS segmentations among different cells is supported, the current description needs to be revised, e.g., with NB-IoT NTN-specific description or a separate description for NB-IoT NTN to indicate that previously buffered warningMessageSegment and related parameters cannot be discard.

Observation 1: For legacy LTE/eMTC, upon changing cell, the UE in idle mode will discard any previously buffered warningMessageSegment and clear the current values of messageIdentifier and serialNumber for the PWS messages.

We think to introduce the "non-discard" description specifically for NB-IoT NTN would complicate the existing specification description. Also considering that in the GEO deployment scenario most commonly used for NB-IoT NTN, the case where UE cannot receive all segments within a single coverage cell is likely to be rare, the gain from introducing inter-cell PWS reception would not be obvious. So we suggest not supporting inter-cell PWS reception, even for intra-eNB case.

Proposal 1: It’s suggested not supporting inter-cell PWS reception, even for intra-eNB case.

2.1.2 To skip reading SIB1-NB for PWS acquisition

In RAN2#128 meeting, RAN2 has agreed to extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT, but it’s still FFS whether SIB1-NB acquisition is needed.

In general, when the network changes (some of the) system information, it first notifies the UEs about this change, e.g., via Paging message or Direct Indication Information. And then in the next modification period, the network transmits the updated system information. But for ETWS notification and/or CMAS notifications, the network can transmit the SIB10, SIB11, or SIB12 in the nearest scheduled subframes, without waiting for the next modification period.

In below, the corresponding description for eMTC UE behavior in the specification is cited. It can be seen that, after UE acquires etws-Indication or cmas-Indication, the UE can re-acquire SIB1 and further SIB10, SIB11 or SIB12 immediately, i.e., without waiting until the next system information modification period boundary:

TS 36.331 5.3.2.3 Reception of the Paging message by the UE //skip the unrelated part 1> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12;

In order to simplify the receiving process and quickly acquire PWS, in previous meetings some companies suggested that UE can skip the acquisition of SIB1-NB prior to obtaining the PWS information. Instead, they suggest that NW by implementation can provide kind of pre-configured PWS scheduling information within SIB1-NB, even in the absence of an actual PWS notification. Meanwhile, UE could store the scheduling information in advance and utilize it to require SIB10-NB~SIB12-NB upon receiving of ETWS notification and/or CMAS notifications. However, due to the following reasons, we think this way is infeasible:

- Firstly, we think it is not practical for NW to determine pre-configured PWS scheduling info as the network has NO idea about the size of PWS contents.

- Secondly, even assuming that kind of fixed scheduling configuration can be provided by eNB, considering the implementation of eNB can adjust the scheduling for SIB based on the latest network load and the scheduling information may keep changed, then, when the UE receives PWS notification, if it uses previously stored (may even be stored a long time ago) scheduling information to obtain SIB10-NB~SIB12-NB, the risk of message reception failure due to outdated scheduling information would be relatively high.

- Finally, considering PWS notification is rare case, to continuously provide its scheduling information when there is no PWS notification at all would be very bad from the aspect of signaling overhead.

So it’s suggested to still follow the legacy acquisition scheme, e.g., upon receiving of ETWS notification and/or CMAS notifications, ETWS or CMAS capable UE firstly needs to re-acquire SIB1 immediately and then acquire SIB10, SIB11 and SIB12 according to the latest scheduling information in SIB1.

Proposal 2a: It’s suggested to follow the legacy scheme to acquire SIB10-NB~SIB12-NB, e.g., upon receiving the PWS notification from eNB, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

Proposal 2b: It’s no need to support any scheme of providing pre-configured scheduling information for SIB10-NB~SIB12-NB when there is no PWS notification.

  - Other issue

2.2.1 PWS acquisition for UE in RRC_CONNECTED

In previous meeting, RAN2 has agreed that, upon reception of ETWS/CMAS notification, eNB could release (by eNB implementation) the PWS-capable NB-IoT UEs in RRC_CONNECTED to RRC_IDLE to trigger them to acquire SIB10-NB~SIB12-NB messages in RRC_IDLE.

It’s easy to see that, the subsequent challenge after releasing the PWS-capable UEs is that, those UEs may trigger re-access to the network later almost at the same time, causing serious signaling storm problem.

To mitigate the potential signaling storm issue, one straightforward option can be based on network implementation, e.g., eNB configures different extendedWaitTime for different UEs via the RRC release message. However, considering that the eNB may not have clear knowledge of which UEs will need to re-access in the future, and also considering the number of UEs to be released may be large, it will be difficult for the eNB to set suitable waiting time for each UE.

Another option can be to include a waiting time range (e.g., similar as the backoff parameter in RACH procedure) in SIB or RRC release message. The UEs that are released due to PWS reception and need to re-access in the future can randomly select a waiting time within this waiting time range on their own, thereby also achieving an effect of distributing accesses from UEs.

Proposal 3: It’s suggested RAN2 to discuss the following potion to mitigate the potential signaling storm issue caused by releasing UEs to idle state to acquire PWS messages:

- To introduce a waiting time range (e.g., similar as the backoff parameter in RACH procedure) in SIB or RRC release message to facilitate the UEs to randomly select a waiting time in this range on its own. UE will wait for the corresponding time period after PWS acquisition and before next access.

Conclusion

Based on the above analysis, the following proposals are given:

Proposal 1: It’s suggested not supporting inter-cell PWS reception, even for intra-eNB case.

Proposal 2a: It’s suggested to follow the legacy scheme to acquire SIB10-NB~SIB12-NB, e.g., upon receiving the PWS notification from eNB, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

Proposal 2b: It’s no need to support any scheme of providing pre-configured scheduling information for SIB10-NB~SIB12-NB when there is no PWS notification.

Proposal 3: It’s suggested RAN2 to discuss the following potion to mitigate the potential signaling storm issue caused by releasing UEs to idle state to acquire PWS messages:

- To introduce a waiting time range (e.g., similar as the backoff parameter in RACH procedure) in SIB or RRC release message to facilitate the UEs to randomly select a waiting time in this range on its own. UE will wait for the corresponding time period after PWS acquisition and before next access.

Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024

[2] RP-250472, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. March, 2025