3GPP TSG-RAN WG2 Meeting #131	R2-2505471

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.1.3

Source:	MediaTek Inc.

Title:	Further Discussion on the Remaining RRC Issues for Network-Side Data Collection

Document for:	Discussion, Decision

Introduction

The email discussion titled "[POST130][026][AI PHY] 38.331 CR (Ericsson)" identifies the remaining open issues related to RRC for AI PHY features. This contribution aims to address the identified open issues specifically related to network-side data collection.

Discussion

  - Open issue RRC-21: Time related content of logged data

Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity).

This issue refers to the RAN2#129bis agreement:

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

RAN2#130 agreement:

Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

For AI/ML model training, data collection primarily occurs in RRC_CONNECTED state, where network-based timestamping remains feasible. However, UE-generated timestamps are required in specific scenarios:

- Event-Triggered Logging: When the UE autonomously starts logging due to a configured event (e.g., signal quality above/below a threshold), the network cannot predict the trigger timing. The UE must timestamp the first sample at logging initiation.

- Buffer/Power Constraints: If logging pauses due to a full buffer or low-power state, the UE timestamps the first sample upon resuming. This ensures temporal continuity despite interruptions.

In both the above cases, using absolute timestamps (e.g., UTC-based) avoids complexity from relative time calculations. The network can reconstruct timelines by aligning these anchors with subsequent samples, even if gaps exist.

Proposal 1: The UE provides an absolute timestamp on the first data sample of each group when it starts or resumes data logging.

  - Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Issue description: RAN2#127 made the following agreement regarding NW-side data collection:

1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

It was agreed in RAN2#129bis meeting that the measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc). However, the topic of dynamic activation/deactivation has not yet been addressed, as noted in the stage-2 running CR draft.

While dynamic activation/deactivation could theoretically provide the network with more granular control—allowing precise management of when and for which configurations data is collected, and potentially reducing the need for frequent RRC reconfigurations—such flexibility is not essential for AI/ML data collection. This is because data collection for AI/ML model training is a long-term and continuous process. Sufficient data must be accumulated for each configuration to ensure statistical relevance and model robustness. Frequent changes to the targeted configuration could fragment the dataset, reduce data quality, and complicate model training and analysis. Consistent data collection periods are important to maintain the integrity and comparability of the dataset.

The baseline approach is straightforward: the UE starts data collection upon reception of the measurement configuration, unless event-triggered logging is specified, in which case data collection begins when the event occurs. If the network needs to pause or stop data collection, it can do so by sending a new RRC reconfiguration message.

Proposal 2: Dynamic activation/deactivation of data collection and logging is not supported. The UE starts data collection and logging immediately upon reception of the measurement configuration, unless event-triggered logging is configured.

  - Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

Issue description: Whether data availability indication should be sent when the UE has data and low power state is sent and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched.

It is beneficial for the UE to indicate data availability when reporting a low power state. This approach offers several advantages:

- Prevents Data Loss: Notifying the network about any available data before the UE transitions to RRC Inactive or IDLE ensures that valuable measurement data is not discarded. This maximizes the utility of the UE’s measurement and prevents wastage of resources and energy.

- Improves Buffer Management: Allowing the network to retrieve available data when the UE indicates both low power and data availability enables earlier buffer release. This helps to optimize memory usage on the UE and prevents buffer overflow.

- Provides Network Awareness: By extending the buffer status indicator (bufferStatus-r19) to include additional values such as belowThreshold and empty, the network gains a more accurate and granular understanding of the UE’s buffer status.

LoggedDataCollectionAssistance-r19 ::=    SEQUENCE {

lowPowerState-r19      ENUMERATED {true}                                         OPTIONAL,

bufferStatus-r19       ENUMERATED {full, aboveThreshold, belowThreshold, Empty}  OPTIONAL,

...

}

Proposal 3: When the UE enters a low power state, it sends a low power state indication together with the buffer status to the network.

Proposal 4: The buffer status indicator (bufferStatus-r19) is extended to include additional values, i.e. belowThreshold and empty.

  - Open issue RRC-30: Semi-persistent resources for data collection

Issue description: RAN2 excluded usage of aperiodic CSI resource for data collection, but it is still unclear whether semi-persistent resources are needed for this.

For AI-BM prediction, data collection typically requires regular and consistent measurement reports to ensure sufficient training data for the AI models. Periodic CSI resources are already designed to support such continuous and predictable data collection.

Semi-persistent resources are generally beneficial when the measurement/reporting needs are not strictly periodic but still require repeated allocation over a certain timeframe (e.g., for bursty data). However, AI-BM prediction relies on continuous, time-correlated data, which aligns well with the capabilities of periodic resource allocation.

Unless there is a specific use case within AI-BM prediction where data needs to be collected in a non-periodic but still predictable manner, the existing periodic resources should suffice. However, the necessity of semi-persistence resources should be justified by RAN1.

Proposal 5: RAN2 assumes that semi-persistent resources for data collection for model training are generally not needed. RAN1’s input on the necessity of semi-persistent resources is required.

  - Open issue RRC-31: The release of logged AIML data in UE’s buffer

Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered:

- Power off or deregistration

- 48 hrs after the release of logged measurement configuration

- Explicit indication from the serving gNB.

Consistent with the handling of logged MDT, the UE should release the logged AI/ML data in its buffer when the UE is powered off or deregisters from the network. Additionally, if the network does not intend to retrieve the stored data—such as in certain corner cases (e.g., network overload)—the serving gNB can send an explicit indication to the UE to release the logged data.

However, the option of releasing data 48 hours after the logged measurement configuration is released is not considered reasonable. This is because the UE typically discards any logged data upon transitioning to RRC_IDLE, experiencing RLF, or entering RRC_INACTIVE. It is unlikely that the UE would remain in RRC_CONNECTED for 48 hours solely for data buffering. Therefore, the UE would have already discarded the buffered data due to RRC state transitions, RLF, or an explicit release indication from the network before 48 hours elapse.

Proposal 6: The UE shall release the logged AI/ML data in its buffer under the following conditions:

- When the UE is powered off or deregisters from the network

- Upon receiving an explicit indication to release the data from the serving gNB

  - Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases

Issue description: It can be discussed whether the enhancements for NW-side data collection are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication.

For AS layer memory, it is proposed that UE implementations utilize a shared buffer for all AI-related use cases, rather than maintaining separate buffers for each use case. This approach enables buffer sharing, thereby reducing overall memory consumption.

Consequently, the management of AS layer memory for NW-side data collection should be considered common across all AI-related use cases.

Similarly, the mechanism for logged data availability—such as checking whether the buffer is full or has reached a predefined threshold to trigger a data availability report—should also be common for all AI-related use cases, given the use of a shared buffer.

Proposal 7: The AS layer memory is shared for all AI-related use cases.

Proposal 8: The enhancements for NW-side data collection including AS layer memory, logged data availability, low power state indication are common for all AI-related use cases.

Conclusion

Open issue RRC-21: Time related content of logged data

Proposal 1: The UE provides an absolute timestamp on the first data sample of each group when it starts or resumes data logging.

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Proposal 2: Dynamic activation/deactivation of data collection and logging is not supported. The UE starts data collection and logging immediately upon reception of the measurement configuration, unless event-triggered logging is configured.

Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

Proposal 3: When the UE enters a low power state, it sends a low power state indication together with the buffer status to the network.

Proposal 4: The buffer status indicator (bufferStatus-r19) is extended to include additional values, i.e. belowThreshold and empty.

Open issue RRC-30: Semi-persistent resources for data collection

Proposal 5: RAN2 assumes that semi-persistent resources for data collection for model training are generally not needed. RAN1’s input on the necessity of semi-persistent resources is required.

Open issue RRC-31: The release of logged AIML data in UE’s buffer

Proposal 6: The UE release the logged data for AI/ML in UE buffer when:

- Power off or deregistration

- Receiving Explicit indication to release the data from the serving gNB.

Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases

Proposal 7: The AS layer memory is shared for all AI-related use cases.

Proposal 8: The enhancements for NW-side data collection including AS layer memory, logged data availability, low power state indication are common for all AI-related use cases.

Reference

[1] [POST130][026][AI PHY] 38.331 CR (Ericsson)