3GPP TSG-RAN WG2 #131 	R2-2505576

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	8.2.3

Source:	KT Corp.

Title:	Discussion on random access aspects of Ambient IoT

Document for:	Discussion/Decision

WID/SID:	Ambient_IoT_solutions

1	Introduction

In this contribution, we would like to discuss several open issues as shown in the email discussion documents “Summary of A-IoT MAC open issues ([POST130][027][AIoT] MAC Running CR) [1]”.

- CBRA failure detection timing (Issue 2-4)

- Random ID differentiation in Msg2 (Issue 2-5)

We present our observations on each issue and propose baseline solutions for Rel-19, with considerations for possible enhancements in Rel-20.

2	Discussion

2.1	CBRA failure detection (Issue 2-4)

In the current agreements for CBRA procedure, the boundary for detecting CBRA failure after Msg1 transmission is still under discussion. Two candidate options exist:

- Option B: The boundary is the reception of either the next R2D trigger message or the subsequent paging message.

- Option C: The boundary is the reception of either the k-th R2D trigger message (k: FFS) or the subsequent paging message.

Option B offers a simpler state machine for the device and reduces unnecessary monitoring time, which is beneficial for power saving. Option C provides additional flexibility for the reader by allowing multiple R2D trigger opportunities before declaring failure, but introduces extra complexity for both device and network due to the need to manage parameter k. For Rel-19 baseline, minimizing complexity and power consumption is generally preferred while ensuring sufficient time for Msg2 reception.

RAN2 to adopt Option B as the Rel-19 baseline:
→ Boundary for CBRA failure detection = reception of the next R2D trigger message or subsequent paging message.
→ Keep Option C as an optional enhancement for future releases (e.g., Rel-20) where dense deployments or high-collision environments may require additional reader flexibility.

2.2	Random ID differentiation in Msg2 (Issue 2-5)

In Msg2, only including the Random ID (RN16) can lead to collision scenarios where different devices select the same RN16. It means that when Msg2 contains only the Random ID (RN16), different devices could select the same RN16, leading to collision ambiguity and reduced contention resolution efficiency. This makes it difficult for the reader to differentiate devices and could impact contention resolution efficiency. RAN2#130 agreed that adding a Frequency Index along with RN16 in Msg2 is feasible to reduce such collisions, but left the decision on actual inclusion as FFS.

Adding a Frequency Index would:

- Allow the reader to distinguish devices with the same RN16 but on different frequency resources.

- Improve robustness in dense device scenarios.

However, this comes at the cost of additional message overhead, which may be unnecessary in low-density deployments.

RAN2 to define Frequency Index as an optional field in Msg2:
→ Presence of Frequency Index indicated by a presence bit.
→ Reader includes Frequency Index only when collision risk is high 
→ Default behavior in low-density scenarios is to omit the field to save overhead.

3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to adopt Option B as the Rel-19 baseline:
→ Boundary for CBRA failure detection = reception of the next R2D trigger message or subsequent paging message.
→ Keep Option C as an optional enhancement for future releases (e.g., Rel-20) where dense deployments or high-collision environments may require additional reader flexibility.

Proposal 2	RAN2 to define Frequency Index as an optional field in Msg2:
→ Presence of Frequency Index indicated by a presence bit.
→ Reader includes Frequency Index only when collision risk is high (e.g., dense deployments or high load).
→ Default behavior in low-density scenarios is to omit the field to save overhead.

4	References

RP-2505521, Summary of A-IoT MAC open issues (outcome of [POST130][027][AIoT] MAC Running CR), Huawei, HiSilicon, RAN WG2 #131

RAN2#130 chairman’s notes.