3GPP TSG-RAN WG2 Meeting #131	R2-2505586

Bengaluru, India, 25th – 29th August 2025

Agenda item:		8.7.5

Source:		Lenovo

Title:		Remaining open issues for RLC enhancements

Document for:		Discussion and Decision

# Introduction

In this contribution we discuss remaining open issues for the RLC related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

# Discussion

2.1 Avoiding Unnecessary Retransmissions

At the RAN2#129bis meeting, the following agreements regarding unnecessary retransmission avoidance were achieved:

Unnecessary retransmissions avoidance - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact. - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work - Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

At RAN2#130 following agreements were reached:

Agreements for unnecessary retransmission avoidance in RLC - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work - (UE capability-2) Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. (UE capability-2) Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

Open issue RLC-10:

It has been observed that when stopReTxObsoleteSDU is configured at the transmitting entity without configuring t-RxDiscard at the peer receiving entity, the resolution of the transmitting window stalling issue may not be ensured. The question arises whether UE behaviour needs to be optimized or explicitly specified for such a scenario, e.g. the case where stopReTxObsoleteSDU is configured at the UE (TX side) without t-RxDiscard being configured at the peer (NW RX side). If the network configures both stopReTxObsoleteSDU at the transmitting entity and t-RxDiscard at the receiving entity, no issue is expected. Essentially network implementation can handle the operation without requiring additional specification.

For the uplink case, where the UE is the transmitting side, if an RLC bearer is configured with stopReTxObsoleteSDU, the network as receiving side can maintain t-RxDiscard to determine whether a packet should be discarded. In this case, t-RxDiscard does not need to be configured at the UE. For the downlink case, where the UE is the receiving side, if t-RxDiscard is configured for the UE, it remains an implementation decision for the network whether to apply the transmitting side discard functionality.

Based on the above, it is considered that the issue can be handled through network implementation and no additional configuration constraints are required in the specifications. However, if there remains strong concern among companies, a potential standards-based approach could be to include configuration guidance in the RRC specification to address the interaction between stopReTxObsoleteSDU and t-RxDiscard.

Proposal 1: (RLC-10) RAN2 does not pursue with limitations for the configuration of Tx and Rx side discarding.

Open issue RLC-11

During the CR Review discussion, there was an open issue listed regarding the case that there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, i.e., there is no SDU to retransmit the poll with. According to our view, if stopReTxDiscardedSDU is configured, and only SDUs whose (re)transmissions have been stopped remain in the buffer, there is generally no need for the RLC transmitter to include a poll.
However, the real question is whether the Tx window is stalled or not. In the rare case that the transmitting window is stalled, the only approach to ensure an immediate solution to the window stalling problem seems to be re-sending the poll in a retransmitted SDU when t-PollRetransmit expires. We cannot only rely on a solution where Status report will be eventually triggered by Rx side, since SR will be only triggered by RX based on a detected gap. We do believe though that window stalling might be a rare case anyway, therefore the question is whether we need to specify UE behaviour for such corner case. We have a slight preference for confirming the working assumption and not further addressing the corner case, e.g. winow stalling issue.

If the transmitting window is not stalled, there seems to be no reason to keep t-PollRetransmit running if the transmissions of all transmitted but non-ACKed SDUs have been stopped due to discard indication from PDCP.

Proposal 2: (RLC-11) Confirm the working assumption that there is no need to address the window stalling issue with polling retransmission.

During the previous RAN2 meetings, it was also agreed that the value of the new timer will not be lower than that of the t-Reassembly timer. This means that there could exist a corner case where the new timer is set to the same value of the t-Reassembly timer. It has also been agreed that the abandoned SDUs as determined by the new timer will be positively acknowledged in a status report, whereas the expiry of t-Reassembly will also trigger a status report containing negative acknowledgments for incomplete and/ or missing SDUs. Since both the new timer as well as the t-Reassembly have the same start condition, this could lead to discrepancy in the information provided to the RLC Tx entity depending on which status report is received at the transmitter side. Hence, we think RAN2 should discuss a way forward to prevent such a discrepancy, and we think that a simple solution would be to capture a note in the spec indicating that such a discrepancy should be prevented, e.g., by giving precedence to the status report constructed based on the expiry of the new timer over the expiry of t-Reassembly for the same RLC SDUs.

Proposal 3: RAN2 to discuss capturing an additional note in the spec to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value.

2.2 Timely Retransmissions

At the RAN2#129bis meeting, the following agreements were made regarding timely retransmissions:

Autonomous retransmissions and polling enhancements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact. - (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement. - RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. - (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. - Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance - The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

Open issue (RLC-12):  Impact of discard on PDCP SN gap report

At the RAN2#129 meeting, companies discussed the issue with PDCP SN gap report during mobility events. The following agreements were made:

From RAN2 point of view, for Rel-18 alternative 3 should not be supported. RAN2 doesn’t see a big problem to solve for Rel-18. RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives.

The issue is that a SN gap report might get lost during handover and t-Reordering starts unnecessarily at the target RAN node. In RAN2#129, it was concluded that RAN2 doesn’t consider this as a problem which requires some solution for Rel-18, but for Rel-19 RAN2, e.g. due to the introduction of RLC enhancements, it was concluded to rediscuss the issue and to decide whether the issue needs to be addressed by a standard solution. In our view, it is still questionable whether this is a critical issue since the consequence would be only one t-reordering delay without an UE-based enhancement, e.g. new triggering condition for a PDCP SN gap report.

Based on the contributions in the RAN2#130 meeting, there are basically the following two options on the table:

- Option 1: The UE sends the PDCP SN gap report again after connecting to the target gNB.

- Option 2: UE triggers a PDCP SN gap report when it receives a status report in which an SDU indicated in a previous SN gap report is negatively acknowledged.

Although the first option may work, it is a very inefficient one in our view. This is because a gap report lost during a mobility event is not a typical/average case. Hence it is not sensible to have UE send a SN gap report after every mobility event just to handle a corner case.

The option 2, on the other hand, retransmits the PDCP SN gap report to the target RAN node only when the PDCP SN gap report is actually lost and hence there would be no unnecessary PDCP SN gap report retransmission.

For example, the target gNB may send a PDCP status report, e.g. after a mobility event. On the UE side, if it receives a PDCP status report indicating a PDCP SDU is missing but the SDU has already been discarded, The UE can infer that the corresponding SN gap report must have been lost. It then triggers a new SN gap report, which helps the gNB advance the reordering window.

If RAN2 decides that the problem needs to be addressed by a solution for Rel-19, we prefer to agree on the option 2.

Proposal 4. (RLC-12) If RAN2 agrees to introduce a solution for Rel-19, the UE should trigger a PDCP SN gap report when it receives a PDCP status report in which an SDU indicated in a previous SN gap report is negatively acknowledged.

From our perspective it makes sense to configure the enhanced poll to be triggered before the autonomous transmission if both features are configured simulataneously. In order to limit the resource utilization for autonomous retransmission, as well as to provide the RLC transmitter an opportunity to receive a timely status report, we think that if both autonomous retransmission as well enhanced polling are configured simultaneously, the enhanced poll should be triggered first, i.e., enhancedPollingThreshold is configured with a greater value than autonomousReTxThreshold.

Proposal 5. RAN2 to agree that enhancedPollingThreshold is configured with a greater value than autonomousReTxThreshold, if both autonomous retransmission and enhanced polling are configured simultaneously.

# Conclusion

In this contribution, we discuss the remaining open issues for RLC AM enhancements for XR. We have the following observation and proposals:

Proposal 1: (RLC-10) RAN2 not to pursue with limitations for the configuration of Tx and Rx side discarding.

Proposal 2: (RLC-11) Confirm the working assumption that there is no need to address the window stalling issue with polling retransmission.

Proposal 3: RAN2 to discuss capturing an additional note in the spec to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value.

Proposal 4. (RLC-12) If RAN2 agrees to introduce a solution for Rel-19, the UE should trigger a PDCP SN gap report when it receives a PDCP status report which includes an SDU indicated in a previous SN gap report as discarded is negatively acknowledged (NACK).

Proposal 5. RAN2 to agree that enhancedPollingThreshold is configured with a greater value than autonomousReTxThreshold, if both autonomous retransmission and enhanced polling are configured simultaneously.

# References

TS 38.322 v 18.0.0

R2-2410287, AM RLC enhancement, Lenovo