3GPP TSG-RAN WG2 Meeting #131                                      R2-2505591

Bengaluru, India, 25 - 29 Aug 2025

Source:        ZTE Corporation, OPPO, Interdigital, LG, Apple, Charter, Nokia

Title:           Discussion on the co-existence of SBFD and LTM

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The SBFD+LTM related agreement/discussion during the previous meeting is given as below:

RAN2#129-bis agreement Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM RAN2#130 progress [CB] Rapp suggest two alternatives, a) only support RACH-based LTM, or b) neither. - Rapp report there is no discussions before CB and we can just show hand. Option a) support SBFD with RACH-based LTM cell switch, 6 Option b) do nothing 11

This contribution provides our view on SBFD+LTM CFRA.

- Discussion

LTM contains two CFRA scenarios, LTM early sync procedure and LTM cell switch procedure. During the previous meeting discussion, most companies do not support SBFD RO in LTM early sync, mainly because of the following reasons:

- Benefit is unclear: anyway the UE may not be scheduled for L1 handover immediately after LTM early sync, most companies think even if the latency of LTM early sync is reduced, the final latency of the whole LTM procedure may still be high. Thus, the benefit of introducing SBFD RO in LTM early sync is small/unclear.

- Spec impact is large: If the SBFD RO in LTM early sync is supported, the spec has to allow candidate cell to send the SBFD RO configuration to the source cell, which increases spec effort and may involve RAN3’s work.

Thus, the following section only discuss SBFD in LTM cell switch procedure.

Benefit on HO latency reduction

Using SBFD RO provides another set of radio resources for random access. It will be quicker for the UE to access successfully when UE can utilize early SBFD RO in target cell, and the RA successful rate will increase because SBFD RO can further fall back to non-SBFD RO. Since LTM cell switch is also introduced with the motivation of reducing handover latency (fast MAC CE activated-handover), the motivation is well-aligned and the benefit is clear.

Observation 1: In LTM case, it will be quicker for the UE to access successfully when UE can utilize early SBFD RO in target cell, and the RA successful rate will increase because SBFD RO can further fall back to non-SBFD RO.

RAN2 already agrees to add RO type indicator in RACH-ConfigDedicated. Based on this knowledge:

- Case 1. In L3 handover, target cell sends HandoverCommand to the source cell, the HandoverCommand contains RRCReconfigurationWithSync and RACH-ConfigDedicated of the target cell. Since RACH-ConfigDedicated can contain the RO type as SBFD RO, it means during L3 handover, the target cell can support UE to initiate CFRA in SBFD RO.

- Case 2. In LTM, the only difference is source cell sends MAC CE as handover command rather than RRC signaling. It contains the following sub-cases:

  - Case 2-1: In current mechanism, if CFRA parameter (e.g., preamble index, SSB index, Msg1 repetition number) is included in the Rel-18 LTM cell switch command MAC CE, for the CFRA parameters that already provided in the LTM MAC CE, UE directly uses them, and does not need to further check the corresponding CFRA parameters in the RACH-ConfigDedicated of the target cell. Therefore, we think it is straightforward that if RO type indicator is also indicated in MAC CE, then UE can directly read the MAC CE for RO type. This reduces MAC spec effort since it will totally follow the legacy MAC logic where e.g., UE only reads the Msg1 repetition number in MAC CE, no need to further read the parameter in RACH-ConfigDedicated.

  - Case 2-2: If CFRA parameter is not included in the Rel-18 LTM cell switch command MAC CE, and UE does not have valid TA for target cell, the UE has to read the RACH-ConfigDedicated of the target cell to perform CFRA. Since RAN2 already agreed RACH-ConfigDedicated can include the RO type indication, it proves that even in LTM, target cell can allow UE to use SBFD RO for CFRA. Therefore, it is also questionable that why target cell only allows UE to use SBFD RO by RACH-ConfigDedicated, not by LTM cell switch command MAC CE, considering both the two CFRA approaches belongs to the same LTM framework and to handover to a same target cell.

  - Case 2-3: If CFRA parameter is included in the Rel-18 LTM cell switch command MAC CE which does not contain RO type, and the target cell does not provide RACH-ConfigDedicated, UE will have no source to acquire the RO type for this LTM CFRA. At this case, the RO type indicator should be added in the LTM cell switch command MAC CE.

Observation 2: If CFRA parameter and RO type is included in the LTM cell switch command MAC CE, UE only needs to read the LTM cell switch command MAC CE on the concerned CFRA parameters, no need to further check the corresponding parameter in RACH-ConfigDedicated. This reduces MAC spec effort since it will totally follow the legacy MAC logic, and the UE behaviour will be straightforward and simple.

Observation 3: If CFRA parameter is not included in the LTM cell switch command MAC CE, UE should use the RACH-ConfigDedicated of the target cell to perform CFRA. This case proves that even in LTM, target cell can allow UE to use SBFD RO. So it is questionable that why only the RACH-ConfigDedicated can allow SBFD RO, but LTM MAC CE way is excluded. From our point of view, there is no need and no reason to have such restriction.

Observation 4: If CFRA parameter is included in the Rel-18 LTM cell switch command MAC CE which does not contain RO type, but the target cell does not provide RACH-ConfigDedicated, UE will have no source to acquire the RO type for this LTM CFRA. At this case, the RO type indicator should be added in the LTM cell switch command MAC CE.

Minor spec impact in RAN2

For supporting SBFD RO in LTM cell switch, the only RAN2 spec impact is to add SBFD RO type in Rel-18 LTM cell switch command MAC CE.

Observation 5: The spec impact of supporting SBFD RO + LTM cell switch procedure is minor, i.e., the only RAN2 spec impact is to add SBFD RO type in the Rel-18 LTM cell switch command MAC CE.

No spec impact in RAN3

Moreover, supporting RO type indication in LTM cell switch command MAC CE does not have any impact on RAN3’s work/specification, for the following reasons:

- RAN3 quotes the whole RAN2 IE in F1AP LTM signaling transfer. Candidate cell sends the CFRA resource to the source cell before handover within a CU, and the CFRA resource sent in current F1 interface just cites the RAN2 IE RACH-ConfigDedicated. For example, if RO type of the candidate cell is indicated in RACH-ConfigDedicated, the RO type indication will be sent to source cell within the OCTET STRING without any additional indication in RAN3. See below yellow highlight container in F1AP message:

- 38.473-i50 - 9.2.2.7 UE CONTEXT MODIFICATION REQUEST This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU. Direction: gNB-CU gNB-DU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. LTM CFRA Resource Config List; 0..1; YES; ignore. >LTM CFRA Resource Config Item IEs; 1 .. <maxnoofLTMCells>; EACH; ignore. >>Cell ID; M; NR CGI 9.3.1.12; -. >>LTM CFRA Resource Configuration; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8].; -. >>LTM CFRA Resource Configuration for SUL; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier.; -. LTM Configuration ID Mapping List; O; 9.3.1.294; YES; reject.

- The current Rel-18 LTM cell switch command MAC CE can work in intra-DU case. In intra-DU LTM, the source cell and the candidate cell (target cell) belong to one DU, so these cells share the same control and it is feasible to coordinate/indicate/change the cell parameters between each other without any additional RAN3 specification effort. This can be proved by Msg1 repetition number in the LTM cell switch MAC CE, where the source cell can decide to perform a specific Msg1 repetition number (or even a different Msg1 repetition number than the target cell indicates in RACH-ConfigDedicated) for accessing the target cell, and include it in the LTM cell switch command MAC CE, in at least intra-DU case. Similarly in SBFD+LTM, once candidate cell indicates SBFD RO is available (e.g., the RRCReconfiguration of the candidate cell includes SBFD RO configuration), the source cell should be able to indicate whether UE uses SBFD RO to access the target cell.

Observation 6: Add RO type indication in Rel-18 LTM Cell Switch Command MAC CE does not bring any RAN3 work/spec impact for intra-DU LTM case.

Considering the clear benefit/clear motivation/minor RAN2 spec impact/no RAN3 spec impact, we suggest to add RO type indication in the Rel-18 LTM cell switch command MAC CE. It is sufficient to conclude this discussion during the one (last) meeting and will not impact the RAN2 completion of the Rel-19 SBFD WI.

There is spare room left in the MAC CE.

Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

Proposal 1: Add a 1-bit RO type indication in Rel-18 LTM Cell Switch Command MAC CE.

- Conclusion

In this contribution, we propose the following observations and proposal:

Observation 1: In LTM case, it will be quicker for the UE to access successfully when UE can utilize early SBFD RO in target cell, and the RA successful rate will increase because SBFD RO can further fall back to non-SBFD RO.

Observation 2: If CFRA parameter and RO type is included in the LTM cell switch command MAC CE, UE only needs to read the LTM cell switch command MAC CE on the concerned CFRA parameters, no need to further check the corresponding parameter in RACH-ConfigDedicated. It follows the same logic as current MAC writing ways, and it is straightforward and simple.

Observation 3: If CFRA parameter is not included in the LTM cell switch command MAC CE, UE should use the RACH-ConfigDedicated of the target cell to perform CFRA. This case proves that even in LTM, target cell can allow UE to use SBFD RO. So it is questionable that why only the RACH-ConfigDedicated can allow SBFD RO, but LTM MAC CE way is excluded. From our point of view, there is no need and no reason to have such restriction.

Observation 4: If CFRA parameter is included in the Rel-18 LTM cell switch command MAC CE which does not contain RO type, but the target cell does not provide RACH-ConfigDedicated, UE will have no source to acquire the RO type for this LTM CFRA. At this case, the RO type indicator should be added in the LTM cell switch command MAC CE.

Observation 5: The spec impact of supporting SBFD RO + LTM cell switch procedure is minor, i.e., the only RAN2 spec impact is to add SBFD RO type in LTM cell switch command MAC CE.

Observation 6: Add RO type indication in Rel-18 LTM Cell Switch Command MAC CE does not bring any RAN3 work/spec impact for intra-DU LTM case.

Proposal 1: Add a 1-bit RO type indication in Rel-18 LTM Cell Switch Command MAC CE.